3GPP TSG-RAN WG4 Meeting # 100-e                	R4-2112876

Electronic Meeting, Aug.16 - 27, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the activation delay for deactivated PUCCH SCell

Agenda item:	9.10.2.3

Document for:	Discussion

Introduction

In RAN4#99bis-e meeting, the RRM requirements for PUCCH SCell activation and deactivation were further discussed and the agreements are captured in the way forward [1]. In this paper, we will continue the discussion on the UE behavior options during PUCCH SCell activation and how to define the activation delay requirements accordingly.

Beam information (SSB index) indication

In last meeting, it was agreed that the UE shall indicate the beam information to network for determining the associated SSB in PDCCH order for RA in the following cases [1]:

- If the target PUCCH SCell is unknown cell in FR2, and if there is no active serving cell on that FR2 band (following the same conditions in TS38.133 section 8.3.2 for intra-band FR2 SCell activation), or

- If the target PUCCH SCell is unknown cell in FR1 and if it is not contiguous to an active serving cell in the same band (following the same conditions in TS38.133 section 8.3.2 for intra-band contiguous FR1 SCell activation)

For the cases where beam information needs to be indicated, the question comes if it is feasible to transmit the beam information of PUCCH SCell on the PCell. It is understood PUCCH for one SCell is configured on either PCell or PUCCH SCell. As the SCell is configured with PUCCH by RRC, we wonder if a UE configured with PUCCH SCell can also use the PUCCH in PCell/PSCell. In any case, the feasibility and solutions of transmitting beam information of PUCCH SCell on the PCell/PSCell needs to be discussed. As this is more in RAN2 scope, we propose sending an LS to RAN1/RAN2 asking for the answers.

Proposal 1: For the cases where beam information needs to be indicated, RAN4 send LS to RAN1/2 asking for the feasibility and potential solutions for transmitting the beam information of PUCCH SCell on the PCell/PSCell.

On the uplink, it was agreed to consider the UL spatial relation during PUCCH SCell activation at least for valid case. It is FFS if this shall be considered for invalid case. We understood the UE is able to determine the uplink beam for the RA preamble transmission upon reception of PDCCH order. The same UL beam can be assumed for PUCCH transmission. In some cases, if the spatial relation of PUCCH resource activation command is initiated, this may bring additional MAC uncertainty to the PUCCH SCell activation. To simplify the discussion, we may assume the spatial relation activation command and TCI activation command are received in the same MAC CE, and the requirements are defined under this assumption.

Issue 1-2-5: Whether the UL spatial relation is needed for PUCCH Scell activation?

- Agreements:

  - The UL spatial relation of PUCCH on target being-activated Scell should be considered for PUCCH Scell activation for valid case.

    - The UL spatial relation of PUCCH on target being-activated Scell should be considered for PUCCH Scell activation in FR2 only.

      - the time uncertainty of the MAC CE for UL spatial relation activation of PUCCH in target being-activated Scell shall be defined in the baseline FR2 Scell activation delay part (Tactivate_basic). Details are FFS.

    - FFS: whether the UL spatial relation of PUCCH on target being-activated Scell should be considered for PUCCH Scell activation for invalid case.

Proposal 2: For invalid case, the PUCCH SCell activation delay is defined assuming the spatial relation activation command and TCI activation command are received in the same MAC CE.

PUCCH SCell activation delay

In last meeting, the ending point of the PUCCH SCell activation delay was concluded for valid TA but is open for invalid TA case. In our view, the ending point depends on the UE behavior during PUCCH SCell activation, hence we would focus on the SCell activation procedure itself where the ending point can be easily concluded.

We first check how the CSI report is transmitted at LTE PUCCH SCell activation. According to 3GPP TS 36.133 section 7.7.6 as below, the UE is required to be able to transmit valid CSI report on the PUCCH SCell within the activation delay requirement. As it is to be transmitted on the PUCCH SCell, it can happen only after completion of the random access procedure which indicates the end of the PUCCH SCell activation.

Observation #1: In LTE PUCCH SCell activation, the UE is required to be able to transmit valid CSI report on the PUCCH SCell within the activation delay requirement, which indicates the end of the PUCCH SCell activation.

If the UE has a valid TA for transmitting on an SCell then the UE shall be able to transmit valid CSI report and apply actions related to the SCell activation command as specified in [17] for the SCell being activated on the PUCCH SCell no later than in subframe n+Tactivate_basic

If the UE does not have a valid TA for transmitting on an SCell then the UE shall be capable to perform downlink actions related to the SCell activation command as specified in [17] for the SCell being activated on the PUCCH SCell no later than in subframe n+Tactivate_basic and shall be capable to perform uplink actions related to the SCell activation command as specified in [17] for the SCell being activated on the PUCCH SCell no later than in subframe n+Tdelay_PUCCH SCell and shall transmit valid CSI report for the SCell being activated on the PUCCH SCell no later than in subframe n+Tdelay_PUCCH SCell,

If the UE has a valid TA

In LTE, the SCell activation delay requirement for deactivated PUCCH SCell has been defined in 3GPP TS 36.133 section 7.7.6. According to it, if the UE has a valid TA for transmitting on the PUCCH SCell, random access procedure is not needed. The UE can transmit the CSI report on the PUCCH SCell after the downlink actions are activated. The activation delay for activating the PUCCH SCell remains the same as the single SCell activation delay Tactivate_basic.

For PUCCH SCell activation in NR, the same principle can be applied. With the valid TA, the UE is able to transmit the CSI report on PUCCH SCell hence the activation delay requirement is the same as the activation delay for activating a non-PUCCH SCell.

Proposal 3: If the UE has a valid TA for transmitting on the PUCCH SCell in NR, the activation delay requirement is the same as the activation delay for activating a non-PUCCH SCell i.e. Tactivation_time as defined in TS 38.133 section 8.3.2.

If the UE does not have a valid TA

If the UE does not have a valid TA for transmitting on the PUCCH SCell in LTE, the activation delay is discussed for the downlink and uplink actions separately as cited below. The downlink actions related to the SCell activation command can be performed no later than the single activation delay Tactivate_basic as they are not impacted by the uplink timing. But the uplink actions are possible only after the UE has acquired the TA and is able to transmit a valid CSI reporting. Thus the activation delay for uplink actions in PUCCH SCell is further extended by taking into account the random access procedure to acquire the TA and the time to apply the TA for uplink transmission i.e. T1+T2+T3. It is reasonable to follow the same principles in NR to discuss the activation delay for downlink and uplink actions separately if the UE does not have a valid TA.

Proposal 4: If the UE does not have a valid TA for transmitting on the PUCCH SCell in NR, the activation delay shall be discussed for downlink and uplink actions separately.

If the UE does not have a valid TA for transmitting on an SCell then the UE shall be capable to perform downlink actions related to the SCell activation command as specified in [17] for the SCell being activated on the PUCCH SCell no later than in subframe n+Tactivate_basic and shall be capable to perform uplink actions related to the SCell activation command as specified in [17] for the SCell being activated on the PUCCH SCell no later than in subframe n+Tdelay_PUCCH SCell and shall transmit valid CSI report for the SCell being activated on the PUCCH SCell no later than in subframe n+Tdelay_PUCCH SCell, where:

Tdelay_PUCCH SCell = Tactivate_basic + T1 + T2 + T3

Where:

-	T1 is the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell. T1 is up to 25 subframes and the actual value of T1 shall depend upon the PRACH configuration used in the PUCCH SCell.

-	T2 is the delay for obtaining a valid TA command for the sTAG to which the SCell configured with PUCCH belongs. T2 is up to 13 subframes.

-	T3 is the delay for applying the received TA for uplink transmission. T3 is 6 subframes.

In NR, the SCell activation delay requirement is much more refined than in LTE. In particular, the time periods for HARQ (THARQ) and CSI reporting (TCSI_reporting) are separated from the time used for SCell activation (Tactivation_time). As the CSI reporting requires the valid UL TA, the downlink action should not be deferred by the CSI reporting and thus can be performed immediately after Tactivation_time. Therefore, if the UE does not have a valid TA for transmitting on an SCell, the UE shall be capable to perform downlink actions related to the SCell activation command for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_timeNR slot length.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

Tactivation_time is the SCell activation delay in millisecond.

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

Proposal 5: If the UE does not have a valid TA for transmitting on an SCell, the UE shall be capable to perform downlink actions related to the SCell activation command for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_timeNR slot length.

For the uplink actions, the activation delay requirement is further extended because of the random access procedure to acquire the valid TA. In LTE, the additional delay i.e. T1+T2+T3 is added on top of the basic activation delay requirement for the single downlink SCell. The assumption behind is that the CSI measurements and reporting is performed after completion of the RACH procedure and they are processed in sequence. However, the CSI report may be sent as beam information previously on PCell/PSCell e.g in FR2 unknown case as discussed before. Thus the CSI report in the end of activation seems not necessary. Even if it needs to be sent again on PUCCH SCell, the CSI measurements are performed by monitoring the reference signals and can be done as long as the downlink synchronization is acquired. Hence the UE can already start the CSI measurements after activation of downlink actions and the CSI reporting can be processed in parallel with the RACH procedure. In some cases, the UE may have the CSI measurement results ready before RACH completion and be able to transmit the CSI reporting at the first available CSI reporting resources after RACH. In order to speed up the SCell activation, the activation delay requirement for PUCCH SCell shall be defined assuming no dedicated time period for CSI measurements and reporting.

Proposal 6: The activation delay requirement for PUCCH SCell shall be defined assuming no dedicated time period for CSI measurements and reporting.

If the UE can expect available downlink CSI reference resources within the time period for RACH and applying the TA, it will transmit valid CSI report immediately after the RACH completion thus the delay for CSI reporting can be saved in the SCell activation delay requirement. The activation delay considering the uplink actions can be defined as: the UE shall be capable to transmit valid CSI report and apply uplink actions related to the activation command for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_time+TRACHNR slot length , where TRACH is the delay to perform RACH procedure and apply the TA. In case beam information needs to be transmitted, the CSI report may also be sent together with the beam information hence RACH completion can be considered as the ending point of PUCCH SCell activation.

Proposal 7:  If the UE does not have a valid TA for transmitting on an SCell, the UE shall be capable to perform uplink actions related to the SCell activation command for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_time+TRACHNR slot length, where TRACH is the delay to perform RACH procedure and apply the TA.

Proposal 8: If the UE does not have a valid TA, RACH completion is considered as the ending point of PUCCH SCell activation.

Conclusion

This contribution discussed the SCell activation delay requirement for PUCCH SCell based on the requirements defined in LTE. In our views, the same principles can be adopted in NR, and some adaptations are proposed to align with the SCell activation delay requirements in NR. The proposals are summarized as below:

Proposal 1: For the cases where beam information needs to be indicated, RAN4 send LS to RAN1/2 asking for the feasibility and potential solutions for transmitting the beam information of PUCCH SCell on the PCell/PSCell.

Proposal 2: For invalid case, the PUCCH SCell activation delay is defined assuming the spatial relation activation command and TCI activation command are received in the same MAC CE.

Observation #1: In LTE PUCCH SCell activation, the UE is required to be able to transmit valid CSI report on the PUCCH SCell within the activation delay requirement, which indicates the end of the PUCCH SCell activation.

Proposal 3: If the UE has a valid TA for transmitting on the PUCCH SCell in NR, the activation delay requirement is the same as the activation delay for activating a non-PUCCH SCell i.e. Tactivation_time as defined in TS 38.133 section 8.3.2.

Proposal 4: If the UE does not have a valid TA for transmitting on the PUCCH SCell in NR, the activation delay shall be discussed for downlink and uplink actions separately.

Proposal 5: If the UE does not have a valid TA for transmitting on an SCell, the UE shall be capable to perform downlink actions related to the SCell activation command for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_timeNR slot length.

Proposal 6: The activation delay requirement for PUCCH SCell shall be defined assuming no dedicated time period for CSI measurements and reporting.

Proposal 7:  If the UE does not have a valid TA for transmitting on an SCell, the UE shall be capable to perform uplink actions related to the SCell activation command for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_time+TRACHNR slot length, where TRACH is the delay to perform RACH procedure and apply the TA.

Proposal 8: If the UE does not have a valid TA, RACH completion is considered as the ending point of PUCCH SCell activation.

References

- R4-2108345, WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements, CATT