3GPP TSG-RAN WG4 Meeting # 100-e                                                        R4-2112923

Electronic Meeting, Aug. 16-Aug. 27, 2021

Source:	ZTE Corporation

Title:	TP for 36.717-03-02: CA_1A-7A-20A-38A

Agenda Item:	11.5.3

Document for:	Approval

- Introduction

This contribution provides a text proposal to add UL configuration CA_1A-20A to CA_1A-7A-20A-38A for TR 36.717-03-02.

- Reference

- TR 36.717-03-02, LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17, v0.3.0

- Text Proposal

----- Start of TP -----

  - 7.x		LTE-A inter-band CA: Band 1 and Band 7 and Band 20 and Band 38 with 2 bands UL

    - 7.x.1	List of specific combination issues

      - 7.x.1.1	Channel bandwidths per operating band for CA

Table 7.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-7A-20A-38A | CA_1A-20A | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

      - 7.x.1.2	Co-existence studies for LTE-A UL CA_1A-20A and DL CA_1A-7A-20A-38A

For 2UL/4DL LTE CA band combination, the additional MSD caused by the intermodulation products of two ULs from CA_1A-20A falls into band 7 or band 38 would be defined. However, such MSD are discussed in the corresponding fall back 2UL/3DL LTE CA band combination of CA_1A-7A-20A and CA_1A-20A-38A, where no intermodulation products of two ULs from CA_1A-20A falls into band 7 or band 38.

      - 7.x.1.3		MSD

According to the co-existence studies analysis, when uplink CA configurations CA_1A-20A is paired with downlink CA configuration CA_1A-7A-20A-38A there are no additional analysis of MSD because they are solved in fallback band combination.

      - 7.x.1.4		∆TIB and ∆RIB values

The same requirements of lower order combination can be applied.

----- End of TP -----