3GPP TSG-RAN WG4 Meeting # 100-e                                                        R4-2112925

Electronic Meeting, Aug. 16-Aug. 27, 2021

Source:	ZTE Corporation

Title:	TP for 36.717-03-02: CA_3A-20A-38A

Agenda Item:	11.5.3

Document for:	Approval

- Introduction

This contribution provides a text proposal to add UL configuration CA_3A-20A to CA_3A-20A-38A for TR 36.717-03-02.

- Reference

- TR 36.717-03-02, LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17, v0.3.0

- Text Proposal

----- Start of TP -----

  - 6.x		LTE-A inter-band CA: Band 3 and Band 20 and Band 38 DL with 2 bands UL

    - 6.x.1	List of specific combination issues

      - 6.x.1.1	Channel bandwidths per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-20A-38A | CA_3A-20A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

      - 6.x.1.2	Co-existence studies for LTE-A UL CA_3A-20A and DL CA_3A-20A-38A

For 2UL / 3DL own receiver desensitization study 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.x.1.2-1

Table 6.x.1.2-1: Harmonic and IMD analysis for UL_CA_3A-20A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 1710 | 1785 | 832 | 862 |
| 2nd order harmonics frequency range (MHz) | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd order harmonics frequency range (MHz) | 3420 | 3570 | 1664 | 1724 |
| 3rd order harmonics frequency range (MHz) | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd order harmonics frequency range (MHz) | 5130 | 5355 | 2496 | 2586 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 953 | 848 | 2542 | 2647 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2558 | 2738 | 121 | 14 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4252 | 4432 | 3374 | 3509 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1696 | 1906 | 5084 | 5294 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4268 | 4523 | 711 | 876 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5962 | 6217 | 4206 | 4371 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1738 | 1543 | 6308 | 5978 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5038 | 5233 | 7672 | 8002 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 834 | 1074 | 3691 | 3406 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6156 | 6794 | 7079 |

Based on Table 6.x.1.2-1:

- IMD2, IMD3 produce caused by  CA_3_20 uplink may fall into own Rx of band 38.

      - 6.x.1.3	MSD

According to co-existence studies, both IMD2 and IMD3 may fall into the Rx of band 38. Here we only define MSD caused by IMD2 for this band combination in table 5.x.4-1. For MSD caused by IMD3, Note 1 in the table is specfied which is already existed in the Table 7.3.1A-0g in TS36.101-1.

Table 5.x.4-1: MSD summary for 2UL/3DL CA_3A-20A-38A

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| CA_3A-20A-38A | CA_3A-20A | 3 | 1760 | 5 | 25 | 1855 | 5 | N/A | FDD | N/A |
|  |  | 20 | 850 | 5 | 25 | 809 | 5 | N/A | FDD | N/A |
|  |  | 38 | 2610 | 5 | 25 | 2610 | 5 | 28.4 | TDD | IMD21 |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |

      - 6.x.1.4	∆TIB and ∆RIB values

The same requirements of lower order combination which are specified in TS36.101 can be applied.

----- End of TP -----