3GPP TSG-RAN WG4 Meeting #100-e	R4-2112981

# Electronic Meeting, August 16-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.827 | CR | draft | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 38.827: correction of Positioning ambiguities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_MIMO_OTA_test |  |  |  |  |  | Date: |  |  | 2021-7-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TR 38.827, FR2 MIMO OTA testing is based on 3D multiple probes system, the relative orientations between probes and the DUT have an impact on the measured MIMO OTA metric, the positioning guidance in the current version shows ambiguity issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the positioning guidance to eliminate the ambituities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 DUT positioning and rotation would not be aligned in each test lab. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of change 1>

A.3	DUT positioning guidelines

Table A.3-1 below lists the DUT positioning conditions along with a diagram. The XY plane or P0 condition is just shown for information as a reference to the coordinate system defined in Annex A.1.

Table A.3-1: Summary of possible DUT positioning options

| Testing condition | DUT orientation angles | Diagram |
| --- | --- | --- |
| XY plane or P0 Orientation 1 (default) | α=0; β=0; γ=0 |  |
| P0 Orientation 2 – Option 1 (based on re-positioning approach) | α=180o; β=0; γ=0 |  |
| P0 Orientation 2 – Option 2 (based on re-positioning approach) | α=0; β=180o; γ=0 |  |
| Free space data mode screen up (FS DMSU) | α=0; β=-90; γ=0 |  |
| Free space data mode portrait (FS DMP) | α=0; β=-45; γ=0 |  |
| Free space data mode landscape (FS DML) | α=90 (left tilt); β=-45; γ=0 |  |
| Note: the repositioning concept is applicable to FR2 only. |  |  |

Near-field coupling effects between the antenna and the pedestals/positioners/fixtures generally cause increased signal ripples. Re-positioning the DUT by directing the beam peak away from those areas can reduce the effect of signal ripple on TP measurements. The images on the left of Figure A.3-1 illustrate how to reposition the DUT when the near field coupling effects likely originate from the left, while the images on the right Figure A.3-1 illustrate how to reposition the DUT when the near field coupling effects likely originate from the bottom. In either case, Orientation 1 is used for the measurement of one hemisphere while Orientation 2 is used for the measurement of the opposite hemisphere. This re-positioning approach is applicable to FR2 only.

Figure A.3-1: Illustration of DUT re-positioning. The region with reduces signal ripple is illustrated in yellow. The sector which contains the probes is highlighted in blue.

Due to the non-commutative nature of rotations, the order of rotations is important and needs to be defined when multiple DUT orientations are tested.

The rotations around the x, y, and z axes can be defined with the following rotation matrices

and

.

with the respective angles of rotation, , ,  and

Additionally, any translation of the DUT can be defined with the translation matrix

with offsets tx, ty, tz in x, y, and z, respectively and with

The combination of rotations and translation is captured by the multiplication of rotation and translation matrices.

For instance, the matrix M

describes an initial rotation of the DUT around the x axis with angle α, a subsequent rotation around the y axis with angle β, and a final rotation around the z axis with angle γ. After those rotations, the DUT is translated by tx, ty, tz in x, y, and z, respectively.

For the DUT positioning conditions without re-positioning approach, the turntable implementing the rotation in  shall match the intended DUT  for P0 Orientation 1. For re-positioning approach, the relative orientation between the DUT and the probes for P0 Orientation 2 shall be the same as the relative orientation between DUT and probes as for P0 Orientation 1.

< end of change 1>