3GPP TSG-RAN WG4 Meeting #100-e	R4-2113193

Electronic Meeting, August 16-27, 2021

Source:	ZTE Corporation

Title:          Discussion on NR FR2 HST scenario-A

Agenda Item:	9.9.2.1

Document for:	Approval

- Introduction

The transmission scheme and deployment for scenario-A were discussed in RAN 4# 99-e meeting. While the deployment type for scenario-A was not reached and the issues for uni-directional deployment was captured in the WF [1].

In this contribution, we give our views about deployment and transmission scheme for HST_FR2 scenario-A.

- Discussion

In RAN4# 99-e meeting, the following issues for scenario-A were reached [1]:

- Large difference in propagation delays - FFS the impact of the large difference in propagation delays from different RRHs in a cell when DPS scheme is used: - Large difference in propagation delays exist in - Uni-directional RRH deployment - Some schemes for bi-directional RRH deployment - Whether or not one deployment scenario should be precluded in Rel-17 needs to consider the decision from RRM session. - RRM session will investigate and decide on potential methods to mitigate the propagation delay issue

The max timing gap between two RRH for scenario-A uni-directional deployment is about 3.3 us. For DL transmission, propagation delay can be estimated by SSBs from different RRHs; For UL transmission, there is an ongoing discussion about TA adjustment. It is better to wait for the result from RRM session for UL transmission timing.

Proposal 1 :To estimate propagation delay for DL by SSBs from different RRHs when the propagation delay difference of scenario-A uni-directional deployment is quite large, and to await the outcome from RRM session about the schemes for UL propagation delay differences.

Scheme 2 for scenario-A bi-directional deployment is still FFS in RAN4# 99-e meeting [1]:

- FFS Scheme 2 from deployment scenario perspective : - on potential coverage hole issue, - on propagation delay jump issue.

Figure 1 shows the comparison between scheme 1 and scheme 2 for scenario-A bi-directional deployment. Table A-1 in appendix summarizes our evaluation assumptions for these two schemes of scenario-A bi-directional deployment. Figure 2 and figure 3 show scheme 1 and scheme 2 respectively.

Figure 1, Rx power comparison between scheme 1 and scheme 2 for scenario-A bi-directional deployment

Figure 2 Depiction of scheme 1 for scenario-A bi-directional deployment

Figure 3 Depiction of scheme 2 for scenario-A bi-directional deployment

From figure 1, we can find the Rx power of scheme 2 is very similar to that of scheme 1. Figure 4 shows the Rx power from different beam for scheme 2.

Figure 4, Rx power from different beams when CPE moves from Ds to 2*Ds

From figure 2 we can find that there are 2 effective beams when CPE moves from Ds to 2*Ds. These 2 beams are beam 2 of RRH n and beam 3 of RRH n+1.

Observation 1: Based on the simulation result, there are 2 effective beams for scheme 2 of scenario-A bi-directional deployment when CPE moves from one RRH to another which means scheme 1 and scheme2 are equivalent.

Base on the analysis above, the propagation delay for scheme 2 of scenario-A bi-directional deployment is given in figure 5.

Figure 5, Propagation delay for scheme 2 of scenario-A bi-directional deployment

Figure 5 shows that there is a jump of propagation delay for scheme 2 in scenario-A bi-directional deployment and the value is 2.0758 us based on the evaluation assumption.

From figure 5 we can find that scheme 2 faces the problem of propagation delay jump. Whether propagation delay hoping can be handled is still under discussion, so we prefer to down select scheme 1 as the transmission scheme for scenario-A bi-directional deployment if bi-directional deployment is supported for scenario-A.

Observation 2: There is a jump of propagation delay for scheme 2 in scenario-A bi-directional deployment and the value is about 2 us, which is 3 times larger than CP duration. Scheme 1 does not have the problem of propagation delay jump.

Proposal 2: To down select scheme 1 as the transmission scheme for scenario-A bi-directional deployment if bi-directional deployment is supported for scenario-A.

- Conclusion

In this contribution, we provide the following observations and proposals for HST_FR2 scenario-A:

Observation 1: Based on the simulation result, there are 2 effective beams for scheme 2 of scenario-A bi-directional deployment when CPE moves from one RRH to another which means scheme 1 and scheme2 are equivalent.

Observation 2: There is a jump of propagation delay for scheme 2 in scenario-A bi-directional deployment and the value is about 2 us, which is 3 times larger than CP duration. Scheme 1 does not have the problem of propagation delay jump.

Proposal 1 :To estimate propagation delay for DL by SSBs from different RRHs when the propagation delay difference of scenario-A uni-directional deployment is quite large, and to await the outcome from RRM session about the schemes for UL propagation delay differences.

Proposal 2: To down select scheme 1 as the transmission scheme for scenario-A bi-directional deployment if bi-directional deployment is supported for scenario-A.

- Reference

R4-2108660WF on FR2 HST Deployment Scenario Analysis

- Appendix

Table A-1: Evaluation assumptions for scenario-A bi-directional deployment

| Parameters | Schemes for scenario-A bi-directional deployment |  |
| --- | --- | --- |
|  | Scheme 1 | Scheme 2 |
| RRH antenna configuration | [4 8 2 1 1] dg,H=0.5λ, dg,V=0.5λ |  |
| CPE antenna configuration | [4 2 2 1 1] dg,H=0.5λ, dg,V=0.5λ |  |
| RRH antenna panel placement | 0 degree azimuth for right panel 180 degree azimuth for left panel 0 degree down tilt for right panel 0 degree down tilt for left panel |  |
| RRH Tx power | 30 dBm |  |
| RRH antenna height | 15 m |  |
| CPE antenna height | 5 m |  |
| RRH antenna connector loss | 3 dB |  |
| Paht loss model | RMaLOS PL1 TS38901 |  |
| Tx Beam* | Beam 1 Azimuth: -0.8185 degree Elevation: 90.8184 degree | Beam 1 Azimuth: -0.8185 degree Elevation: 90.8184 degree |
|  | Beam 2 Azimuth: -179.1815 degree Elevation: 90.8184 degree | Beam 2 Azimuth: -1.9092 degree Elevation: 91.4316 degree |
|  | - | Beam 3 Azimuth: -178.0908 degree Down tilt: 91.9081 degree |
| Rx Beam* | Left panel Azimuth: 179.1815 degree Elevation: 89.1816 degree | Left panel Azimuth: 179.1815 degree Elevation: 89.1816 degree |
|  | Right panel Azimuth: 0.8185 degree Elevation: 89.1816 degree | Right panel Azimuth: 0.8185 degree Elevation: 89.1816 degree |

- * The track is parallel to x-axis. The CPE moves in the positive direction of the x-axis. The y-axis coordinate of the RRH is Dmin. The 0 degree of elevation is parallel to z-axis.