3GPP TSG-RAN WG4 Meeting #100-e	R4-2113232

# Electronic,  , 16th - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR RRC Re-establishment with CCA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Correction of RRC re-establishment TCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update testing time in accordance to agreement in the previous meeting. Definition of re-establishment delay Correction of test parameters in accordance to CCA configurations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test cases for RRB re-establishment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.11.2.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Draft CR submitted to 6.1.1.6.3.4 Mirror of R4-2115291 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

### A.11.2.2	RRC connection mobility control

#### A.11.2.2.1	RRC re-establishment

##### A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1

A.11.2.2.1.1.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay with CCA in FR1 with known target cell is within the specified limits. These tests will verify the requirements in clause 6.2.1A.

The test parameters are given in table A.11.2.2.1.1.1-1, table A.11.2.2.1.1.1-2 and table A.11.2.2.1.1.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell with CCA, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

Table A.11.2.2.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.2.2.1.1.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case with CCA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial Condition | Active cell | - | Cell1 | Cell 1 is with CCA. |
|  | Neighbour cells | - | Cell2 | Cell 2 is with CCA. |
| Final condition | Active cell | - | Cell2 |  |
| RF Channel Number |  | - | 1 |  |
| DL CCA model | Dynamic channel accessNote 1, 3 | - | As specified in clause A.3.26.2.1 |  |
|  | Semi-static channel access Note 2, 3 | - |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 | - | As specified in clause A.3.26.2.2 |  |
|  | Semi-static channel access Note 2,3 | - |  |  |
| Time offset between cells |  | - | 3 s | Synchronous cells |
| N310 |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| SSB configuration | Dynamic channel access Note 1, 3 | - | SSB.2 CCA | Table A.3.10A.1.2-1 |
|  | Semi-static channel access Note 2, 3 | - | SSB.1 CCA | Table A.3.10A.1.1-1 |
| DBT window configuration |  | - | DBT.1 | Table A.3.28.1-1 |
| SMTC configuration |  | - | SMTC pattern 1 |  |
| DRX cycle length |  | s | OFF |  |
| PRACH configuration |  | - | FR1 PRACH configuration 1 under CCA | Table A.3.8A.2.1-1 |
| T1 |  | s | 5 |  |
| T2 |  | ms | 480 | Time for the UE to detect RLF |
| T3 |  | s | 2 |  |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.11.2.2.1.1.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case with CCA

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |  | T3 | T1 | T2 |  |  | T3 |
| TDD configuration | - | TDDConf.1.1 CCA |  |  |  |  | TDDConf.1.1 CCA |  |  |  |  |
| DL CCA probability PCCA_DL for dynamic channel access Note 4,6 | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  |
| DL CCA probability PCCA_DL for semi-static channel access Note 5,6 | - | PCCA_DL=0.9375 |  |  |  |  | PCCA_DL=0.9375 |  |  |  |  |
| UL CCA probability PCCA_UL | - | 1 |  |  |  |  | 1 |  |  |  |  |
| PDSCH RMC configuration |  | SR.1.1 CCA |  |  |  |  | SR.1.1 CCA |  |  |  |  |
| RMSI CORESET RMC configuration |  | CR.1.1 CCA |  |  |  |  | CR.1.1 CCA |  |  |  |  |
| Dedicated CORESET RMC configuration |  | CCR.1.1 CCA |  |  |  |  | CCR.1.1 CCA |  |  |  |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |  |
| TRS configuration |  | TRS.1.2 TDD |  |  |  |  | N/A |  |  |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| Active DL BWP confgiuration |  | DLBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | ULBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | ULBWP.1.1 |  |
| RLM-RS |  | SSB |  |  |  |  | SSB |  |  |  |  |
|  | dB | 1.54 |  | -infinity |  | -infinity | -3.79 | 4 |  |  | 4 |
| Note2 | dBm/SCS | -95 |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 7 |  | -infinity |  | -infinity | 4 | 4 |  |  | 4 |
| SS-RSRP Note3 | dBm/SCS | -88 |  | -infinity |  | -infinity | -91 | -91 |  |  | -91 |
| Io | dBm/38.16 MHz | -54.65 |  | -58.50 |  | -58.50 | -54.65 | -58.50 |  |  | -58.50 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 5: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |  |  |

A.11.2.2.1.1.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to a known NR intra frequency cell with CCA shall be less than 1350 + MAX (200, (5+K1) x 20)  ms.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay_CCA=TUE_re-establish_delay_CCA+TUL_grant

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay_CCA=50 ms+Tidentify_intra_NR_CCA+i=1Nfreq-1Tidentify_inter_NR_CCA,i+TSI-NR_CCA+TPRACH_CCA

Where

Nfreq = 1

Tidentify_intra_NR_CCA = MAX (200 ms, (5+K1) x TSMTC), where

K1 is the number of SMTC occasions not available at the UE due to DL CCA failures during RRC re-establishment period on the carrier with CCA.

TSMTC = 20 ms is the SMTC periodicity.

Tidentify_inter_NR_CCA = 0 ms

TSI-NR_CCA = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target intra-frequency NR cell.

TPRACH_CCA = TSSB,RO + 10 ms, where:

-	TSSB,RO is the SSB to PRACH occasion association period as defined in Table 8.1-1 of TS 38.213 [39], which is TSSB,RO=10 ms for FR1 PRACH configuration 1 under CCA.

This gives a total of 1350 + MAX (200, (5+K1) x 20) ms.

##### A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1

A.11.2.2.1.2.1	Test Purpose and Environment

The purpose is to verify that the NR inter-frequency RRC re-establishment delay with CCA in FR1 without known target cell is within the specified limits. These tests will verify the requirements in clause 6.2.1A.

<End of Change 1>