3GPP TSG-RAN4 Meeting #100-E 	R4-2113313

# Electronic Meeting, 16th - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.842 | CR | 0017 | rev | - | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 37.842: Addition of array antenna model extension in subclause 5.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA-Core |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At last RAN4 meeting an extension of the antenna array model originally defined in TR 37.842 to support sub-arrays was agreed. The extension of the model and corresponding relevant model parameters was communicated to ITU-R WP 5D and CEPT in an LS (R4-2108080). The antenna model developed for AAS BS is also used for NR BS. This CR was created to document the updated model and corresponding parameters in proper TR in RAN4. The model update will not affect RAN4 coexistene work, but will better reflect AAS base station implementations. The new information is essential for coming work in RAN4 where new features related to AAS BS is continuously developed. Since RAN4 have communicated this information to groups outside 3GPP it is essential to document the background technical information in relevant RAN4 technical reports. Corresponding CR(s) for TR 37.842, TR 38.820 and TR 38.921 is also provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new subclause (5.3.3.4) is added for the antenna model extension and corresponding parameter sets. The information consists of two parts: - A model extension to support sub-array geometries - Parameter sets for Macro Rural, Macro suburban and Macro urban deployment scenarios |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the technical background information in technical reports would not capture relevant information relevant for modelling AAS base stations correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclause 5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.3	Antenna array model

#### 5.3.3.1	General

The AAS BS antenna array model is determined by array element pattern, array factor and signals applied to the system. The element pattern and composite antenna pattern are defined in the subclauses 5.3.3.2 and 5.3.3.3.

NOTE:	The and definition in this subclause is based on the coordinate system in subclause 5.4.4.1 of 3GPP TR 37.840 [2].

#### 5.3.3.2	Array element pattern

# Table 5.3.3.2-1: Array element pattern for antenna array model

| Horizontal Radiation Pattern |  |
| --- | --- |
| Horizontal half-power bandwidth of single array element | For single column antenna: 65º For multi-column antenna: 80º |
| Front-to-back ratio | Am = 30 dB |
| Vertical Radiation Pattern | , SLAv =30 dB |
| Vertical half-power bandwidth of single array element | 65º |
| Array element radiation pattern |  |
| Element Gain without antenna losses | For single column antenna: GE,max = 9 dBi For multi-column antenna: GE,max = 7.5 dBi (note) |
| NOTE: GE,max is obtained from 3GPP TR 37.840 [2], table 5.4.4.2.1-1. |  |

#### 5.3.3.3	Composite antenna pattern

##### 5.3.3.3.1	UE specific beamforming

# Table 5.3.3.3.1-1: Composite antenna pattern for UE specific beamforming

| Configuration | Multiple columns (NVxNH elements) |
| --- | --- |
| Composite Array radiation pattern in dB | For beam i: the super position vector is given by: the weighting is given by: |
| Antenna array configuration (Row×Column) | 10×4 |
| Horizontal radiating element spacing d/ λ | 0.5 |
| Vertical radiating element spacing d/ λ | 0.9 |
| Down-tilt angle | 0° |
| Power allocated to each beam | 6 dB below the maximum transmitted power |

##### 5.3.3.3.2	Vertical cell splitting

# Table 5.3.3.3.2-1: Composite antenna pattern for AAS BS applying vertical cell splitting

| Configuration | Single column (N-elements) |
| --- | --- |
| Composite Array radiation pattern in dBi | For inner cell: For outer cell: the super position vector is given by: the weighting is given by: |
| Antenna array configuration (Row×Column) | 10×1 |
| Vertical radiating element spacing d/λ | 0.9 |
| Power in outer/inner cell | -3 dB/-3 dB |
| Down-tilt angles for vertical cell splitting | 9°/15° |

##### 5.3.3.3.3	Horizontal cell splitting

# Table 5.3.3.3.3-1: Composite antenna pattern for AAS BS applying horizontal cell splitting

| Configuration | Multiple columns (NVxNH elements) |
| --- | --- |
| Composite Array radiation pattern in dBi | For Cell 1: For Cell 2: the super position vector is given by: the weighting is given by: |
| Antenna array configuration (Row×Column) | 10×4 |
| Horizontal radiating element spacing d/ λ | 0.5 |
| Vertical radiating element spacing d/ λ | 0.9 |
| Down-tilt angle | 9° |
| horizontal electrical steering for cell splitting | ±25° |
| Power split (Cell1/Cell2) | -3 dB/-3 dB |

#### 5.3.3.4	Sub-array extension

An extended version of the AAS array antenna model is created to support vertical sub-array geometries with fixed sub-array down-tilt. The model equations are summarized in Table 5.3.3.4-1.

Table 5.3.3.4-1: Extended AAS model

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ |

In Table 5.3.3.4-2, representable parameter sets relevant for an AAS base station operating within 1710 to 4990 MHz are provided.

Table 5.3.3.4-2: Antenna array parameters

| Parameter | Macro Rural | Macro suburban | Macro urban |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º |
| Antenna sub-array configuration (Row × Column) (Note 4) | 4 × 8 elements | 4 × 8 elements | 4 × 8 elements |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V |
| Number of element rows in sub-array | 3 | 3 | 3 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | 3 | 3 | 3 |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 28 | 28 | 28 |
| Base station horizontal coverage range (degrees) | +/-60 | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 3 | 6 | 6 |

Note 1:	The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°.

Note 2:	The element gain includes the loss and is per polarization.

Note 3:	The conducted power per sub-array assumes 4x8x2 sub-arrays (i.e. power per H/V polarized element).

Note 4:	4 × 8 means there are 4 vertical and 8 horizontal radiating sub-arrays.

Note 5:   For the case of 3 elements per sub array, dv will be 2.1 wavelengths.