3GPP TSG-RAN WG4 #100-e	R4-2113335

Electronic meeting, 16th August – 27th August 2021

Source: 	Ericsson

Title:  	Active BWP switching requirements for extending NR operation to 71GHz

Agenda Item:	9.16.7.4

Document for:	Discussion

# Introduction

The WF for extending current NR operation to 71 GHz [1] was approved in RAN#99-e meeting.  In this contribution, we present some active BWP switching requirements for extending NR operation to 71GHz.

# Disucssion

According to [1], issues of impact of higher SCS on RRM requirements were agreed:

- Define new RRM requirements due to higher data/SSB SCS for at least the following topics: - Timing - UE transmit timing - Timing advance (TA) - Interruptions - Active BWP switching delay - Measurement gaps - Interruption time - Study impact on RRM requirements due to higher SSB SCS for at least the following topics: - Intra-frequency measurement - Inter-frequency measurement - Other RRM requirements, if identified are not precluded.

Active BWP switch delay

Proposal 1: Active BWP switch delay for 52.6-71GHz still follows 600us for type 1 UE and 200us for type 2 UE.

Proposal 2: Table of Active BWP switch delay is updated with higher SCS’s following assumption in Proposal1.

Table 8.6.2-1: BWP switch delay

|  | NR Slot length | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  | (ms) | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| 4 | 0.03125 | [20] | [65] |
| 5 | 0.015625 | [39] | [129] |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

# Conclustion

Proposal 1: Active BWP switch delay for 52.6-71GHz still follows 600us for type 1 UE and 200us for type 2 UE.

Proposal 2: Table of Active BWP switch delay is updated with higher SCS’s following assumption in Proposal1.

Table 8.6.2-1: BWP switch delay

|  | NR Slot length | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  | (ms) | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| 4 | 0.03125 | [20] | [65] |
| 5 | 0.015625 | [39] | [129] |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

# References

[1] R4-2108354, WF on NR Ext to 71GHz – RRM, Qualcomm