3GPP TSG-RAN WG4 Meeting # 100-e                              R4-2113398

Electronic Meeting, 16th– 27th August, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion and reply draft LS on ambiguity in deciding TL,C

Agenda Item:	5.1.2.1

Document for:	Approval

Introduction

In previous meeting, RAN4 received the LS [1] on ambiguity in deciding TL,C from RAN5. Regarding ∆TC,c in clause Configured transmitted power , RAN5 has two different understandings:

- The source of ∆TC,c  is the same as NOTE 3 in table 6.2.1-1, therefore the 1.5dB relaxation shouldn’t be considered again when deciding TL,C.

- Strictly following above core requirements, the 1.5dB relaxation should be considered twice when deciding ∆TC,c  and TL,C.

The numeric example of understanding 1 and 2 can been seen below. With understanding 2 the lower limit of Pumax is further relaxed by 1.5dB.

| Understanding | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Lower limit (dBm) PCMAX_L,f,c – MAX(T(PCMAX_L,f,c), TL,c) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 21.5 | 2.0 | 2 | 19.5 |
| 2 | 23 | 0 | 1.5 | 21.5 | 2.0 | 3.5 | 18 |

Please note both understanding have been adopted in TS 38.521-1 for different bands respectively, therefore it’s important that RAN4 provides clear guidance on which understanding is correct.

In this paper, we’d like to clarify this ambiguity in order to align with each other. A draft reply LS are attached in the Annex.

Discussion

Based on the contribution [2] and [3], the Note 3 in table 6.2.1-1 is to mitigate the effect of duplex filter band edge attenuation, which was originated from LTE 36.101-v8.5.1. Based on the agreed CR [4], this relaxation was introduced into the Pcmax in LTE 36.101-v8.7.0 for the same technical reason. It’s clear that the understanding 1 is RAN4’s common understanding.

Proposal 1: The understanding 1 “The source of ∆TC,c  is the same as NOTE 3 in table 6.2.1-1, therefore the 1.5dB relaxation shouldn’t be considered again when deciding TL,C” is correct.

In order to mitigate the ambiguity, there are two corrections which were proposed in previous meetings.

Option 1 (R4-2016494 [5]): Clarifying that tolerance TL,c doesn’t consider 1.5dB relaxation when deciding T(PCMAX,f,c) (Current requirements aren’t changed)

Option 2 (R4-2100139 [6]): dTc is removed from relevant PCMAX_L,f,c formulas. (Current requirements will be changed)

If we take option 2, the relevant PCMAX_L,f,c formulas will be changed. That means the corresponding Lower limit (dBm) PCMAX_L,f,c – MAX(T(PCMAX_L,f,c), TL,c) will be changed. We compared the Lower limit (dBm) using these two approaches as shown below.

Table 1 Lower limit comparison between these two approaches

| Case A (Huawei CR R4-2016494) |  |  |  |  |  |  | Case B(Nokia CR R4-2100139) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ppowerclass | Power Reduction | Pcmax_L(∆TC,c =1.5) | T(PCMAX_L,f,c) | TL,c | Lower limit (dBm) PCMAX_L,f,c – MAX(T(PCMAX_L,f,c), TL,c) | Lower limit Gap dB (caseA minus caseB) | Ppowerclass | Power Reduction | Pcmax_L(∆TC,c =0) | T(PCMAX_L,f,c) | TL,c | Lower limit (dBm) PCMAX_L,f,c – MAX(T(PCMAX_L,f,c), TL,c) |
| 23 | 0 | 21.5 | 2 | 2 | 19.5 | 0 | 23 | 0 | 23 | 2 | 3.5 | 19.5 |
| 23 | 1 | 20.5 | 2.5 | 2 | 18 | -0.5 | 23 | 1 | 22 | 2 | 3.5 | 18.5 |
| 23 | 2 | 19.5 | 3.5 | 2 | 16 | -1.5 | 23 | 2 | 21 | 2 | 3.5 | 17.5 |
| 23 | 3 | 18.5 | 4 | 2 | 14.5 | -2 | 23 | 3 | 20 | 2.5 | 3.5 | 16.5 |
| 23 | 4 | 17.5 | 5 | 2 | 12.5 | -3 | 23 | 4 | 19 | 3.5 | 3.5 | 15.5 |
| 23 | 5 | 16.5 | 5 | 2 | 11.5 | -2.5 | 23 | 5 | 18 | 4 | 3.5 | 14 |
| 23 | 6 | 15.5 | 5 | 2 | 10.5 | -1.5 | 23 | 6 | 17 | 5 | 3.5 | 12 |
| 23 | 7 | 14.5 | 5 | 2 | 9.5 | -1.5 | 23 | 7 | 16 | 5 | 3.5 | 11 |
| 23 | 8 | 13.5 | 5 | 2 | 8.5 | -1.5 | 23 | 8 | 15 | 5 | 3.5 | 10 |
| 23 | 9 | 12.5 | 6 | 2 | 6.5 | -2.5 | 23 | 9 | 14 | 5 | 3.5 | 9 |
| 23 | 10 | 11.5 | 6 | 2 | 5.5 | -2.5 | 23 | 10 | 13 | 5 | 3.5 | 8 |
| 23 | 11 | 10.5 | 6 | 2 | 4.5 | -1.5 | 23 | 11 | 12 | 6 | 3.5 | 6 |
| 23 | 12 | 9.5 | 6 | 2 | 3.5 | -1.5 | 23 | 12 | 11 | 6 | 3.5 | 5 |
| 23 | 13 | 8.5 | 6 | 2 | 2.5 | -1.5 | 23 | 13 | 10 | 6 | 3.5 | 4 |
| 23 | 14 | 7.5 | 7 | 2 | 0.5 | -2.5 | 23 | 14 | 9 | 6 | 3.5 | 3 |
| 23 | 15 | 6.5 | 7 | 2 | -0.5 | -2.5 | 23 | 15 | 8 | 6 | 3.5 | 2 |
| 23 | 16 | 5.5 | 7 | 2 | -1.5 | -1.5 | 23 | 16 | 7 | 7 | 3.5 | 0 |
| 23 | 17 | 4.5 | 7 | 2 | -2.5 | -1.5 | 23 | 17 | 6 | 7 | 3.5 | -1 |
| 23 | 18 | 3.5 | 7 | 2 | -3.5 | -1.5 | 23 | 18 | 5 | 7 | 3.5 | -2 |
| 23 | 19 | 2.5 | 7 | 2 | -4.5 | -1.5 | 23 | 19 | 4 | 7 | 3.5 | -3 |
| 23 | 20 | 1.5 | 7 | 2 | -5.5 | -1.5 | 23 | 20 | 3 | 7 | 3.5 | -4 |
| 23 | 21 | 0.5 | 7 | 2 | -6.5 | -1.5 | 23 | 21 | 2 | 7 | 3.5 | -5 |
| 23 | 22 | -0.5 | 7 | 2 | -7.5 | -1.5 | 23 | 22 | 1 | 7 | 3.5 | -6 |
| 23 | 23 | -1.5 | 7 | 2 | -8.5 | -1.5 | 23 | 23 | 0 | 7 | 3.5 | -7 |
| 23 | 24 | -2.5 | 7 | 2 | -9.5 | -1.5 | 23 | 24 | -1 | 7 | 3.5 | -8 |
| 23 | 25 | -3.5 | 7 | 2 | -10.5 | -1.5 | 23 | 25 | -2 | 7 | 3.5 | -9 |
| 23 | 26 | -4.5 | 7 | 2 | -11.5 | -1.5 | 23 | 26 | -3 | 7 | 3.5 | -10 |
| 23 | 27 | -5.5 | 7 | 2 | -12.5 | -1.5 | 23 | 27 | -4 | 7 | 3.5 | -11 |
| 23 | 28 | -6.5 | 7 | 2 | -13.5 | -1.5 | 23 | 28 | -5 | 7 | 3.5 | -12 |
| 23 | 29 | -7.5 | 7 | 2 | -14.5 | -1.5 | 23 | 29 | -6 | 7 | 3.5 | -13 |
| 23 | 30 | -8.5 | 7 | 2 | -15.5 | -1.5 | 23 | 30 | -7 | 7 | 3.5 | -14 |

Based on the lower limit comparison between these two approaches, the lower limits for Nokia’s correction are tighter than the current requirements. Considering this correction is for Rel-15 maintenance, it may have an impact on legacy UEs if we change the RF requirements.

Observation 1: It will have an impact on legacy UEs if we tighten the RF requirements in Rel-15 maintenance as option 2.

Proposal 2: Not to change the current requirements for lower limits of PUMAX,f,c and RAN4 can implement the corrections as option 1 to clarify it.

Summary

Based on the analysis and discussion above, all the proposals are listed below:

Proposal 1: The understanding 1 “The source of ∆TC,c  is the same as NOTE 3 in table 6.2.1-1, therefore the 1.5dB relaxation shouldn’t be considered again when deciding TL,C” is correct.

Observation 1: It will have an impact on legacy UEs if we tighten the RF requirements in Rel-15 maintenance as option 2.

Proposal 2: Not to change the current requirements for lower limits of PUMAX,f,c and RAN4 can implement the corrections as option 1 to clarify it.

- References

- R5-206676, “LS on ambiguity in deciding TL,C”, Huawei

- R4-090681, “UE uplink power control”, Nokia

- R4-091016, “UE uplink power control”, Nokia,NSN, Motorola, Ericsson, Freescale

- R4-093365, “Pcmax definition”, Qualcomm Europe, Nokia, Ericsson, ST-Ericsson, Motorola, Samsung

- R4-2016494, “Update of configured transmitted power to remove ambiguity in TL,C”, Huawei, HiSilicon

- R4-2100139, “CR REL15 on ambiguity in deciding TL,C”, Nokia, Nokia Shanghai Bell

- Annex

3GPP TSG-RAN WG4 Meeting # 100-e                                  R4-211

Electronic Meeting, 16th– 27th August, 2021

Title:	Reply LS on ambiguity in deciding TL,C

Response to:	LS R5-206676 on ambiguity in deciding TL,C from RAN5

Release:	Release 15

Work Item:	New Radio Access Technology (NR_newRAT-Core)

Source:	TSG RAN WG4

To:	TSG RAN WG5

Cc:

Contact person:	Zhang Peng

Zhangpeng169@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- 1	Overall description

RAN4 thanks RAN5 LS on ambiguity in deciding TL,C. RAN4 has discussed understandings and achieved the following agreement:

The understanding 1 “The source of ∆TC,c is the same as NOTE 3 in table 6.2.1-1, therefore the 1.5dB relaxation shouldn’t be considered again when deciding TL,C” is correct.

Therefore, the numeric example of understanding 1 should be used for the UE conformance testing.

| Understanding | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Lower limit (dBm) PCMAX_L,f,c – MAX(T(PCMAX_L,f,c), TL,c) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 21.5 | 2.0 | 2 | 19.5 |

- 2	Actions

To TSG RAN WG5

ACTION: 	RAN4 respectfully asks RAN5 to take account the above RAN4 agreements in the future.

- 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN4 Meeting#101e 	 1 – 12 November 2021	Electronic Meeting

TSG-RAN4 Meeting#102e 	 21 – 25 Febuary 2022 Electronic Meeting