3GPP TSG-RAN WG4 Meeting # 100-e                              R4-2113427

Electronic Meeting, 16– 27 August, 2021

Source: 	Huawei, HiSilicon

Title: 	Further discussion on NTN simulation assumptions

Agenda Item:	9.13.2.1

Document for:	Approval

Introduction

In RAN4#99-e, RAN4 made some progress on the NTN simulation assumptions [1]. However, there are still some open issues for the assumptions. In this paper, we’d like to further discuss NTN simulation assumptions to address the remaining issues.

Co-existence simulation scenarios

For 23dBm handheld UE, all the UL SINRs for GEO scenario are less than -10dB. That means there is no UL throughput for 23dBm handheld UE in GEO scenario. Thus, GEO can’t be deployed in S-band for 23dBm handheld UE.

Proposal 1: To remove GEO scenario since PC3 handheld UE has no UL throughput.

The proposed frequency and bandwidth for co-existence study [1] are shown below.

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Rural | 2 GHz | 20MHz | FDD, TDD | 1 |
| TN Urban macro | 2 GHz | 20MHz | FDD, TDD | 1 |
| TN Dense Urban | 2 GHz | 20MHz | FDD, TDD | 1 |
| GEO | 2 GHz | 5/10/15/20 MHz for FR1 | FDD | 1, 31 |
| LEO | 2 GHz | 5/10/15/20 MHz for FR1 | FDD | 1, 31 |
| HAPS | 2 GHz | TBD | FDD | [1] |
| Note 1: 2 phases will be considered for FRF: FRF=1 in phase 1 for simplification; FRF=3 in phase 2 or it is found FRF=1 is too stringent. |  |  |  |  |

For FRF=3, the adjacent channel interference can occur among different satellite beam in one satellite. Referring to the definition of the interfering beams [2] as below, there are 6 adjacent beam for the beam of interest. Thus, one satellite with FRF=3 can be considered as the NTN-NTN co-existence scenario.

Proposal 2: One satellite with FRF=3 can be considered as candidate NTN-NTN co-existence scenario.

Simulation parameters

In table 2.3-2, the satellite max Tx power in dBm for 20MHz is open for all the scenarios.

Proposal 3: the satellite max Tx power in dBm for 20MHz can be assumed as below.

| Satellite orbit |  |  | GEO |  |  |  | LEO-1200 |  |  |  | LEO-600 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km |  |  |  | 1200 km |  |  |  | 600 km |  |  |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Satellite EIRP density |  | 2GHz | 59 dBW/MHz |  |  |  | 40 dBW/MHz |  |  |  | 34 dBW/MHz |  |  |  |
| Satellite max TX power in dBm | BW (MHz) |  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 |
|  | SCS 15kHz |  | 44.53 | 47.71 | 49.53 | 50.71 | 46.53 | 49.71 | 51.53 | 52.71 | 40.53 | 43.71 | 45.53 | 46.71 |
|  | SCS 30kHz |  | 43.98 | 47.37 | 49.36 | 50.37 | 45.98 | 49.37 | 51.36 | 52.37 | 39.98 | 43.37 | 45.36 | 46.37 |

Proposal 4: the handover margin can be 3dB for NTN.

In table 2.3-6, the “cell radius” should be replaced by “cell range” because of the different concepts for “cell radius” between ITU-R and 3GPP. Alternatively, we can use ISD to avoid any ambiguities in the assumption.

In table 2.3-6, the suburban macro can be removed since the agreed scenarios didn’t include suburban macro.

In table 2.3-7, the ACS requirements for BS is 46dB instead of 45dB.

Proposal 5: to correct the above errors in table 2.3-6 and 2.3-7.

TN BS antenna parameters

For AAS antenna, it refers to TR 38.921 pattern. However, for the specific parameters, the following table can be assumed based on R4-2102840.

|  |  | Rural | Macro urban |
| --- | --- | --- | --- |
| 1.1 | Antenna pattern | TS 38.921 |  |
| 1.2 | Element gain (dBi) (Note 2) | 7.1 | 6.4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V | 90º for H 65º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements | 8 × 8 elements |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 |
| 1.9 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 | 25 |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 | 120 |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-120 |
| 1.12 | Mechanical downtilt (degrees) | 3 | 10 |

Proposal 6: to assume specific Antenna parameters for AAS antenna pattern based on R4-2102840.

HAPS simulation assumptions

For approved HAPS simulation assumptions [3], the indoor UE percentage is not aligned with current NTN’s assumptions. 70% and 50% is too high. Generally, 20% is more reasonable. In table 5, Urban macro channel model can refer to TR 38.901 because channel model in 38.803 was not updated.

Proposal 7: to assume 20% indoor UE for TN in HAPS coexistence simulation. Urban macro channel model can refer to TR 38.901.

- Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: To remove GEO scenario since PC3 handheld UE has no UL throughput.

Proposal 2: One satellite with FRF=3 can be considered as candidate NTN-NTN co-existence scenario.

Proposal 3: the satellite max Tx power in dBm for 20MHz can be assumed as below.

| Satellite orbit |  |  | GEO |  |  |  | LEO-1200 |  |  |  | LEO-600 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km |  |  |  | 1200 km |  |  |  | 600 km |  |  |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Satellite EIRP density |  | 2GHz | 59 dBW/MHz |  |  |  | 40 dBW/MHz |  |  |  | 34 dBW/MHz |  |  |  |
| Satellite max TX power in dBm | BW (MHz) |  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 |
|  | SCS 15kHz |  | 44.53 | 47.71 | 49.53 | 50.71 | 46.53 | 49.71 | 51.53 | 52.71 | 40.53 | 43.71 | 45.53 | 46.71 |
|  | SCS 30kHz |  | 43.98 | 47.37 | 49.36 | 50.37 | 45.98 | 49.37 | 51.36 | 52.37 | 39.98 | 43.37 | 45.36 | 46.37 |

Proposal 4: the handover margin can be 3dB for NTN.

In table 2.3-6, the “cell radius” should be replaced by “cell range” because of the different concepts for “cell radius” between ITU-R and 3GPP.

In table 2.3-6, the suburban macro can be removed since the agreed scenarios didn’t include suburban macro.

In table 2.3-7, the ACS requirements for BS is 46dB instead of 45dB.

Proposal 5: to correct the above errors in table 2.3-6 and 2.3-7.

Proposal 6: to assume specific Antenna parameters for AAS antenna pattern.

Proposal 7: to assume 20% indoor UE for TN in HAPS coexistence simulation. Urban macro channel model can refer to TR 38.901.

- References

- R4-2108645, Simulation assumptions for NTN co-existence study, Samsung, CATT

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)

- R4-2108646, Simulation assumptions for HAPS co-existence, Nokia