3GPP TSG-RAN WG4 Meeting #100-e	R4-2113609

Electronic Meeting, 16 August – 27 August 2021

Source:	Ericsson, Bell Mobility

Title:	TP for TR 38.717-04-02: CA_n41-n66-n71-n78

Agenda item:			8.12.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-04-02 to include CA_n41A-n66A-n71A-n78A as defined in WID [1].

# 2. Text Proposal

# ---Start of changes---

## 5.2.x	CA_n41-n66-n71-n78

#### 5.2.x.1	Operating bands for CA

Table 5.2.x.1-1: Inter-band CA operating bands of CA_n41-n66-n71-n78

| NR CA Band | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- |
| CA_n41-n66-n71-n78 | n41, n66, n71, n78 |

#### 5.2.x.1	Channel bandwidths per operating bands for CA

Table 5.x.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n66A-n71A-n78A | CA_n41A-n66A CA_n41A-n71A CA_n41A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n66(2A)-n71A-n78A | CA_n41A-n66A CA_n41A-n71A CA_n41A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n41A-n66A-n71A-n78A(2A) | CA_n41A-n66A CA_n41A-n71A CA_n41A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66(2A)-n71A-n78A(2A) | CA_n41A-n66A CA_n41A-n71A CA_n41A-n78A CA_n66A-n71A CA_n66A-n78A CA_n71A-n78A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.2.x.3		UE co-existence studies

The coexistence studies have been captured into the constituent fallback modes in TR 38.716-03-02, there is no additional harmonic and intermodulation impact for the additional band receiver.

#### 5.2.x.4	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c  could reuse the values for CA_n41-n66-n71-n78 with 1UL that have been captured  into TR 36.716-04-01.

#### 5.2.x.5	REFSENS requirements

There is no need to specify additional MSD requirement for this UL CA configuration.

---End of changes---

# Reference

[1] 		RP-211194, “Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for DL 4 bands and 2UL bands”, Samsung