3GPP TSG-RAN WG4 Meeting #100-e                                                    	R4- 2113612

Online, 16th August – 27th August, 2021

Source:	Vodafone

Title:	TP for TR 37.717-31-11: DC_20-32-38_n1

Agenda item:			8.16.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-31-11 to include DC_20-32-38_n1.

The TIB,c and RIB values provided are derived from the relevant subsets already present in 36101 and 38101-3.

# 2. Reference

# 3. Text Proposal

<Start of Text Proposal>

### 5.1.x	DC_20-32-38_n1

#### 5.1.x.1	Configuration for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_20A-32A-38A_n1A | DC_20A_n8A DC_38A_n8A |

#### 5.1.x.2	∆TIB and ∆RIB values

Table 5.1.x.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-32A-38A_n1A | 20 | 0.3 |
|  | 38 | 0.5 |
|  | n1 | 0.5 |

Table 5.1.x.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_20A-32A-38A_n1A | 20 | 0 |
|  | 32 | 0 |
|  | 38 | 0 |
|  | n1 | 0 |

#### 5.1.1.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

<End of Text Proposal>