3GPP TSG-RAN WG4 Meeting #100-e	R4-2113621

Electronic Meeting, 16 Aug. – 27 Aug. 2021

Agenda Item:	9.12.2.3

Source:	Ericsson

Title:	Discussion on MMSE-IRC receiver for CRS interference

Document for:	Discussion

# 1	Introduction

During the last RAN4 #99e meeting, several agreements have been made on how to evaluate the impact from CRS interference on NR LTE spectrum sharing.

Companies agreed on following rate-matching schemes for evaluation:

- Baseline RM scheme for performance comparison with PDSCH CRS-IM - For scenario 1 with LTE and NR DSS: - Rel-15 serving cell CRS-RM - Rel-16 CRS-RM for 1 interference cell - Case A: the 1 interference cell with RM is always the first dominant interference, i.e., INR1 - Case B: the 1 interference cell with RM is NOT always the first dominant interference. Interested companies can provide simulation results for Case B. - e.g., INR of the 1 interference cell with RM is INR1 or INR2 with 50%: 50% probability. If the INR for the interference cell with RM is INR1, then the INR for the other interference cell is INR2, and vice versa. The INR levels for the two interference cells can be changed per [1000] slots. - The above example for Case B is optional, and other options for Case B are not precluded. - Case B is for initial simulation only, and FFS whether to consider it for performance requirement definition. - Rel-15 RB symbol level RM for 2 interference cells - For scenario 2 - No RM - Rel-15 CRS-RM for 1 interference cell - Case A: the 1 interference cell with RM is always the first dominant interference, i.e., INR1 - Case B: the 1 interference cell with RM is NOT always the first dominant interference. Interested companies can provide simulation results for Case B. - e.g., INR of the 1 interference cell with RM is INR 1 or INR2 with 50%: 50% probability. If the INR for the interference cell with RM is INR1, then the INR for the other interference cell is INR2, and vice versa. The INR levels for the two interference cells can be changed per [1000] slots. - The above example for Case B is optional, and other options for Case B are not precluded. - Case B is for initial simulation only, and FFS whether to consider it for performance requirement definition. - Rel-16 CRS-RM for 2 interference cells

And here are agreed CRS-IM schemes to be evaluated:

- Assumption on CRS-IM - Both CRS-IC and LLR weighting for the initial performance evaluation - FFS for performance requirements definition. - FFS NW assistant information existed or not, companies are encouraged to bring analysis with different options.

Companies are encouraged to evaluate the performance differences among those schemes above.

Besides this, there are still some open issues that need to be discussed.

Therefore, in this contribution, we are going to share our observations over the simulation results, and we also share our views on left open issues. We will try to give the direction of defining requirements of phase II as well.

# 2	Evaluations

Scenario 1 DSS

As discussed in the last meeting, there are 4 types of rate-matching schemes and 2 IM schemes that encouraged to be evaluated for this scenario:

- Rel-15 CRS-RM (Reference)

- Rel-16 CRS-RM for 1 interference cell case A

- Rel-16 CRS-RM for 1 interference cell case B (optional)

- Rel-15 RB symbol level RM for 2 interference cells

- CRS-IC

- LLR weighting

Figure 2-1 CRS configuration

Here we have evaluated the performance of these schemes above except the optional one, see the figure below. Note we set the overhead for TBS determination to 18 for PDSCH on the serving cell for all the scenarios above.

Figure 2-2 Performance of difference schemes under DSS scenario (MCS4)

Figure 2-3 Performance of difference schemes under DSS scenario (MCS13)

From the figures above we can tell that the performance of CRS-IC and LLR weighting are really close to that of several rate-matching schemes.

For both MCS4 and MCS13, the performance of CRS-IM is little bit better than the performance of the reference. The performance of rate-matching scheme 1 and 3 is worse than the performance of the reference.

Observation 1: CRS-IC and LLR weighting both have very close performance compared to that of those rate-matching schemes, especially that of reference (Rel-15 RM) in scenario 1

Observation 2: The performance of CRS-IC and LLR weighting is a little bit better than that of the reference in scenario 1

Observation 3: The performance of rate-matching scheme 1 and 3 are worse than that of the reference in scenario 1

Scenario 2 NR

As discussed in the last meeting, there are 4 types of rate-matching schemes and 2 IM schemes that encouraged to be evaluated for this scenario:

- No RM (Reference)

- Rel-15 CRS-RM for 1 interference cell case A

- Rel-15 CRS-RM for 1 interference cell case B (optional)

- Rel-16 CRS-RM for 2 interference cells

- CRS-IC

- LLR weighting

Here we have evaluated the performance of these schemes above except the optional one, see the figure below. Note we set the overhead for TBS determination to 18 for CRS-RM scenarios but no overhead for other scenarios (No RM, CRS-IC and LLR weighting). This results in the different maximum throughput.

Figure 2-4 Performance of difference schemes under NR scenario (MCS4)

Figure 2-5 Performance of difference schemes under NR scenario (MCS13)

From the figures above we can tell that CRS-IC and LLR weighting all have ~2dB gain for MCS4 and ~1dB gain for MCS13 compared to the reference (No IC). Meanwhile, the performance of CRS-IC and LLR weighting is really close to each other. Besides, we also found that the performance of the reference (no IC) is better than both Rel-15 RM and Rel-16 RM.

Observation 4: CRS-IC and LLR weighting both have 1~2 dB gain compared to the reference for scenario 2

Proposal 1: Consider CRS-IM for defining performance requirement

Proposal 2: Do not need to specify the CRS-IM schemes while defining requirements since CRS-IC and LLR weighting have very close performance

# 3	General issues

Scenario

For phase I studies, companies agreed on prioritize synchronous network scenario. As second priority, RAN4 could evaluate the feasibility and usefulness of the asynchronous network scenario and specify if feasible and useful.

In this case, we propose to study the asynchronous network scenario in the next RAN4 meeting (after RP #93 meeting)

Proposal 3: RAN4 studies the scenario of asynchronous network after RP #93 meeting

PDCCH CRS-IM

Whether to define PDCCH performance requirements depends on UE complexity to perform CRS mitigation before PDCCH decoding and how much impact will be caused due to neighboring cell LTE CRS interference, and also how much gain can be achieved while using IM schemes.

We are open for further discuss on current stage based on the evaluation results.

Proposal 4: Determine whether to consider having PDCCH performance requirements based on the evaluation of performance impact and gain

# 4	Reference receiver

Network assistance information

We don’t support any kinds of network assistance information. One reason is that this feature is up to UE implementation, including which and when the information is needed.

Another very important thing is that if no matter what kinds of network assistance information is agreed to be introduced, it won’t be implemented by BS side very soon. For example it could not be implemented in the next few releases, so that UEs that supporting this feature need to do the IM by themselves anyway. And by the time, finally, the (if) agreed network assistance information is updated on the BS side, there will be rather no LTE-NR spectrum sharing or even no LTE UE at all. Then the additional network assistance information has less meaning of being introduced.

In fact, UE with capability of doing IM can measure the RSRP of neighboring cells in order to determine whether to trigger the IM processing. Moreover, the blind detection for needed information can be done only once for each UE rather than doing it several times, so that there won’t be too much power consuming.

Proposal 5: Not to introduce any type of network assistance information

# 5	Interference model

PDSCH loading level on interference cell

During last RAN4 meeting, companies reached following agreement for PDSCH loading:

- PDSCH loading level on interference cell - In time domain, probability of occurrence of data transmission in interference cells: simulate 20%, and it is also encouraged to simulate 0%, 50% and 100%. - In frequency domain: full bandwidth allocation.

Our evaluations above are all based on the 20% PDSCH loading level on interference cell, i.e., LTE PDSCH is transmitted on 20% of subframes during the simulation. Besides this, we also have simulated other choices of loading level, see the result below:

Figure 5-1 Performance of 100% PDSCH loading level for scenario 1

Figure 5-2 Performance of 100% PDSCH loading level for scenario 2

Observation 5: Under 100% PDSCH load level, performance of rate-matching schemes and IM schemes are very close for scenario 1

Observation 6: under 100% PDSCH load level, performance of LLR weighting has ~2dB gain over the performance of CRS-IC and reference

Observation 7: Performance under 100% PDSCH load level is ~10dB worse than the performance under 20% PDSCH load level

Proposal 6: Consider only 20% PDSCH load for defining requirement

# 6	Target NR PDSCH parameters

MCS for target NR PDSCH

One left open issue is that whether to cover 64 QAM MCS19 for further evaluation or even defining requirements. We have evaluated the performance of 64QAM MCS19 and found that the performance of five schemes can not reached the max TP even in high SNR.

Observation 8: For scenario 2 with MCS19, performance of five schemes can not reached the Max TP even in high SNR

Proposal 7: Focusing on MCS4 and MCS13 in defining performance requirement in phase II

# 5	Summary

In this contribution, we share our simulation results and continue to discuss the remaining left open issues on UE demodulation requirements of MMSE-IRC receiver for suppressing inter-cell CRS interference.

Observation 1: CRS-IC and LLR weighting both have very close performance compared to that of those rate-matching schemes, especially that of reference (Rel-15 RM) in scenario 1

Observation 2: For MCS4, the performance of CRS-IC and LLR weighting has <1dB gain over that of the reference at SNR point of 70% max TP in scenario 1

Observation 3: For MCS13, no obvious gain has been observed comparing the performance of IM and rate-matching schemes in scenario 1

Observation 4: CRS-IC and LLR weighting both have 1~2 dB gain compared to the reference for scenario 2

Proposal 1: Consider CRS-IM for defining performance requirement

Proposal 2: Do not need to specify the CRS-IM schemes while defining requirements since CRS-IC and LLR weighting have very close performance

Proposal 3: RAN4 studies the scenario of asynchronous network after RP #93 meeting

Proposal 4: Determine whether to consider having PDCCH performance requirements based on the evaluation of performance impact and gain

Proposal 5: Not to introduce any type of network assistance information

Observation 5: Under 100% PDSCH load level, performance of rate-matching schemes and IM schemes are very close for scenario 1

Observation 6: under 100% PDSCH load level, performance of LLR weighting has ~2dB gain over the performance of CRS-IC and reference

Observation 7: Performance under 100% PDSCH load level is ~10dB worse than the performance under 20% PDSCH load level

Proposal 6: Consider only 20% PDSCH load for defining requirement

Observation 8: For scenario 2 with MCS19, performance of five schemes can not reached the Max TP even in high SNR

Proposal 7: Focusing on MCS4 and MCS13in defining performance requirement in phase II

# References

- R4-2108662, “WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR”, China Telecom

# Annex

Simulation assumption used for above evaluation

Table 1 NR Cell Parameters

| Parameters | Serving cell FDD |
| --- | --- |
| Network scenario | Sync |
| Number of carriers | single carrier |
| Carrier frequency | [2 GHz] |
| PDSCH allocation in frequency domain | Full PRB |
| Physical cell ID | 0 |
| Active DL BWP index | 1 |
| Channel bandwidth | 10MHz |
| TDD configuration | N/A |
| SCS | 15kHz |
| PDCCH allocation | Scenario 1: symbols #2 Scenario 2: symbols #0 and #1 |
| SSB position in burst | SSB index #0 |
| SSB periodicity | 20ms |
| TRS configuration | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Density 3, 1 port Periodicity 20 slots Offset: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| CSI-RS configuration | Not necessary of PDSCH Demod |
| PRB bundling size for target PDSCH | 2 |
| Antenna configuration | 4x2 4x4? |
| Rank | 1 |
| Overhead for TBS determination | Scenario 1-1,1-2,1-3, 2-2, 2-3, 2-4: 18 Scenario 1-4,1-5,1-6, 2-1, 2-5, 2-6: 0 |
| PDSCH DMRS ports | port 0 ~3 |
| Maximum number of OFDM symbols for DL front loaded DMRS | 1 |
| Number of additional DMRS | 1 |
| MCS | QPSK, MCS 4; 16QAM, MCS 13; (64QAM MCS 19) |
| Precoding granularity | 2 |
| PRB bundling size | 2 |
| PDSCH precoder | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination |
| Antenna Correlation | ULA Low |
| Propagation condition | TDLA30-10 |
| Time Offset relative to serving cell | 0 us |
| Traffic model | Full buffer |
| Number of HARQ Processes | 4 |
| Maximum HARQ transmission | 4 |

Table 2. NR PDSCH Parameters

| PDSCH configuration | Mapping type | Type A |
| --- | --- | --- |
|  | k0 | 0 |
|  | Starting symbol (S) | Scenario 1-1,1-2,1-3, 2-2, 2-3, 2-4: 3 Scenario 1-4,1-5,1-6, 2-1, 2-5, 2-6: 2 |
|  | Length (L) | Rel-15 or Rel-16 CRS-RM: 9 No CRS-RM (scenario 2): 12 |
|  | PDSCH aggregation factor | 1 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
|  | Resource allocation type | Type 0 |
|  | RBG size | Config2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Position of the first DM-RS for downlink | 3 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |

Table 3. LTE Interf. cell parameters

|  | Cell 0(DSS) | LTE Interf. cell 1 | LTE Interf. cell 2 |
| --- | --- | --- | --- |
| carrier centre subcarrier | Same as NR carrier centre subcarrier |  |  |
| BW(MHz) | 10 |  |  |
| Resource allocation | Random full band (50PRB) on/off model, proportional to the average resource utilization in the interfering cells; ON/OFF pattern depends on the Possion distribution |  |  |
| Non-full buffer interference Model | Interfering PDSCH transmissions in interfering cells are randomly & independently active over the full band with an activity in time domain equal on average to the targeted resource utilization |  |  |
| Average resource utilization | 20%, [0%, 50%, 100%] |  |  |
| CP Type | Normal |  |  |
| Time offset(us) | 0 | 3 | ​-1 |
| Frequency offset(HZ) | 0 | 300 | -100 |
| PDSCH MCS | Random 16QAM |  |  |
| Precoding | TM4 Random PMI |  |  |
| Rank | Randomly changing rank per allocated subband from subframe to subframe: 80% rank-1, 20% rank-2 |  |  |
| Number of antenna ports | 4 | 4 | 4 |
| v-shift | 0 | 1 | 2 |
| INR(dB) | / | 10.45 | 4.6 |
| Antenna Correlation | ULA Low |  |  |
| Note 1: No MBSFN is configured on LTE carrier |  |  |  |

TR 36.863 Table 7.2-1

TR 36.863 Table 7.2-1