3GPP TSG-RAN WG4 Meeting # 100-e	R4-2113727

Electronic Meeting, August. 16-27, 2021

Title:	TP to TR 38.717-03-01: Addition of CA_n66-n70-n71

Source:	Nokia, DISH Network

Agenda item:	8.9.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-03-01 to add CA_n66-n70-71(2A)

************************************* Start of TP*****************************************

## 6.x	CA_n66-n70-n71

### 6. x.1	Operating bands for CA

Table 6.7.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n66A-n70A-n71A | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

#### 6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported bandwidths per CA band combination for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n66A-n70A-n71(2A) |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  |  |  | n70 |  | 15 |  | 5 |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  |  |  | 30 |  |  |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n71 |  | See CA_n71(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

************************************* End of TP*****************************************