3GPP TSG-RAN WG4 Meeting #100-e	R4-2113745

# Electronic meeting, August 16th-27th, 2021

Source:		Ericsson

Title:			NTN: system parameters

Agenda item:		9.13.1.1

Document for:		Approval

# Introduction

The NTN WI ([1]) has been approved in RAN#88e meeting to specify requirements for the support of NTN. It has been revised in last RAN#92-e meeting [2].

In last RAN4 meeting, some system parameters were agreed ([3]) for the 2 considered bands. This contributions is summarizing those agreements and further discuss the remaining open issues.

# Discussion

## NTN bands

Based on the agreed WF ([3]) and our contributions ([4] and [5]), the first 2 NTN bands should be specified as proposed in the following table.

| NTN operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| s11 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| s2 | 1626.5 MHz – 1660 5 MHz | 1525 MHz – 1559 MHz | FDD |
| NOTE 1: Coexistence of terrestrial and satellite components shall be addressed following ITU Recommendation M.1036-6 and Resolution 212 (WRC-19 revision) |  |  |  |

## Channel Bandwidth

As agreed in ([3]), the NTN requirements will be specified for the following combination of BS/UE channel bandwidths, SCS and NTN bands as shown in the following table.

| NTN Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- |
|  | 15 | Yes | Yes | Yes | Yes |
| s1 | 30 |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |
|  | 15 | Yes | Yes | Yes | Yes |
| s2 | 30 |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |

## Channel arrangement

### Chanel spacing

The first NTN bands (s1 and s2) would support a 100 kHz channel raster. The nominal channel spacing between two adjacent NTN carriers in those NTN bands be defined according to the following:

▪	Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

where BWChannel(1) and BWChannel(2) are the BS channel bandwidths of the two respective NTN carriers.

### Channel raster

The NR-ARFCN was specified to cover any band between 0 MHz and 100 GHz.

For NTN, it’s questionable if this frequency range should still be considered, very small chunks of spectrum (less than 5 MHz) have been allocated to Mobile Satellite Service as primary service by ITU-R below 1500 MHz.

If might then be possible to simplify the ARFCN values allocation for NTN, reducing the range of values, this would need further discussion, considering other companies’ view.

### Synchronization raster

Similarly to NTN ARFCN, it would be possible to simply the NR GSCN allocation, this should be aligned with the outcomes of NTN ARFCN discussion.

Proposal: Further study if NTN ARFCN and GSCN should be simplified, reducing the range of values.

# Conclusion

In this contribution, we further discussed the NTN system paraemeters and made the following proposal:

Proposal1: Specify the following system parameters for NTN s1 and s2 bands:

| NTN operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| s11 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| s2 | 1626.5 MHz – 1660 5 MHz | 1525 MHz – 1559 MHz | FDD |
| NOTE 1: Coexistence of terrestrial and satellite components shall be addressed following ITU Recommendation M.1036-6 and Resolution 212 (WRC-19 revision) |  |  |  |

| NTN Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- |
|  | 15 | Yes | Yes | Yes | Yes |
| s1 | 30 |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |
|  | 15 | Yes | Yes | Yes | Yes |
| s2 | 30 |  | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes |

Proposal2: Further study if NTN ARFCN and GSCN should be simplified, reducing the range of values.

# References

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-211557, Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2108099, WF on NTN solutions Part1, Thales

- R4-2113740, NTN: General, Ericsson

- R4-2113741, NTN: Regulatory information, Ericsson