3GPP TSG-RAN WG4 Meeting # 100-e	R4-2113776

# Electronic Meeting, August. 16-27, 2021

Title: 	Simulation results on CRS-IM receiver

Source: 	Huawei, HiSilicon

Agenda item:	9.12.2.3

Document for:	Discussion

Background

In RAN4 #99-e meeting, WF R4-2108662 is approved and corresponding initial simulation assumptions are determined. In this paper, we provide our initial simulation results.

Simulation results

The simulation assumptions are shown in follows:

- General: - Scenario 1: - Rel-15 serving cell CRS-RM - Scheme #1: Rel-16 CRS-RM for 1 interference cell (case A the rate matched CRS is always the first dominant interference) - Scheme #2: Rel-16 CRS-RM for 1 interference cell (case B the rate matched CRS is NOT always the first dominant interference) - Scheme #3: Rel-15 RB symbol level CRS-RM for 2 interference cells - Scheme #4: CRS-IM using CRS-IC WITH NW assist - Scheme #5: CRS-IM using CRS-IC WITHOUT NW assist - Scheme #6: CRS-IM using LLR weighting WITH NW assist - Scheme #7: CRS-IM using LLR weighting WITHOUT NW assist - Scenario 2: - No RM - Scheme #1: Rel-15 CRS-RM for 1 interference cell (case A the rate matched CRS is always the first dominant interference) - Scheme #2: Rel-16 CRS-RM for 1 interference cell (case B the rate matched CRS is NOT always the first dominant interference) - Scheme #3: Rel-16 CRS-RM for 2 interference cells - Scheme #4: CRS-IM using CRS-IC WITH NW assist - Scheme #5: CRS-IM using CRS-IC WITHOUT NW assist - Scheme #6: CRS-IM using LLR weighting WITH NW assist - Scheme #7: CRS-IM using LLR weighting WITHOUT NW assist - CBW/SCS - FDD 10MHz/15kHz - Number of interfering cells modelled in the simulation - 2 - Interference power level - with INR1 = 10.45 dB and INR2 = 4.6 dB - CRS pattern: - For scenario 1, v-shift = 0, 1, 2 for the serving and two interference cells - For scenario 2, v-shift = 1, 2 for the two interference cells - PDSCH loading level on interference cell - In time domain, probability of occurrence of data transmission in interference cells: simulate 20%, and it is also encouraged to simulate 0%, 50% and 100%. - Time offset and frequency shift for sync network - Time offset: The serving cell is 3 us and -1 us for interfering cell 1 and cell 2 respectively - Frequency shift: The serving cell is 300 Hz and -100 Hz for interfering cell 1 and cell 2 respectively. - Transmission rank - 80% and 20% probability for rank 1 and rank 2 transmission in the interfering cell(s). - Modulation scheme for the interference PDSCH - 16 QAM randomly modulated symbols in the interfering PDSCH when exists - Precoding scheme for the interference PDSCH - Random precoding - MBSFN configuration for the interference LTE cell - No MBSFN - Rank for target NR PDSCH - Rank 1 - MCS for target NR PDSCH - Cover QPSK MCS 4 and 16QAM MCS 13 for initial simulation - Precoding model for target NR PDSCH - Random precoding (Single panel Type 1) with PRB bundling size is 2 with PRB bundling type is static - HARQ process number for target NR PDSCH - 4 for FDD, further discuss for TDD - PDSCH mapping type and DMRS configuration for target NR PDSCH - PDSCH mapping type A with full PRB allocation, use DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data. - Start symbol (S) and symbol length (L) for the target PDSCH - For the scenario that Rel-15 or Rel-16 CRS-RM is configured (for scenario 1 and 2): Focus on S = 3, L = 9. Interested companies can bring analysis for S = 3, L = 11. - For the scenario that CRS-RM is not configured (for scenario 2): S = 2, L = 12 - Overhead for TBS determination for the target PDSCH - For the scenario that Rel-15 or Rel-16 CRS-RM is configured (for scenario 1 and 2): 18 - For the scenario that CRS-RM is not configured (for scenario 2): 0 - NR Carrier number - Focus on NR single carrier scenario - Tx antenna and LTE CRS port number - For initial simulation, prioritize 4 CRS ports and interested companies can provide results for 2 CRS ports case. - Rx antenna number - 2Rx and 4Rx - Propagation condition - TDLA30-10 and ULA low

Scenario 1

Simulation results for scenario 1 are captured in Figure 2-1:

- 2RX , MCS4

- 4RX , MCS4

- 2RX , MCS13

- 4RX , MCS13

Figure 2-1 Simulation results for scenario 1

Summary of simulation results for scenario 1 are captured in Table 2-2:

Table 2-2: Summary of simulation results for scenario 1

|  | MCS 4 |  |  |  | MCS 13 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4T2R |  | 4T4R |  | 4T2R |  | 4T4R |  |
|  | SNR(dB)@70% of max TP | SNR gain | SNR(dB)@70% of max TP | SNR gain | SNR(dB)@70% of max TP | SNR gain | SNR(dB)@70% of max TP | SNR gain |
| Rel-15 serving cell CRS-RM(Baseline) | 2.1 | / | -1.16 | / | 10.68 | / | 7.18 | / |
| Scheme #1 | 0.3 | 1.8 | -3.11 | 1.95 | 8.97 | 1.71 | 5.36 | 1.82 |
| Scheme #2 | 3.3 | -1.2 | 0.04 | -1.2 | 12.15 | -1.47 | 8.5 | -1.32 |
| Scheme #3 | 0.4 | 1.7 | -3.15 | 1.99 | 10.56 | 0.12 | 6.66 | 0.52 |
| Scheme #4 | -1.5 | 3.6 | -4.92 | 3.76 | 7.44 | 3.24 | 3.46 | 3.72 |
| Scheme #5 | -1.6 | 3.7 | -4.94 | 3.78 | 7.41 | 3.27 | 3.47 | 3.71 |
| Scheme #6 | -0.4 | 2.5 | -3.97 | 2.81 | 8.28 | 2.4 | 4.43 | 2.75 |
| Scheme #7 | -0.4 | 2.5 | -3.92 | 2.76 | 8.33 | 2.35 | 4.45 | 2.73 |

Based from our simulation results, we have following observations for scenario 1:

Observation 1: Rel-16 CRS-RM for 1 cell which is dominate interference has the best performance among all RM schemes for both MCS 4 and MCS 13 and this is the best RM scheme which has 1.71~1.95 dB performance gain over Rel-15 serving cell CRS-RM.

Observation 2: Rel-16 CRS-RM for 1 cell which is not always dominate interference has the worst performance among all RM schemes and has negative gain over Rel-15 serving cell CRS-RM.

Observation 3: Rel-15 RB symbol level RM has the same performance with Rel-16 CRS-RM for MCS=4, but there is severe performance degradation for MCS=13

Observation 4: CRS-IC has the best performance among all the schemes and has 3.2~3.7 dB performance gain.

Observation 5: LLR weighting has 2.4~2.8 dB performance gain over Rel-15 serving cell CRS-RM.

Observation 6: IC and LLR weighting without network assistance has the same performance with that with network assistance.

Scenario 2

Simulation results for scenario 2 are captured in Figure 2-2:

- 2RX , MCS4

- 4RX , MCS4

- 2RX , MCS13

- 4RX , MCS13

Figure 2-2 Simulation results for scenario 2

Summary of simulation results for scenario 2 are captured in Table 2-3:

Table 2-3: Summary of simulation results for scenario 2

|  | MCS 4 |  |  |  | MCS 13 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4T2R |  | 4T4R |  | 4T2R |  | 4T4R |  |
|  | SNR(dB)@70% of max TP | SNR gain | SNR(dB)@70% of max TP | SNR gain | SNR(dB)@70% of max TP | SNR gain | SNR(dB)@70% of max TP | SNR gain |
| No -RM(Baseline) | 1.06 | / | -2.30 | / | 9.83 | / | 6.62 | / |
| Scheme #1 | None | / | None | / | None | / | None | / |
| Scheme #2 | None | / | None | / | None | / | None | / |
| Scheme #3 | None | / | None | / | None | / | None | / |
| Scheme #4 | -1.72 | 2.78 | -5.12 | 2.82 | 7.02 | 2.81 | 3.47 | 3.16 |
| Scheme #5 | -1.66 | 2.72 | -5.10 | 2.8 | 7.15 | 2.69 | 3.8 | 3.14 |
| Scheme #6 | -0.83 | 1.89 | -3.52 | 1.22 | 7.78 | 2.05 | 4.86 | 1.76 |
| Scheme #7 | -0.90 | 1.96 | -3.56 | 1.26 | 7.86 | 1.98 | 4.86 | 1.77 |

Based from our simulation results, we have following observations for scenario 2:

Observation 7: The throughput of all RM schemes can’t reach 70% of peak date rate.

Observation 8: Rel-15 CRS-RM for 1 cell which is always dominate interference has the best performance among all RM schemes.

Observation 9: Rel-15 CRS-RM for 1 cell which is not always dominate interference has the worst performance among all RM schemes and has negative gain over baseline.

Observation 10: CRS-IC has the best performance among all the schemes and has 2.8~3.2 dB performance gain over baseline.

Observation 11: LLR weighting has 1.76~2.05 dB performance gain over baseline.

Observation 12: IC and LLR weighting without network assistance has the same performance with that with network assistance.

Conclusion

In this paper, we provide our simulation results for CRS-IM receiver. The observations are:

For scenario 1:

  - Observation 1: Rel-16 CRS-RM for 1 cell which is dominate interference has the best performance among all RM schemes for both MCS 4 and MCS 13 and this is the best RM scheme which has 1.71~1.95 dB performance gain over Rel-15 serving cell CRS-RM.

  - Observation 2: Rel-16 CRS-RM for 1 cell which is not always dominate interference has the worst performance among all RM schemes and has negative gain over Rel-15 serving cell CRS-RM.

  - Observation 3: Rel-15 RB symbol level RM has the same performance with Rel-16 CRS-RM for MCS=4, but there is severe performance degradation for MCS=13

  - Observation 4: CRS-IC has the best performance among all the schemes and has 3.2~3.7 dB performance gain.

  - Observation 5: LLR weighting has 2.4~2.8 dB performance gain over Rel-15 serving cell CRS-RM.

  - Observation 6: IC and LLR weighting without network assistance has the same performance with that with network assistance.

For scenario 2:

  - Observation 7: The throughput of all RM schemes can’t reach 70% of peak date rate.

  - Observation 8: Rel-15 CRS-RM for 1 cell which is always dominate interference has the best performance among all RM schemes.

  - Observation 9: Rel-15 CRS-RM for 1 cell which is not always dominate interference has the worst performance among all RM schemes and has negative gain over baseline.

  - Observation 10: CRS-IC has the best performance among all the schemes and has 2.8~3.2 dB performance gain over baseline.

  - Observation 11: LLR weighting has 1.76~2.05 dB performance gain over baseline.

  - Observation 12: IC and LLR weighting without network assistance has the same performance with that with network assistance.

Reference

- WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR. China Telecom.