3GPP TSG-RAN WG4 Meeting #100-e	R4-2113799

# Electronic Meeting, 16th - 27th Aug, 2021

Title: 	Discussion on PRACH demodulation requirements for FR2 HST

Source: 	Huawei, HiSilicon

Agenda item:	9.9.5.3.3

Document for:	Discussion

Background

During RAN#99-e meeting, WF [1] on Demodulation requirement for FR2 HST was approved. In this contribution, we share our views about PRACH demodulation requirements for FR2 HST.

Discussion

Test setup

Timing offset configuration

- Timing offset configuration - Option 1: Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e., 0.8us - Option 2: Update the timing offset configuration based on the largest expected cell radius, i.e., derived from scenario B - Note - Scenario A (Ds=700m, Dmin=10m), cell radius = 700m - Scenario B (Ds=700m, Dmin=150ms), cell radius = 716m

For Option 1, the Rel-15 FR2 timing offset configuration is reused, i.e. maximum 0.8us. For Option 2, the maximum timing offset can be about 2.4 us corresponding to the maximum cell radius derived from Scenario B. In Rel-16 FR1 HST, 0.8 us is reused from the non-HST test setup although the maximum cell radius can be about 716m that is same as HST FR2. Also as per the simulation results in section 3, there is very small difference for two options. In our view, we think both two options are feasible and we slightly prefer to reuse Rel-15 FR2 timing offset configuration for PRACH, i.e., 0.8us.

Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e., 0.8us.

Simulations

Here we provide the simulation results for FR2 HST PRACH for alignment.

Table 3-1 Ideal simulation results for FR2 HST PRACH

| Case number | Timing offset | SNR (dB) |
| --- | --- | --- |
| 1 | [0,0.1...0.8] | -12.48 |
| 2 | [0,0.1...2.4] | -12.14 |

Proposals

In this contribution, we discuss on PRACH performance requirements for NR HST FR2. Our observations and proposals are:

- Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e., 0.8us.

Reference

R4-2108637, WF on Demodulation requirement for FR2 HST, RAN4#99-e, Samsung