3GPP TSG-RAN WG4 Meeting #100-e	R4-2113847

Electronic Meeting, August 16-27, 2021

Title: 	Simulation assumption for measurement accuracy for RedCap UE with 1 RX

Source: 	Huawei, HiSilicon

Agenda item:	9.20.3.2

Document for:	Approval

Introduction

The revised WID on support of reduced capability (RedCap) NR devices has been approved in [1]. RAN4 has started work on the support of RedCap NR devices, where the related study item has been concluded in TR 38.875. One objective in the WI is UE complexity reduction. One of UE complexity reduction features is reduced minimum number of RX branches.

- Reduced minimum number of Rx branches: - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - A means shall be specified by which the gNB can know the number of Rx branches of the UE.

This contribution provides the link-level simulation assumption for 1Rx branches for measurements accuracy.

Simulation Assumption

The following measurements are to be studied:

- RSRP based on NR-SSS

- RSRQ based on NR-SSS

- SINR based on NR-SSS

Two cells are simulated, interfering Cell 1 (unknown to the UE) and measured Cell 2 (known to the UE).

Table 1: General parameters

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 4 GHz | 30 GHz |
| System bandwidth | 5 MHz; 10 MHz | 50 MHz; 100 MHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes | No / Yes |
| DRX | No | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions | 1 tx or single layer transmissions |
| UE receive antennas | 1RX | 1RX |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 5; other number of samples may also be studied upon a need | 5; other numbers of samples may also be studied upon a need |
| SS burst set configuration in Cell 1 and Cell 2: |  |  |
| - Subcarrier spacing | 15 kHz; 30 kHz | 120 kHz; 240 kHz |
| - Number of SS blocks per SS burst set | 1 | 1 |
| - SS burst periodicity | 5 ms | 5 ms |
| - Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| - SS block BW | 20 PRBs | 20 PRBs |
| - Actual SS block transmissions | always transmitted | always transmitted |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS and NR-SSS sequences | - | To be indicated by companies | To be indicated by companies |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | 3 ms |
| SNR | dB | 6 dB | 1 dB |
| Es/IoT (calculated from SNR) | dB | N/A | -5.97 dB |
| Propagation conditions | - | AWGN, EPA5, ETU30, ETU70, CDL-A/B/C Note, TDL-A/B/C Note |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |

Performance Metric

5%, 50% and 95% CDF of delta-RSRP shall be calculated by using the following equation:

- Delta RSRP = Measured RSRP – Ideal RSRP.

- Measured RSRP is defined as the true value assuming real channel estimation.

Reference

- RP-211038, Revised WID on support of reduced capability NR devices, Ericsson