3GPP TSG-RAN WG4 Meeting#100-e                               R4-2113924

E-meeting,16th Aug– 27th Aug, 2021

Agenda item:	9.16.6

Source: 	ZTE Corporation

Title: 	Initial coexistence simulation results for 52.6-71GHz

Document for:	Discussions

Introduction

In the previous RAN4 meetings, there were some initial discussions how to define ACLR/ACS requirement for 52.6-71GHz and some tentative simulation assumption was proposed in the approved WF [1], therefore based on the latest simulation assumption, some initial simulation results for indoor scenario are provided in the this contribution, for dense urban scenario, simulation results will be provide later on.

Discussion

## 2.2. Indoor scenario

### 2.2.1DL

Table 1. simulation results for downlink in indoor scenario

| Case | ACIR [dB] | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60GHz, 100MHz | Average throughput loss | 14.19 | 8.38 | 4.59 | 2.27 | 0.93 | 0.25 | 0.04 | 0.01 |
|  | Cell edge through loss | 57.95 | 40.29 | 24.04 | 11.67 | 3.51 | 0.73 | 0.12 | 0.01 |
| 60GHz, 400MHz | Average throughput loss | 14.42 | 8.46 | 4.56 | 2.21 | 0.88 | 0.26 | 0.05 | 0.01 |
|  | Cell edge through loss | 54.33 | 36.79 | 21.30 | 9.61 | 2.59 | 0.11 | 0.08 | 0.00 |
| 70GHz, 100MHz | Average throughput loss | 5.86 | 3.45 | 1.88 | 0.94 | 0.37 | 0.09 | 0.02 | 0.00 |
|  | Cell edge through loss | 45.24 | 32.15 | 20.52 | 12.40 | 5.62 | 1.29 | 0.13 | 0.00 |
| 70GHz, 400MHz | Average throughput loss | 6.00 | 3.50 | 1.93 | 0.95 | 0.37 | 0.09 | 0.01 | 0.00 |
|  | Cell edge through loss | 46.57 | 34.86 | 23.57 | 13.41 | 5.94 | 1.44 | 0.18 | 0.02 |

### 2.2.2 UL

Table 2. simulation results for uplink in indoor scenario

| sCase | ACIR [dB] | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60GHz, 100MHz | Average throughput loss | 0.32 | 0.10 | 0.03 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | Cell edge through loss | NaN | NaN | NaN | NaN | NaN | NaN | NaN | NaN |
| 60GHz, 400MHz | Average throughput loss | 0.13 | 0.04 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | Cell edge through loss | NaN | NaN | NaN | NaN | NaN | NaN | NaN | NaN |
| 70GHz, 100MHz | Average throughput loss | 0.22 | 0.08 | 0.03 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | Cell edge through loss | NaN | NaN | NaN | NaN | NaN | NaN | NaN | NaN |
| 70GHz, 400MHz | Average throughput loss | 0.09 | 0.03 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | Cell edge through loss | NaN | NaN | NaN | NaN | NaN | NaN | NaN | NaN |

## 2.2. Dense urban

### 2.1.1DL

TBD

### 2.1.2 UL

TBD

## 2.3. Comparison between updated results and TR 38.803

Based on the simulation results in section 2.2 for indoor scenario and data from 38.803, it seems that more stringent requirement are needed for 52.6-71GHz due to different antenna configuration and lower output power limit.

Table 5.5-9S: Interpolated ACIR values for DL to meet the 5% throughput loss criteria at 70GHz

| Scenario |  | Indoor |  | Dense urban |  |
| --- | --- | --- | --- | --- | --- |
| NF [dB] |  | 13 | 15 | 13 | 15 |
| ZTE | Average | 9.25 | 9.10 | 5.00 | 5.00 |
|  | 5%-tile | 19.21 | 18.13 | NA | NA |

Conclusions

In this contribution, some initial simulation results for indoor scenario are provided in the this contribution and observation are made as following:

Observation: more stringent DL ACIR requirement are needed for 52.6-71GHz due to different antenna configuration and lower output power limit.

References

- R4-2107915	WF on 60 GHz coexistence, ACLR, and ACS, approved.