3GPP TSG-RAN WG4 Meeting#100-e                               R4-2113926

E-meeting,16th Aug– 27th Aug, 2021

Agenda item:	9.18.2

Source: 	ZTE Corporation

Title: 	Discussion on phase discontinuity and power inconsistency tolerance across different repetitions

Document for:	Approval

Introduction

In the last RAN4 meeting, there were extensive discussions on phase discontinuity for NR coverage enhancement and reply LS [2] has been sent back to RAN1. However there are also some remaining open issues as listed in WF [1] left for further discussions, therefore in this contribution, we want to share some further considerations.

Discussion

## 2.1. Non-zero un-scheduled gap

For non-zero unscheduled gap, the following issues needs further discussion.

- Whether off power requirement can be met on the un-scheduled symbols

- The length of non-zero un-scheduled gap

In the last meeting, the feasibility of phase continuity and power consistency for non-zero un-scheduled gap case for a gap 13 symbols are confirmed feasible,however the gap could be extended up to 14OS or 1ms for different SCS is still FFS.

- Whether maximum X can be 14OS or 1ms for different SCS FFS

- Further study if new RF requirements for UE during the gap are needed.

Firstly, in our understandings, the maximum gap for phase continuity and power consistency should be absolute time domain length regardless of the supported SCS at least for FR1, since the potential factors might impact the phase continuity and power consistency should mainly come from RF part where SCS is usually not visible at all, therefore we think that 1ms for different SCS is more reasonable configuration instead of 14OS or 13 OS related with the configured SCS.

- Proposal 1: the length of non-zero un-scheduled gap could be up to 1ms for different SCS.

In addition, in last RAN4 meeting, there were some discussions how to define the transmit OFF power during the gap period, some candidate options are listed as following:

For transmit power on the gap symbols that less than or equal to 1ms, RAN4 down select solution from following options:

- Option 1: define new transmit off power for gap symbols less than or equal to 1ms explicitly for Rel-17 coverage enhancement case

- Option 2: RAN4 do not introduce new transmit off power

- Option 3: The existing OFF power requirement apply to the un-scheduled gap

Based on our understandings, during the non-scheduled gap, there are different approaches to simulate OFF status at UE side. E.g.

1) to keep PA operating point unchanged and PLL configuration unchanged and set power level of baseband at very low level to simulate the OFF status;

- to keep PA operating point unchanged and PLL configuration unchanged and set the power level of Tx chain as same as previous PUCCH/PUSCH transmission and set the attenuator if applicable as maximum level to simulate the OFF status (changing the front-end attenuation level might also introduce some phase discontinuity, need to double check whether this phase discontinuity is revertible).

- The cons of the above two approaches might introduce additional hardware implementation efforts, however with these approaches, UE OFF power could be still guaranteed which could well protect other surrounding UE’s receiver performance if there are downlink symbols in-between different repetitions.

- If exceptions for OFF power are allowed during non-zero unscheduled, then as mentioned before, other surrounding UE’s receiver performance cannot be guaranteed if there are downlink symbols scheduled in-between different PUSCH/PUCCH repetitions. Even if there are no downlink symbols scheduled in between different repetitions, then UE to transmit signal without network scheduling will also increase the harmful and unnecessary co-channel interference for the whole network.

- Based on the above considerations, OFF power requirement for non-zero un-scheduled gap should still be guaranteed, however considering the less measurement time for OFF power compared with Rel-15/16, then some extended testing uncertainty would be considered in RAN5.

- Proposal 2: OFF power requirement for non-zero un-scheduled gap should still be guaranteed and some extended testing uncertainty could be considered due to less measurement time compared with Rel-15/16.

## 2.2. Non-zero gap with other signals/channels for the UE

- In the last RAN4 meeting, there were two following scenarios for non-zero gap with other signals/channels identified for further discussions.

- Scenario 1: if the other scheduled signals/channels during the non-zero gap have the same settings in antenna port, occupied PRBs and UL power than the repeated transmission signals/channels

- Whether it is beneficial to define a Guard period for scenario 1:

- Option 1: In the case of different channel in between two repetitions, a guard period before returning to the repetitions is defined. Length of guard period is Y is FFS but shall not exceed 2 symbols.

- Option 2: FFS. It is not clear why guard period can help make the phase aligned with before transmission.

- For the scenarios 1, since the other scheduled signals/channels have same antenna port, occupied PRBs and UL power than the repeated transmissions signals, we don’t see the motivation to have such kind of guard period since there should be also no RF frequency retuning/RF transceiver reconfiguration. If there were some RF frequency retunning due to different frequency position of allocated PRBs, then phase continuity cannot been guaranteed anymore since PLL resetting might introduce some unknown phase errors.

- Proposal 3: support the option 2 for scenario 1 Non-zero gap with other signals/channels for the UE.

- Scenario 2: If the other scheduled signals/channels during the non-zero gap have the different settings in antenna port, occupied PRBs or UL power than the repeated transmission signals/channels

- Option 1: a guard period before returning to the repetitions is defined. Guard allows also different RBs and power levels for the other channels as agreed to be studied in the previous WF. Length of guard period is Y is FFS but shall not exceed 2 symbols.

- Option 2: Phase continuity and power consistency cannot be guaranteed.

- Option 3: Further investigation is needed

- If the phase discontinuity for non-zero gap with other uplink transmission is within such tolerance value, this case can be supported.

- If the antenna port, occupied PRBs or UL power has been changed for during the non-zero gap, then phase continuity and power consistency cannot be guaranteed based on the related discussions, therefore we propose to not consider not consider non-zero gap with other signals/channels with different configurations.

- Proposal 4: not consider non-zero gap with other signals/channels with different configurations (e.g. power, PRB content) for UE in Rel-17.

## 2.3. TA adjustment impact on phase continuity

In the last RAN4 meeting, there were some initial discussion on TA adjustment impacts and phase continuity and some issues were identified as following:

For RAN1 Question 1 on TPMI change and Question 2 on TA impact: there are transmission timing errors associated with TA adjustment and UE uplink timing autonomous adjustments. Transmission timing errors cause the phase change.

- For network commanded TA adjustments:

- It is known by both gNB side and UE side.

- FFS how to handle the transmission timing error in TA inaccuracy.

- For UE autonomous adjustments:

- FFS whether the autonomous adjustments can be handled by UE or BS and how to handle.

We think that TA adjustment between different repetitions could be avoided and not encouraged since this might introduce additional phase discontinuity due to TA command inaccuracy.

For UE autonomous adjustment error, this potential error is transparent to network side, therefore we think that this should be left up to UE implementation.

Proposal 5: TA adjustment between different repetitions could be avoided and not encouraged and UE autonomous adjustment error should be left up to UE implementation.

## 2.4. Required tolerance of the phase continuity and amplitude consistency

In the last RAN4 meeting, the majority of interested companies support to have some LLS to evaluate the acceptable tolerance from network performance perspective. Therefore in this contribution, the following scenarios are evaluated to check the impacts of phase discontinuity and power inconsistency on the gNB performance and detailed simulation assumptions are listed in Annex 5.

Table 1. evaluation cases for phase continuity and power consistency

| Scenario | Configurations | Note |
| --- | --- | --- |
| Case 1: Phase discontinuity across different repetitions | 700MHz/4GHz, 2 repetition, Baseline: 0 Relative Phase error:10: 20: 150 | Relative phase discontinuity is added on top of 2nd transmission repetition |

Figure 1. The performance impact of phase error on joint channel estimation for PUSCH for 700MHz

Figure 2. The performance impact of phase error on joint channel estimation for PUSCH for 4GHz

Based on initial simulation results in Figure 1, it can be found that 150o phase error will cause significant performance degradation, the acceptable phase error between different repetitions should be within 10o-30o.

Proposal 6: propose the phase error between different repetitions within 10o-30o.

Table 2. evaluation cases for the power consistency

| Scenario | Configurations | Note |
| --- | --- | --- |
| Case 2: Power inconsistency across different repetitions | 700MHz/4GHz, 2 repetition, Baseline : 0 Relative Amplitude error: 0: 0.5: 2dB | Relative power inconsistency is added on top of 2nd transmission repetition |

Figure 3. The performance impact of amplitude error on gNB receiver performance for 700MHz

Figure 4. The performance impact of amplitude error on gNB receiver performance for 4GHz

Based on initial simulation results in Figure 2, it can be found that 0.5dB amplitude error will also cause significant performance degradation (e..g at least 1.5-2dB deterioration @10% BLER), the acceptable phase error between different repetitions should be less than 0.5dB.

Proposal 7: propose the amplitude error between different repetitions less than 0.5dB.

Conclusion

- In this contribution, we shared some further analysis on the remaining open issues listed in the approved WF [1] and some proposals and observations are made as following:

- Proposal 1: the length of non-zero un-scheduled gap could be up to 1ms for different SCS.

- Proposal 2: OFF power requirement for non-zero un-scheduled gap should still be guaranteed and some extended testing uncertainty could be considered due to less measurement time compared with Rel-15/16.

- Proposal 3: support the option 2 for scenario 1 Non-zero gap with other signals/channels for the UE.

- Proposal 4: not consider non-zero gap with other signals/channels with different configurations (e.g. power, PRB content) for UE in Rel-17.

- Proposal 5: TA adjustment between different repetitions could be avoided and not encouraged and UE autonomous adjustment error should be left up to UE implementation.

Proposal 6: propose the phase error between different repetitions within 10o-30o.

Proposal 7: propose the amplitude error between different repetitions less than 0.5dB.

References

- R4-2107881,WF on phase continuity and power consistency for PUCCH and PUSCH repetition, approved

- R4-2107880,Reply LS on PUCCH and PUSCH repetition, approved.

- R4-2111706, LS on joint channel estimation for PUSCH and PUCCH, approved.

- R4-2105477,Email discussion summary for [98-bis-e][140] NR_cov_enh

Annex: Simulation assumptions

- Table A-1 Simulation assumption for different DMRS patterns in the time domain

| Parameter | Value |
| --- | --- |
| - Carrier frequency | - 700MHz |
| - Channel model | - TDL-C (delay spread: 300ns) as in 38.901 for Rural scenario |
| - UE speed | - 3 km/h |
| - BS antenna configuration | - 4Rx |
| - UE antenna configuration | - 1 Tx |
| - System bandwidth | - 20 MHz |
| - Sub-carrier spacing | - 15 kHz |
| - Occupied RB | - 4 |
| - MCS | - 0/1 |
| - Waveform | - DFT-s-OFDM |
| - Number of OS per repetition | - 14 (PUSCH mapping type A) |
| - DMRS overhead | - 1/2 DMRS symbols |
| - Number of repetitions | - 8 |
| - Frequency hopping | - Disabled |
| - Channel estimation | - Practical |
| - Receiver type | - MMSE |

- Table A-2 Simulation assumption for different DMRS patterns in the time domain

| Parameters | Values |  |
| --- | --- | --- |
| - Scenario | - Urban , O2I | - Rural, O2I |
| - Duplexing scheme and frequency | - 4GHz TDD | - 4GHz TDD |
| - Frame structure for TDD | - DDSUU (S: 10D:2G:2U) for 4GHz |  |
| - Transmission bit rate for data channel (bit/s) | - 1 Mbps for eMBB | - 100 kbps for eMBB |
| - Subcarrier spacing | - 30kHz | - 30kHz for TDD |
| - Channel model for link-level simulation | - TDL-C | - TDL-C |
| - Pathloss model (select from LoS or NLoS) | - NLos | - NLos |
| - Delay Spread | - 300ns | - 300ns |
| - UE velocity | - 3 km/h | - 3 km/h for O2I |
| - Number of RF chains for BS | - 4 | - 4 |
| - Number of UE antennas | - 1 | - 1 |
| - Number of RF chains for UE | - 1 | - 1 |
| - Number of occupied RBs | - 30 | - 4 |
| - MCS | - MCS4 | - MCS2 |
| - DMRS overhead | - Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data |  |
| - Frequency hopping | - Disabled |  |
| - Channel estimation | - Practical |  |
| - Receiver type | - MMSE |  |
| - BLER Target | - 10% iBLER for eMBB |  |
| - Waveform | - DFT-s-OFDM |  |
| - Number of repetitions | - 2 repetitions with JCE |  |