3GPP TSG-RAN WG4 Meeting # 100-e 	    R4-2114046

Electronic Meeting 16th August – 27th August 2021

Agenda Item:	6.1				NR Positioning Support
6.1.6.2.3.3		gNB Rx-Tx time difference requirements

Source: 	Ericsson

Title:	gNB Rx-Tx time difference requirements

Document for:	Discussion

# Introduction

As discussed in RAN4#99-e, further data points were found to be crucial to be able to define the gNB Rx-Tx measurement accuracy requirements, as depicted in the last meetings way forward [1]. Those additional data points are found to be the lowest possible RB allocation per bandwidth group that can be configured. To ensure that the worst case is being addressed properly the corner cases, i.e. lowest possible number of RBs per agreed RB bandwidth range were simulated and are taken into account when defining gNB Rx-Tx measurement accuracy requirements.

Furthermore, comparing companies’ results for each configuration, the obtained accuracy figures from simulation still can vary to a degree of high standard deviation, therefore an additional step guaranteeing margin after averaging the accuracy figures is proposed to take into account any possible high standard deviation across companies’ results.

# Discussion gNB Rx-Tx time difference analysis

After identifying missing simulation data points from [1], the simulations carried out resulted in measurement accuracy figures depicted in Table 1. Those results are within the expected range of previous results addressing the RB bandwidth range for which the accuracy figures would have been implemented, only differing a few Tc, if any.

Table 1 Additional data points of simulated gNB TOA accuracy for low threshold of band width range accuracy

| FR | BW (MHz) | SCS (kHz) | SRS BW (RB) | FFT size | Sampling rate (Tc) | UL-SRS-NumSymbols | UL-SRS-CombSizeN | Accuracy @ 90% (Tc), SINR >= -13dB | Accuracy @ 90% (Tc), SINR >= +3dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | 10 | 15 | 44 | 1024 | 128 | 1 | 2 | 66 | 58 |
| FR1 | 20 | 15 | 88 | 2048 | 64 | 1 | 2 | 33 | 30 |
| FR1 | 50 | 15 | 176 | 4096 | 32 | 1 | 2 | 15 | 15 |
| FR1 | 50 | 30 | 88 | 2048 | 32 | 1 | 2 | 15 | 15 |
| FR1 | 100 | 30 | 176 | 4096 | 16 | 1 | 2 | 8 | 7 |
| FR1 | 100 | 60 | 88 | 2048 | 16 | 1 | 2 | 9 | 8 |
| FR2 | 200 | 60 | 176 | 4096 | 8 | 1 | 2 | 5 | 4 |
| FR2 | 100 | 120 | 44 | 1024 | 16 | 1 | 2 | 10 | 8 |
| FR2 | 200 | 120 | 88 | 2048 | 8 | 1 | 2 | 5 | 4 |

Therefore, it is proposed to take into account these simulation results for defining the gNB Rx-Tx measurement accuracy requirements for agreed bandwidth ranges, where new simulation data can depict accuracy for the lower bound threshold of said bandwidth range. The accuracy values itself will be treated accordingly, by taking the average of companies’ results per SRS configuration and add margin accordingly dependend on the standard deviation across companies’ results.

Take into account new data points from simulation results for lowest RB number of bandwidth group to define gNB Rx-Tx measurement accuracy requirements

When analyzing previous meetings contribution regarding simulation results, it can be derived that for some SRS configurations the standard deviation of the simulation results across companies’ is mostly in the range of few Tc. However, to take those uncertainties into account and provide measurement accuracy requirement figures that incorporate such differences, a further step after taking the average of companies’ simulation results can be considered necessary. While adding the plain standard deviation value on top of the average as margin in some cases would result in a worse accuracy requirement than each individual simulation result accuracy before averaging, it is proposed to add half the standard deviation, rounded up to the next higher integer. Very huge standard deviations have been addressed in the course of the last meetings in such a way that there will be no accuracy requirements defined, e.g. for 24RB and Ês/Iot ≥ -13, therefore the proposal of adding half standard deviation rounded to the next integer seems a viable solution to address the issue.

Add half of standard deviation rounded to the next integer as additional margin

Following the discussion from above, one can apply the proposals to the values in the accuracy requirements table. For bandwidth ranges where new RB values have been simulated, it is marked as in brackets, since the collection of all companies’ results will be addressed in RAN#100-e. Furthermore, where the simulation was already carried out for the lowest RB number for said bandwidth range, the method of adding half the standard deviation rounded up to the next integer was applied. This results in the following tables which are also incorporated in [2]

Table 13.2.2.2-1: gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 1-C, 1-H and 1-O

| Accuracy | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| [66] | ≥ -13 | 15 | 44 ≤ BW ≤ 84 |
| [33] |  |  | 88 ≤ BW ≤ 168 |
| [15] |  |  | 176 ≤ BW |
| 122 | ≥ +3 |  | 24 ≤ BW ≤ 40 |
| [60] |  |  | 44 ≤ BW ≤ 84 |
| [31] |  |  | 88 ≤ BW ≤ 168 |
| [15] |  |  | 176 ≤ BW |
| 42 | ≥ -13 | 30 | 48 ≤ BW ≤ 84 |
| [15] |  |  | 88 ≤ BW ≤ 168 |
| [8] |  |  | 176 ≤ BW |
| 32 | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| [15] |  |  | 88 ≤ BW ≤ 168 |
| [8] |  |  | 176 ≤ BW |
| 21 | ≥ -13 | 60 | 48 ≤ BW ≤ 84 |
| [9] |  |  | 88 ≤ BW |
| 16 | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| [8] |  |  | 88 ≤ BW |

Table 13.2.2.2-2: gNB Rx-Tx time difference absolute accuracy in FR2 for gNB type 2-O

| Accuracy | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| 9 | ≥ -13 | 60 | 132 ≤ BW ≤ 168 |
| [6] |  |  | 176 ≤ BW |
| 9 | ≥ +3 |  | 132 ≤ BW ≤ 168 |
| [6] |  |  | 176 ≤ BW |
| 22 | ≥ -13 | 120 | 32 ≤ BW ≤ 40 |
| [10] |  |  | 44 ≤ BW ≤ 84 |
| [6] |  |  | 88 ≤ BW |
| 16 | ≥ +3 |  | 32 ≤ BW ≤ 40 |
| [8] |  |  | 44 ≤ BW ≤ 84 |
| [6] |  |  | 88 ≤ BW |

# Conclusion

- Take into account new data points from simulation results for lowest RB number of bandwidth group to define gNB Rx-Tx measurement accuracy requirements

- Add half of standard deviation rounded to the next integer as additional margin

Table 13.2.2.2-1: gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 1-C, 1-H and 1-O

| Accuracy | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| [66] | ≥ -13 | 15 | 44 ≤ BW ≤ 84 |
| [33] |  |  | 88 ≤ BW ≤ 168 |
| [15] |  |  | 176 ≤ BW |
| 122 | ≥ +3 |  | 24 ≤ BW ≤ 40 |
| [60] |  |  | 44 ≤ BW ≤ 84 |
| [31] |  |  | 88 ≤ BW ≤ 168 |
| [15] |  |  | 176 ≤ BW |
| 42 | ≥ -13 | 30 | 48 ≤ BW ≤ 84 |
| [15] |  |  | 88 ≤ BW ≤ 168 |
| [8] |  |  | 176 ≤ BW |
| 32 | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| [15] |  |  | 88 ≤ BW ≤ 168 |
| [8] |  |  | 176 ≤ BW |
| 21 | ≥ -13 | 60 | 48 ≤ BW ≤ 84 |
| [9] |  |  | 88 ≤ BW |
| 16 | ≥ +3 |  | 48 ≤ BW ≤ 84 |
| [8] |  |  | 88 ≤ BW |

Table 13.2.2.2-2: gNB Rx-Tx time difference absolute accuracy in FR2 for gNB type 2-O

| Accuracy | SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- | --- |
| Unit: Tc | Unit: dB | Unit: kHz | Unit: RB |
| 9 | ≥ -13 | 60 | 132 ≤ BW ≤ 168 |
| [6] |  |  | 176 ≤ BW |
| 9 | ≥ +3 |  | 132 ≤ BW ≤ 168 |
| [6] |  |  | 176 ≤ BW |
| 22 | ≥ -13 | 120 | 32 ≤ BW ≤ 40 |
| [10] |  |  | 44 ≤ BW ≤ 84 |
| [6] |  |  | 88 ≤ BW |
| 16 | ≥ +3 |  | 32 ≤ BW ≤ 40 |
| [8] |  |  | 44 ≤ BW ≤ 84 |
| [6] |  |  | 88 ≤ BW |

# References

- R4-2108314		“WF on gNB positioning measurement requirements”, Ericsson

- R4-2114050		“Corrections to gNB Rx-Tx measurement in 38.133”, Ericsson