3GPP TSG-RAN WG4 meeting #100-e	R4-2114084

Electronic Meeting, 16 – 27 August 2021

Source:	Ericsson

Title:	Discussions on scope and general requirements for RedCap

Agenda item:	9.20.3.1

Document for:	Discussions

# 1	Introduction

In this contribution we discuss and provide our view on the scope of the release 17 RedCap work item and also present list of simulation assumptions that are needed to define the detailed requirements.

# 2	Discussion

## 2.1	Scope

Following FFS and options were identified at last meeting [1]:

- Inter-frequency mobility - FFS whether to define inter-frequency RRM requirements for RedCap UE in Rel-17. - Inter-RAT mobility - Option 1: - Do not define inter-RAT RRM requirements for RedCap UE in Rel-17. - Option 2: - Do not define inter-RAT RRM requirements on 2G/3G for RedCap UE in Rel-17 - FFS: whether define inter-RAT RRM requirements on LTE for RedCap UE in Rel-17

Inter-frequency and inter-RAT support

Our view is to support both intra- and inter-frequency mobility for RedCap since we consider both to be critical features, however considering the time plan of release 17 and more specifically the number of meetings left, it is proposed to prioritize the work to define the intra-frequency requirements in release 17, and the inter-frequency requirements can be developed if time allows within this release.

For same reason, we propose to not define the inter-RAT requirements in release 17. For example, RAN4 needs to first agree on the simulation assumptions and thereafter carry out the simulation campaign to develop the exact requirements which may take few meetings. Therefore our view is to develop support for the different mobility features (inter-frequency, inter-RAT) step by step by, this approach may allow companies to focus and develop high quality requirements for the core features. It is noted that a similar approach was adopted when developing the eMTC requirements in LTE.

- Proposal #1: RAN4 to prioritize the work to develop to the intra-frequency requirements in release 17 RedCap WI. Inter-frequency requirements are developed if time allows in release 17.

- Proposal #2: RAN4 to not define inter-RAT RRM requirements for RedCap UE in Rel-17.

# 3	Simulation assumptions

According to the agreed work plan in [2], RAN4 is to identify the simulation work and discuss the simulation assumptions. In this section, we go through the different types of requirements that would need simulation campaign to derive the exact requirements and provide initial simulation assumptions for each of those.

## 3.1	Cell detection

Due to the reduced minimum number of Rx branches, the neighbour cell detection requirements are affected. The UE may need longer time to detect a neighbour cell (Tdetect,NR_Intra) using 1 or 2 receive branches compared to 2 and 4 as in legacy. The simulation assumptions from release 15 are used as reference [3], and adopted to reflect the reduced receive branches of RedCap UE, see, see Annex A.

- Proposal #3: RAN4 to study the cell detection performance for RedCap UE assuming the simulation assumptions in Annex A.

## 3.2	SSB based RRM measurement

Measurement requirements (measurement period and accuracy requirements) need to be studied assuming reduced number of receive branches compared to NR release 15 assumptions. The SS block measurement simulation assumptions from release 15 are used as reference [4], and adopted to reflect the reduced receive branches of RedCap UE, see, see Annex B.

- Proposal #4: RAN4 to study the SSB based RRM measurement performance for RedCap UE assuming the simulation assumptions in Annex B.

## 3.3	Radio link monitoring and beam failure detection

New RLM and beam failure detection requirements are needed due to reduced complexity operation. For example, the RLM evaluation period, PDCCH transmission parameters for out-of-sync and in-sync are going to be different. Thus new simulation assumptions, based on corresponding release 15 assumptions in [5], are proposed to study the RLM performance for RedCap, see Annex C. Using these assumption, the intention is to study the number of samples required for the UE to evaluate the RLM and BFD.

- Proposal #5: RAN4 to study the RLM and BFD performance for RedCap UE assuming the simulation assumptions in Annex C.

It is also noted that L1 RSRP reporting is also needed for beam management, more specifically the candidate beam management uses the L1 RSRP reporting. Since current requirements were derived assuming 2 receive antennas, new simulations are needed to study the perform of 1 receive antenna. For this purpose, additional L1 RSRP simulation assumptions are provided in Annex C.

- Proposal #6: RAN4 to study the L1 RSRP measurement performance for RedCap UE assuming the simulation assumptions in Annex C.

## 3.4	SI reading

The SI (PBCH) reading requirements are likely to be affected due to reduce number for receive branches. New simulation assumptions, based on corresponding release 15 assumptions in [6], are proposed to study the SI reading performance for RedCap, see Annex D.

- Proposal #7: RAN4 to study the SI reading performance for RedCap UE assuming the simulation assumptions in Annex D.

# 3	Conclusion

In this contribution we have discussed the RRM scope of release 17 RedCap work item, identified list of simulations that are needed to derive the core requirements and have presented the simulation assumptions. Based on the discussions, we have made following proposals:

- Proposal #1: RAN4 to prioritize the work to develop to the intra-frequency requirements in release 17 RedCap WI. Inter-frequency requirements are developed if time allows in release 17.

- Proposal #2: RAN4 to not define inter-RAT RRM requirements for RedCap UE in Rel-17.

- Proposal #3: RAN4 to study the cell detection performance for RedCap UE assuming the simulation assumptions in Annex A.

- Proposal #4: RAN4 to study the SSB based RRM measurement performance for RedCap UE assuming the simulation assumptions in Annex B.

- Proposal #5: RAN4 to study the RLM and BFD performance for RedCap UE assuming the simulation assumptions in Annex C.

- Proposal #6: RAN4 to study the L1 RSRP measurement performance for RedCap UE assuming the simulation assumptions in Annex C.

- Proposal #7: RAN4 to study the SI reading performance for RedCap UE assuming the simulation assumptions in Annex D.

# References

- R4-2108359, “WF on RedCap RRM requirements”, Ericsson

- R4-2111247, “WI RRM work plan for RedCap”, Ericsson

- R4-1706903, “Link level simulation of PSS/SSS detection in NR”, Huawei, HiSilicon

- R4-1706904, “Link-level simulation assumptions for SS block measurements”, Ericsson

- R4-1709896, “Simulation assumptions on radio link monitoring in NR”, “Huawei, HiSilicon”

- R4-1709908, “Updated PBCH Simulation assumption”, Huawei, HiSilicon

# Annex A: Simulation assumption for cell detection

The intention of PSS/SSS detection is to acquire cell ID and symbol level synchronization of neighbour cell. The performance of decoding of PBCH is not included.

Table 1: Cell-specific Parameters for PSS/SSS Detection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| Carrier frequency |  | 4GHz; 30GHz | The same frequency as for Cell 1 is used |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 4GHz; 120 kHz; 240 kHz for 30GHz | The same values as for Cell 1 are used |
| RB Utilization | % | 100 | 100 |
| Power boosting for PSS and SSS | dB | 0 | 0 |
| Power boosting for Data and Control | dB | 0 | 0 |
| Data Modulation | - | QPSK | QPSK |
| CP Length | - | Normal | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 | 1 |
| SS burst set periodicity | ms | 5 | 5 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | CP/2 |
| Relative Delay of 1st Path (asynchronous) | ms | 0 | 1.5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Eor/Ioc for K=1 | dB | 6.4 | -0.75; SINR: -6dB; |
| Number of Tx antennas | - | 1 | 1 |
| PSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| SSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| Propagation Condition | - | FR1: AWGN TDL-A 30ns, Doppler=10Hz TDL-B 100ns, Doppler=300Hz TDL-C 300ns, Doppler=100Hz FR2: AWGN TDL-A 30ns, Doppler=35Hz. |  |
| Ioc Model Note3 | - | AWGN noise |  |

Table 2: UE-specific Parameters for Cell Detection

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 and Cell 2 by the UE | Yes |
| False detect threshold | Required as in a real UE implementation |
| UE having apriority knowledge of system being synchronous or asynchronous | No |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence IDs. |
| CP length detection | Not simulated |
| Receive antennas | 1 Option 1: With receiver beamforming for FR2 Option 2: Without receiver beamforming |

Table 3: Example Cell ID Combinations that can be simulated

| Case # | Cell 1 (Interferer Cell) |  | Cell 2 (Desired Cell) |  | Scenario |
| --- | --- | --- | --- | --- | --- |
| 1 | psc1 | ssc1 | psc2 | ssc2 | Synchronous |
| 2 | psc1 | ssc1 | psc1 | ssc3 | Synchronous |
| 3 | psc1 | ssc1 | psc2 | ssc2 | Asynchronous |
| 4 | psc1 | ssc1 | psc1 | ssc3 | Asynchronous |

Table 4: Example PSS, SSS indices that can be simulated simulations

| PSS Indices |  | SSS Indices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Label | Code index | Label | Code index (m0, m1) | Cell ID | (,) |
| psc1 | 43 | ssc1 | (4, 5) | 337 | (112, 1) |
| psc2 | 86 | ssc2 | (8, 9) | 674 | (224, 2) |
|  |  | ssc3 | (7,.8) | 673 | (224, 1) |

Performance Metric

For NR-PSS/SSS acquisition time, the detection is claimed to be successful only when both the PSS and SSS are correctly detected and the timing offset is within half CP length. The CDF curves of joint PSS/SSS detection latency are suggested to be provided.

# Annex B: Simulation assumptions for SSB based RRM measurement

The following measurements are to be studied:

- RSRP based on SSS,

- RSRP based on SSS and PBCH DMRS,

- Signal quality measurements.

Two cells are simulated, interfering Cell 1 (unknown to the UE) and measured Cell 2 (known to the UE).

Simulation assumptions are provided for SSS based RSRP.

Simulation Assumptions:

Table 1: General parameters

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | 4 GHz | 30 GHz |
| System bandwidth | 10 MHz | 100 MHz |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes | No / Yes |
| DRX | No | No |
| BS transmit antennas for SS blocks | 1 tx or single layer transmissions | 1 tx or single layer transmissions |
| UE receive antennas | 1 rx | 1 rx |
| Data and control channel subcarrier spacing | The same as SS block subcarrier spacing | The same as SS block subcarrier spacing |
| Measurement period (in number of measurement samples) | 5; other number of samples may also be studied upon a need | 5; other numbers of samples may also be studied upon a need |
| SS burst set configuration in Cell 1 and Cell 2: |  |  |
| - Subcarrier spacing | 15 kHz; 30 kHz | 120 kHz; 240 kHz |
| - Number of SS blocks per SS burst set, K | 1 | 1 |
| - SS burst periodicity | 5 ms | 5 ms |
| - Number of transmit antenna ports | 1 (the same port for SSS, PSS, PBCH) | 1 (the same port for SSS, PSS, PBCH) |
| - SS block BW | 127 subcarriers for SS, 288 subcarriers for PBCH | 127 subcarriers for SS, 288 subcarriers for PBCH |
| - Actual SS block transmissions | always transmitted | always transmitted |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| PSS and SSS sequences | - | To be indicated by companies | To be indicated by companies |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS | dB | 0 | 0 |
| Data and control PSD relative to PSS and SSS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | 3 ms |
| SNR | dB | 6 dB | 1 dB |
| Es/IoT (calculated from SNR) | dB | N/A | K=1: -5.97 dB |
| Propagation conditions | - | FR1: AWGN TDL-A 30ns, Doppler=10Hz TDL-B 100ns, Doppler=300Hz TDL-C 300ns, Doppler=100Hz FR2: AWGN TDL-A 30ns, Doppler=35Hz. |  |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |

Performance metric:

For different estimation time periods, the CDF curves and 90th %-iles are to be provided for (without RF impairment margin):

- Delta measurement   = (estimated measurement – ideal measurement) 	[dB];

- Delta cell quality based on N SS block measurements which are L1 filtered but not L3 filtered = (estimated - ideal) [dB], where N=1 (for K=1) and N=K/2 (for K>1).

# Annex C: Simulation assumptions for RLM and BFD

The intention of RLM simulation is to investigate the number of samples required in L1 evaluation period for OOS and IS.

Simulation assumptions for RLM and BFD evaluations

Table 1: RLM and BFD simulation parameters for SS block based on RLM and BFD

| Parameters | Value |  |
| --- | --- | --- |
|  | Sub-6GHz | Above 6GHz |
| Carrier frequency | 4GHz | 30GHz |
| Subcarrier Spacing for SS block | 15KHz, 30KHz | 120KHz |
| Bandwidth of SS block | 24 RBs |  |
| Number of transmitted SS block within a SS burst set period (K) | 1 |  |
| RB utilization | 0% |  |
| Reference signals | - SSS |  |
| Transmission period of SS block | - 20ms |  |
| Number of samples in L1 evaluation period | - 5 samples, 10 samples, 20 samples; |  |
| Transmit antennas | 1 |  |
| Receive antennas | 1 |  |
| Propagation conditions | FR1: AWGN TDL-A 30ns, Doppler=10Hz TDL-B 100ns, Doppler=300Hz TDL-C 300ns, Doppler=100Hz FR2: AWGN TDL-A 30ns, Doppler=35Hz. |  |
| CP type | Normal CP |  |
| SNR | [-14, -13, ., 1, 0] dB |  |

Table 2: RLM and BFD simulation parameters for CSI-RS based on RLM and BFD

| Parameter | Unit | Value |
| --- | --- | --- |
| Bandwidth |  | 24 RBs |
| SCS | kHz | 15, 30, 120 |
| Number of CSI-RS ports (X) |  | 1 |
| CDM Type |  | ‘No CDM’ |
| Density (ρ) |  | 3 |
| CSI-RS periodicity | slots | 20 for 15 kHz 40 for 30 kHz 80 for 120 kHz |
| EPRE ratio to SSS | dB | 0Note 2 |

Performance Metric

The UE performs signal and interference measurements based on RLM-RS (i.e. SS block or CSI-RS) to derive a downlink link quality metric (e.g. SINR).

The CDF curves are to be provided for measurement errors for downlink link quality metric:

•	Delta SINR = (estimated SINR – ideal SINR) 	[dB]

Interest companies are encouraged to provide the measurement accuracy results of SINR.

Simulation assumptions for L1-RSRP measurements

The intention of the simulation is to investigate the performance of CSI-RS based measurements.

## SSB based L1-RSRP measurements

Table 3: Parameters for SSB based L1-RSRP Measurements

| Parameters | Value |  |
| --- | --- | --- |
|  | Sub-6GHz | Above 6GHz |
| Carrier frequency | 4GHz | 30GHz |
| Subcarrier Spacing for SSB | 15KHz, 30KHz | 120KHz, 240KHz |
| SSB bandwidth | 20RBs |  |
| RB utilization | 100% |  |
| Data Modulation | QPSK |  |
| Data and control PSD relative to NR-PSS and NR-SSS | 0 dB |  |
| Transmitted SSB per SSB Periodicity | SSB #0 |  |
| SSB Periodicity | - Baseline: 40ms; - Optional: 5ms, 10ms, 20ms. |  |
| Number of samples in L1 averaging | - 1 sample, 3 samples, 5 samples - Others are not precluded. |  |
| Transmit antennas | 1 |  |
| Receive antennas | 1 |  |
| Propagation conditions | AWGN, EPA5, ETU30 (for 15 kHz SCS), ETU70 (for 15 kHz SCS); | For 30 GHz: CDL-C, optional TDL-A/B/C (delay spread: 30 ns and 300 ns; UE speed: 3 km/h and 30 km/h) |
| CP type | Normal CP |  |
| SSB Es/Noc | [-3~0] dB |  |

## CSI-RS based L1-RSRP measurements

Table 4: Parameters for CSI-RS based L1-RSRP Measurements

| Parameters | Value |  |
| --- | --- | --- |
|  | Sub-6GHz | Above 6GHz |
| Carrier frequency | 4GHz | 30GHz |
| Subcarrier Spacing for CSI-RS | 15KHz, 30KHz, 60KHz | 60KHz, 120KHz |
| CSI-RS bandwidth | - Baseline: 24RBs, 48RBs - Optional: 96RBs, 192 RBs. |  |
| RB utilization | 100% |  |
| Data Modulation | QPSK |  |
| Data and control PSD relative to CSI-RS | 0 dB |  |
| Number of transmitted CSI-RS resource | K=1. |  |
| CSI-RS RE-Mapping-Pattern | Row#2 defined in Table 7.4.1.5.2-1 of TS38.211 |  |
| CSI-RS Density | D=3 |  |
| CSI-RS Periodicity | - Baseline: 20ms; - Optional: 2ms, 5ms, 10ms. |  |
| Number of samples in L1 averaging | - 1 sample, 3 samples, 5 samples, 10 samples - Others are not precluded. |  |
| Transmit antennas | 1 |  |
| Receive antennas | 1 |  |
| Propagation conditions | AWGN, EPA5, ETU30 (for 15 kHz SCS only), ETU70 (for 15 kHz SCS only); | Baseline: CDL-C, Optional: TDL-A/B/C (delay spread: 30 ns and 300 ns; UE speed: 3 km/h and 30 km/h) |
| CP type | Normal CP |  |
| Es/Noc | [-3~0] dB |  |

## Performance Metric

- Performance metrics for L1-RSRP

The following performance metrics can be considered

- The CDF curves (including the 95% and 5% CDF points) for delta L1-RSRP.

Delta L1-RSRP and delta CSI-RSRP is defined as follows:

Delta L1-RSRP = Measured L1-RSRP – Ideal L1-RSRP.

Ideal L1-RSRP is defined as the true value assuming perfect channel estimation using the same sampling rate and sampling occasions as used in the measured L1-RSRP evaluation.

Note1: The definition of SSB based L1-RSRP is the same as SS-RSRP defined in TS38.215.

Note 2: The definition of CSI-RS based L1-RSRP is the same as CSI-RSRP defined in TS38.215.

# Annex D: Simulation assumptions for SI reading

- Simulation Assumptions

The proposed simulation assumptions are used to derive the basic SI reading (PBCH acquisition) time.

Case1: PBCH reading

Basic SI reading time = the number of SS-blocks required for 99% to successfully decode the PBCH.

Case2: PBCH-DMRS time index reading (Without decode PBCH data)

DMRS time index reading time = the number of SS-blocks required for 99% to successfully reading DMRS time index.

Table 1: Parameters for basic SI reading (PBCH acquisition)

| Parameter | Unit | Value |
| --- | --- | --- |
| Carrier frequency | GHz | 4GHz; 30GHz |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 4GHz; 120 kHz; 30GHz |
| Number of Tx antennas | - | 1 Assuming only 1Tx port is used |
| Number of Rx antennas | - | 1 Option 1: With receiver beamforming for FR2 Option 2: Without receiver beamforming |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 |
| SS burst set periodicity | ms | 20 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| RB Utilization | % | 0 |
| PBCH symbols within the SS block |  | PSS-PBCH-SSS-PBCH |
| Data and Control Power offset with respect to PSS and SSS | dB | 0 |
| PBCH power offset with respect to PBCH-DMRS | dB | 0 |
| PBCH-DMRS power offset with respect to PSS and SSS | dB | To be indicated |
| PSS and SSS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS RE positions within the PBCH resource | - | According to the RAN1 agreements |
| PBCH Modulation | - | QPSK |
| PBCH Payload (including the CRC) | bits | 56bit |
| PBCH SNR | dB | -12 : 0 dB, at least including -6dB,-8 dB |
| Propagation Condition | - | FR1: AWGN TDL-A 30ns, Doppler=10Hz TDL-B 100ns, Doppler=300Hz TDL-C 300ns, Doppler=100Hz FR2: AWGN TDL-A 30ns, Doppler=35Hz. |
| UE speed and Doppler Shift | - | 3km/h speed with 83Hz Doppler at 30GHz |

- Performance Metric

The proposed simulation assumptions are used to derive the Basic SI reading time.

Case1: PBCH reading

Basic SI reading time = the number of SS-blocks required for 99% to successfully decode the PBCH.

Case2: PBCH-DMRS time index reading (Without decode PBCH data)

DMRS time index reading time = the number of SS-blocks required for 99% to successfully reading DMRS time index.