# 3GPP TSG-RAN WG4 Meeting # 100-e						R4-2114325

# Electronic Meeting, 16th – 27th August, 2021

Source:		Ericsson

Title:			IAB with luant modem testing

Agenda item:		6.1.2.2.1

Document for:		Approval

# Introduction

In current 38.174-2 specification, the luant modem power on is required to be tested with IAB.

### 4.5.4	IAB with integrated Iuant BS modem

Unless otherwise stated, for the tests in the present document, the integrated Iuant BS modem shall be switched OFF. Spurious emissions according to clauses 6.6.5 and 7.6 shall be measured only for frequencies above 30 MHz with the integrated Iuant BS modem switched ON.

And in current 38.174-1,

### 4.5.2	IAB with integrated Iuant BS modem

Unless otherwise stated, for the tests in the present document, the integrated Iuant BS modem shall be switched OFF. Spurious emissions according to clauses 6.6.5 and 7.6 shall be measured only for frequencies above 20 MHz with the integrated Iuant BS modem switched ON.

In this paper, we present our view related to luant modem testing.

# Discussion

In IAB Rel-16 work item, the IAB type is defined with type 1-H, 1-O and 2-O, as the luant interference is to control the RET for antenna tilting, such interference could be relevant to the IAB type 1-H than 1-O and 2-O. This is because that type 1-O and 2-O has integrate antenna panel and the need of luant modem to control additional RET is not clear especially the luant modem is defined with feeder cable reference point as below:

Figure 4.3.1[2]: Modem configuration and modem reference points for a BS without BS modem

Observation #1: The luant model specification is specified with reference point of BS antenna connector which is BS type 1-C

Current luant modem is specified for BS type 1-C and as the IAB does not include the type of 1-C, it is questionable whether the luant modem itself should be specified in IAB testing conformance specification as test cannot be done with an antenna connector where IAB does not have.

Proposal-1: Luant modem testing aspect should be removed from the IAB test specification.

# Conclusions

In this contribution, we present our view on the lunant modem for IAB with below observation and proposal:

Observation #1: The luant model specification is specified with reference point of BS antenna connector which is BS type 1-C

Proposal-1: Luant modem testing aspect should be removed from the IAB test specification.

# References

- 3GPP TS 38.176-2, " NR; Integrated Access and Backhaul (IAB) conformance testing; Part 2: Radiated conformance testing ".

[2] 3GPP TS 37.461, UTRAN Iuant interface: Layer 1