3GPP TSG-RAN WG4 Meeting # 100-e	R4-2114413

# Electronic Meeting, August 16-27, 2021

Source: 	Huawei

Title: 	Release independence aspects for the FR2 extension up to 71 GHz

Agenda Item:	9.16.8

Document for:	Discussion

# Introduction

At the last RAN#92-e meeting, NR_ext_to_71GHz WID was updated capturing the following agreements related to the FR2 extension up to 71 GHz:

Note 5: FR2 is extended to cover 24.25GHz to 71GHz with FR2-1 for 24.25-52.6GHz and FR2-2 for 52.6-71GHz. o The related UE capabilities and their applicability to the frequency range 52.6 to 71 GHz will have to be analysed on a case by case basis o The application of any of the UE feature introduced for 52.6-71 GHz to existing FR1/FR2 should be discussed case by case. o TSG RAN specifications shall make it very clear (to readers) that frequency bands in the 52.6-71GHz range are only Release-independent from Rel-17 onwards, to ensure that there is clear industry understanding about which FR2 features are applicable for operation in 52.6-71GHz range. NOTE 5a: Whenever the FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered in this release, unless otherwise stated. NOTE 5b: The designations FR2-1 and FR2-2 should only be used when needed.

Referring to the bolded part of the note above, this there is a need to provide clarifications on the applicabiltiy of the NR operating bands or features specific to the NR operation in FR2-2 range sub-range, as only applowed to be release-independent from Rel-17 onwards.

# Discussion

Based on the above extract of the RAN decisions from the revised WID in [1], RAN4 shall consider possible ways to ensure that NR specifications are clear regarding the frequency bands in the 52.6-71GHz range being only Release-independent from Rel-17 onwards. As there is related RAN4 specification in TS 38.307, it is proposed to address the above referred TSG RAN agreement, by adding clarification sentence to the scope of the TS 38.307. The proposed sentence may be worded as below:

NR operating bands or features specific to the NR operation in FR2-2 sub-range (i.e. 52.6 – 71 GHz) may be release independent from Rel-17 onwards. For FR2-2 definition, refer to TS 38.101-2.

With the above we propose to:

Proposal 1: agree on the approach to address the release-independence aspects of NR FR2 extension up to 71 GHz, by adding a clarification sentence to the scope in TS 38.307.

According to the workplan in R4-2110950, Draft CRs shall not be provided until the November 2021 meeting. Therefore, the related draft CR is added for reference in Annex A.

| RAN4#101 Nov’21 | Discussions on - (continued) BS and UE transmitter requirements - (continued) BS and UE receiver requirements - (continued) Requirements for specific band combinations for the scenario(s) Agreements on - BS and UE transmitter requirements - Requirements related to band combinations for the scenario(s) - Provide draft CRs for all related specs |
| --- | --- |
| RAN4#102 Feb’22 | Discussions on - Remaining BS and UE transmitter requirements - Remaining BS and UE receiver requirements - Remaining Requirements for specific band combinations for the scenario(s) Agreements on - BS and UE receiver requirements - Any remaining requirements - All related CRs |

# Conclusions

Based on the discussion, the following is proposed to address the release-independence aspects of the NR operation extension up to 71 GHz:

Proposal 1: agree on the approach to address the release-independence aspects of NR FR2 extension up to 71 GHz, by adding a clarification sentence to the scope in TS 38.307: NR operating bands or features specific to the NR operation in FR2-2 sub-range (i.e. 52.6 – 71 GHz) may be release independent from Rel-17 onwards. For FR2-2 definition, refer to TS 38.101-2.

# References

[1] 		RP-211584	Extending current NR operation to 71GHz, Revised WID, RAN#92-e

# Annex A: draft CR to TS 38.307

3GPP TSG-RAN WG4 Meeting # 100-e	R4-21xxxxx

# Electronic Meeting, August 16-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | - | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.307: release independence aspects for the FR2 extension up to 71 GHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2021-07-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At the last RAN#92-e meeting, NR_ext_to_71GHz WID was updated capturing the following agreements related to the FR2 extension up to 71 GHz: Note 5: FR2 is extended to cover 24.25GHz to 71GHz with FR2-1 for 24.25-52.6GHz and FR2-2 for 52.6-71GHz. o The related UE capabilities and their applicability to the frequency range 52.6 to 71 GHz will have to be analysed on a case by case basis o The application of any of the UE feature introduced for 52.6-71 GHz to existing FR1/FR2 should be discussed case by case. o TSG RAN specifications shall make it very clear (to readers) that frequency bands in the 52.6-71GHz range are only Release-independent from Rel-17 onwards, to ensure that there is clear industry understanding about which FR2 features are applicable for operation in 52.6-71GHz range. NOTE 5a: Whenever the FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be considered in this release, unless otherwise stated. NOTE 5b: The designations FR2-1 and FR2-2 should only be used when needed.. Referring to the bolded part of the note above, this Draft CR provides clarification on the applicabiltiy of the NR operating bands or features specific to the NR operation in FR2-2 range sub-range, as only applowed to be release-independent from Rel-17 onwards. |
| Summary of change: |  | Update of the Scope of the TS 38.307 to reflect the TSG RAN agreement on the release-independace aspects of the NR operation in FR2-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification would not reflect the TSG RAN agreement on the release-independence aspects of the NR operating bands or features specific to the FR2-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

# 1	Scope

The present document specifies requirements for Rel-16 UEs supporting release independent features like:

-	additional NR operating bands and power classes on top of Rel-16 of TS 38.101 [2-5] and TS 38.133 [6];

NR operating bands or features specific to the NR operation in FR2-2 sub-range (i.e. 52.6 – 71 GHz) may be release-independent from Rel-17 onwards. For FR2-2 definition, refer to TS 38.101-2 [3].

----------------------------- End of modified section ------------------------------