3GPP TSG-RAN WG4 Meeting#100-e                	R4-2114423

Electronic Meeting, 19 – 27 August 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on inter-frequency measurements for FR1 HST CA

Agenda item:	9.8.2.1.2

Document for:	Discussion

Introduction

During the RAN4#100-e meeting, a way forward on NR RRM enhancements for FR1 HST [1] was approved. Referring to [1], enhancements on inter-frequency PSS/SSS detection time and measurements in connected mode are open issues. Further, inter-frequency measurements in idle mode are also open issues.

In this contribution, we discuss the above open issues and present our views.

Discussion

From [1], a number of options on how to enhance inter-frequency PSS/SSS detection time requirements with measurement gaps:

- Inter-frequency measurement with MG, connected state

  - PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST

    - Option 1 (QC, Apple, CMCC):

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 6 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(6*M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 6 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5; When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 |  |

    - Option 2 (CATT, vivo, Xiaomi, MTK, HW, Ericsson, Apple)：

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*M2) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5. When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |  |

    - Option 3 (Nokia)：

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, N Note 4 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(N Note 4* M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | N Note 4 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 4: When highSpeedMeasFlag-r16 is not configured, N = 8; otherwise N = 5. |  |

In HST deployment scenarios, SCells are typically co-located with PCells and the target UE speed is the same as for the single carrier case. Thus, the PSS/SSS detection time for inter-frequency should be on the same order as the corresponding cell detection time for intra-frequency. As such, the same methodology for Rel-16 FR1 HST is used to derive the proposed enhancement in Option 3.

Proposal 1: For enhancements of inter-frequency PSS/SSS detection time requirements with measurement gaps, Option 3 is preferred but can compromise on Option 1.

From [1], enhancements on inter-frequency measurement delay requirement with measurement gaps are

- Inter-frequency measurement with MG, connected state

  - Measurement delay requirement for inter-frequency measurement with MG in HST in connected state for HST

    - Option 1 (QC):

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, 6 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle≤ 160ms | max(200ms, ceil(6 x M2 Note 3) x max(SMTC period,DRX cycle)) x CSSFintra |
| 160ms < DRX cycle≤ 320ms | ceil(5 x M2 Note 3 x Kp) x max(SMTC period,DRX cycle) |
| DRX cycle>320ms | ceil( Y Note 4 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 4: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms |  |

    - Option 2 (HW, OPPO):

      - -7 samples for 160ms < DRX cycle≤ 320ms

      - -6 samples for DRX cycle>320ms and SMTC <= 40ms

    - Option 3 (CATT):

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 160ms | Max(200ms, Ceil(8 M2) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| 160ms < DRX cycle≤ 320ms | Max(200ms, Ceil(7 M2) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5. When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |  |

    - Option 4 (CMCC):

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle≤ 160ms | Max(200ms, Ceil(8 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| 160ms <DRX cycle ≤ 320ms | Max(200ms, Ceil([4] M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Y Note 4 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 4: Y1= [3] when SMTC <= 40ms, Y1= [8] when SMTC > 40ms |  |

    - Option 5 (vivo, Xiaomi, OPPO): For inter-frequency measurement with MG, the enhanced requirements specified for LTE-NR inter-RAT measurements in R16 HST could be used as baseline

    - Option 6 (Apple, Ericsson):

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, [6] Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil([6] M2NOTE 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | [6] DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When [highSpeedMeasFlag-r17] is not configured, M2 = 1.5; When [highSpeedMeasFlag-r17] is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 4 M2 Note 3 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

    - Option 7 (MTK, Ericsson, Xiaomi):

    - Option 8 (Nokia):

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, 5 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle≤ 160ms | Max(200ms, Ceil(5 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| 160 ms ≤DRX cycle ≤ 320ms | 4 M2 Note 3 DRX cycle CSSFinter |
| DRX cycle > 320ms | Y Note 4 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 NOTE 4: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms |  |

Using the same technical arguments as for the inter-frequency PSS/SSS detection time requirement, Option 8 is derived based on the Rel-16 FR1 HST methodology for intra-frequency measurement delay requirements.

Proposal 2: For enhancements of inter-frequency measurement delay requirements with measurement gaps, Option 8 is preferred but can compromise on Option 1.

From [1], enhancements on inter-frequency measurement requirements in idle state are open issues

- Inter-frequency measurement in idle state

  - Whether define the enhancement for inter-frequency measurement in idle mode for HST

    - Option 1 (HW, OPPO, QC, Ericsson, Xiaomi, CATT, CMCC, vivo): Yes

    - Option 2 (MTK, Nokia): No

    - Option 3 (Apple): introduce a dedicated UE capability indicating the support of inter-frequency measurement in idle mode for HST

In the idle mode, inter-frequency measurement would have lower priority than intra-frequency measurements. Further, it increases battery energy consumption of UE. Intra-frequency measurements seem to be sufficient to ensure operation.

Proposal 3: For enhancements of inter-frequency measurement requirements in idle state, Option 2 is preferred.

Conclusion

The document has discussed and presented our views on enhancements inter-frequency measurements with measurement gaps in connected mode and inter-frequency measurements in idle mode for FR1 HST CA.

Proposal 1: For enhancements of inter-frequency PSS/SSS detection time requirements with measurement gaps, Option 3 is preferred but can compromise on Option 1.

Proposal 2: For enhancements of inter-frequency measurement delay requirements with measurement gaps, Option 8 is preferred but can compromise on Option 1.

Proposal 3: For enhancements of inter-frequency measurement requirements in idle state, Option 2 is preferred.

References

[1]    R4-2108341, WF on RRM for FR1 HST, CMCC