3GPP TSG-RAN WG4 Meeting #100-e  		                                 R4-2114424

Electronic Meeting, 16th-27th of August, 2021

# Source:	Thales

# Title:	On the S-band NTN Scenarios and Parameters for Calibration and Coexistence Simulations

# Type:	discussion

# Document for:	discussion

# Agenda Item:	9.13.2.1

Release:	Rel-17

# Introduction

RAN4 #99-e Agreements:

During RAN4 #99-e, the following agreement has been captured in NTN_Solutions_Part2 (R4-2108093):

RAN4 #99-e Agreements:

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 2.3-1 NRB configuration per BandWidth size and SCS

| Configuration FR1 S-band | NRB (5MHz BW) | NRB (10MHz BW) | NRB (15MHz BW) | NRB (20MHz BW) |
| --- | --- | --- | --- | --- |
| SCS 15 kHz | 25 | 52 | 79 | TBA |
| SCS 30 kHz | 11 | 24 | 38 | TBA |

Table 2.3-2 Set-1 satellite parameters for co-existence study

| Satellite orbit | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |

| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| --- | --- | --- | --- | --- |

| Satellite max TX power in dBm | BW (MHz) |  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS 15kHz |  | 44.53 | 47.71 | 49.53 | TBA | 46.53 | 49.71 | 51.53 | TBA | 40.53 | 43.71 | 45.53 | TBA |
|  | SCS 30kHz |  | 43.98 | 47.37 | 49.36 | TBA | 45.98 | 49.37 | 51.36 | TBA | 39.98 | 43.37 | 45.36 | TBA |

| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| --- | --- | --- | --- | --- |

| Channel bandwidth |  | 5/10/15/20MHz | 5/10/15/20MHz | 5/10/15/20MHz |
| --- | --- | --- | --- | --- |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km | 50 m |

Payload characteristics for UL transmissions

| G/T | 2 GHz | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| --- | --- | --- | --- | --- |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |

RAN4 #99-e Agreements:

- Proposal 3-1-8-1: RAN4 may consider S-band extension in a new WI on operator’s request, as it’s common practice for any TN band in RAN4.

- Proposal 3-3-1-1: RAN4 may consider mixed pairing of (L-band) and (S-band) in a separate dedicated WI, as it’s common practice for any TN band in RAN4. The core functionality should be forward compatible with mixed pairing.

- Proposal 3-3-2-1: RAN4 may consider pairing of 1610 - 1618.725 MHz UL (L-band) and 2483.5 – 2500 MHz DL (S-band) in a separate dedicated WI, as it’s common practice for any TN band in RAN4. The core functionality should be forward compatible with mixed pairing.

Moreover, the following agreements have been also made for NTN_Solutions_Part1 (R4-2108099):

The goal of this contribution is to further clarify/define some simulation parameters to be taken into account for coexistence scenarios considered by RAN4 studies.

# S-band NTN Coexistence Scenarios

RAN4 #99-e Agreements:

- Proposal 3-1-1-1: RAN4 shall use S-Band Reference Operational Deployment Scenario using 1980-2010 MHz for UL and 2170-2200 MHz for DL.

- Proposal 2-1-1-1: The first NTN band will have the following frequency range definition: 1980-2010 MHz in UL and 2170-2200 MHz in DL. Its band number is FFS.

Note: Companies are encouraged to provide a NTN band numbering scheme for next RAN4 meeting.

With respect to the operational deployment scenarios in S-band, during RAN4#99-e it has been agreed:

The current MSS S-band for NR NTN usage refers to 1980-2010 MHz (UpLink) and 2170-2200 MHz (DownLink), and therefore located far away from terrestrial n41 (2496-2690 MHz).

RAN4 #99-e Agreements:

- Proposal 3-1-8-1: RAN4 may consider S-band extension in a new WI on operator’s request, as it’s common practice for any TN band in RAN4.

Moreover, the following agreements have been taken with respect to the S-band extension (e.g. S-band 2483.5 – 2500 MHz DL):

Using the previous RAN4#99-e agreements for NTN_Solutions_Part1 (see R4-2108099) with respect to S-band extension (DL 2483.5-2500 MHz), together with current MSS S-band definition for NR NTN (DL 2170-2200 MHz), we therefore further propose to reduce the interference scenarios considered by current coexistence work.

Therefore, since S-Band DL 2483.5-2500 MHz extension is not considered by current NTN NR WI, and current S-Band is defined in DL for 2170-2200 MHz only, we propose:

Proposal 1. RAN4 should remove coexistence with n41 (2496-2690 MHz) and update the coexistence scenarios as follows:

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 7 | TN with NTN | TN UL | NTN DL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 2 |
| 8 | TN with NTN | NTN DL | TN UL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 1 |
| 9 | NTN with NTN | NTN DL | NTN DL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |

Moreover, with respect to NTN-NTN coexistence, it seems that RAN4 is not using a very clear definition of NTN to NTN “adjacent band” coexistence. For further exemplification with respect to the adjacent band and/or the adjacent channel NTN-NTN coexistence:

- If NTN-NTN coexistence is considered with respect to satellite bands adjacent to NTN S-band, there are not any adjacent bands allocated to other satellite services that can be considered for this coexistence. Therefore, it does not make sense to consider this coexistence analysis.

- If NTN-NTN coexistence is considered in the same operating NTN S-band (which should be no longer considered as “adjacent band” coexistence), the coexistence between different satellite networks operating in the S-band (in different channels) is governed by bi-lateral agreements between the satellite operators on the basis of regulatory guidelines and ensured through operational configuration. Therefore, again, it does not make sense to consider this coexistence analysis.

The satellite community would not be able to agree to reference coexistence scenarios since each constellation is specific and designed according to business and regulatory requirements. Therefore, the satellite community does not see the need to analyze NTN-NTN coexistence scenarios in RAN4.

Proposal 2. RAN4 should remove NTN-NTN coexistence for LEO-LEO, LEO-GEO and GEO-GEO scenarios in S-band and update the coexistence scenarios as follows:

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 7 | TN with NTN | TN UL | NTN DL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 2 |
| 8 | TN with NTN | NTN DL | TN UL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 1 |
| 9 | NTN with NTN | NTN DL | NTN DL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |

# Updated S-band Simulation Parameters

We further propose to update the following tables as follows:

Proposal 3. The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 2.3-1 NRB configuration per BandWidth size and SCS

| Configuration FR1 S-band | NRB (5MHz BW) | NRB (10MHz BW) | NRB (15MHz BW) | NRB (20MHz BW) |
| --- | --- | --- | --- | --- |
| SCS 15 kHz | 25 | 52 | 79 | 106 |
| SCS 30 kHz | 11 | 24 | 38 | 51 |

Table 2.3-2 Set-1 satellite parameters for co-existence study

| Satellite orbit | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |

| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| --- | --- | --- | --- | --- |

| Satellite max TX power in dBm | BW (MHz) |  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS 15kHz |  | 44.53 | 47.71 | 49.53 | 50.81 | 46.53 | 49.71 | 51.53 | 52.81 | 40.53 | 43.71 | 45.53 | 46.81 |
|  | SCS 30kHz |  | 43.98 | 47.37 | 49.36 | 50.64 | 45.98 | 49.37 | 51.36 | 52.64 | 39.98 | 43.37 | 45.36 | 46.64 |

| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| --- | --- | --- | --- | --- |

| Channel bandwidth |  | 5/10/15/20MHz | 5/10/15/20MHz | 5/10/15/20MHz |
| --- | --- | --- | --- | --- |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  | 0.3474 deg | 3.8206 deg | 3.8206 deg |
| Satellite (central) beam diameter |  | 250 km | 90 km | 50 m |

Payload characteristics for UL transmissions

| G/T | 2 GHz | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| --- | --- | --- | --- | --- |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |

Proposal 4. The satellite Adjacent Beam Spacing or ABS can be computed using the following equations:

ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2)

with ABS [degree]=180/pi x ABS[rad] and

with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern.

Proposal 5. The following ABS values shall be used for RAN4 NTN simulator calibration and coexistence purposes:

- Adjacent Beam Spacing value for GEO of 0.3474 degrees;

- Adjacent Beam Spacing value for LEO of 3.8206 degrees.

Proposal 6. RAN4 shall be aware that current satellite ABS (Adjacent Beam Spacing) used to generate satellite adjacent beams for RAN4 coexistence simulation purposes are lower than HPBW (Half-Power BandWidth) of the main lobe of the antenna pattern used to generate the satellite central beam. Therefore, ABS≠HPBW with ABS<HPBW.

# Conclusions

Proposal 1. RAN4 should remove coexistence with n41 (2496-2690 MHz) and update the coexistence scenarios as follows:

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 7 | TN with NTN | TN UL | NTN DL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 2 |
| 8 | TN with NTN | NTN DL | TN UL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 1 |
| 9 | NTN with NTN | NTN DL | NTN DL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |

Proposal 2. RAN4 should remove NTN-NTN coexistence for LEO-LEO, LEO-GEO and GEO-GEO scenarios in S-band and update the coexistence scenarios as follows:

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in S-band, e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with n34 TDD. | Phase 1 |
| 7 | TN with NTN | TN UL | NTN DL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 2 |
| 8 | TN with NTN | NTN DL | TN UL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 1 |
| 9 | NTN with NTN | NTN DL | NTN DL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |

Proposal 3. The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 2.3-1 NRB configuration per BandWidth size and SCS

| Configuration FR1 S-band | NRB (5MHz BW) | NRB (10MHz BW) | NRB (15MHz BW) | NRB (20MHz BW) |
| --- | --- | --- | --- | --- |
| SCS 15 kHz | 25 | 52 | 79 | 106 |
| SCS 30 kHz | 11 | 24 | 38 | 51 |

Table 2.3-2 Set-1 satellite parameters for co-existence study

| Satellite orbit | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |

| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| --- | --- | --- | --- | --- |

| Satellite max TX power in dBm | BW (MHz) |  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS 15kHz |  | 44.53 | 47.71 | 49.53 | 50.81 | 46.53 | 49.71 | 51.53 | 52.81 | 40.53 | 43.71 | 45.53 | 46.81 |
|  | SCS 30kHz |  | 43.98 | 47.37 | 49.36 | 50.64 | 45.98 | 49.37 | 51.36 | 52.64 | 39.98 | 43.37 | 45.36 | 46.64 |

| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| --- | --- | --- | --- | --- |

| Channel bandwidth |  | 5/10/15/20MHz | 5/10/15/20MHz | 5/10/15/20MHz |
| --- | --- | --- | --- | --- |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  | 0.3474 deg | 3.8206 deg | 3.8206 deg |
| Satellite (central) beam diameter |  | 250 km | 90 km | 50 m |

Payload characteristics for UL transmissions

| G/T | 2 GHz | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| --- | --- | --- | --- | --- |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |

Proposal 4. The satellite Adjacent Beam Spacing or ABS can be computed using the following equations:

ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2)

with ABS [degree]=180/pi x ABS[rad] and

with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern.

Proposal 5. The following ABS values shall be used for RAN4 NTN simulator calibration and coexistence purposes:

- Adjacent Beam Spacing value for GEO of 0.3474 degrees;

- Adjacent Beam Spacing value for LEO of 3.8206 degrees.

Proposal 6. RAN4 shall be aware that current satellite ABS (Adjacent Beam Spacing) used to generate satellite adjacent beams for RAN4 coexistence simulation purposes are lower than HPBW (Half-Power BandWidth) of the main lobe of the antenna pattern used to generate the satellite central beam. Therefore, ABS≠HPBW with ABS<HPBW.

END