3GPP TSG RAN WG4 Meeting #100-e	R4-2114465

Electronic Meeting, August 16-27, 2021

Agenda Item:	8.40.2

Source: 	Ericsson

Title:	Analysis of RRM requirements for FR2 FWA for band n259

Document for:	Discussion

- Introduction

The RF and RRM requirements for FR2 FWA UE with maximum TRP of 23dBm for band n259 are being defined for the ongoing WID [1]. The RF requirements for FR2 for band n259 are being defined for UE power class 5.

In this paper we discuss the RRM performance requirements for n259 for UE PC5.

# Impact on RRM requirement

The REFSENS for n259 for UE PC5 are being discussed in the RF group. Based on the proposal in [2] the REFSENS for band n259 for 50 MHz BW and existing REFSENS for n257 and n258 for UE PC5 in TS 38.101-2 are shown in table 1 for comparison: and EIS spherical coverage for band n262 for UE power class 3 are reproduced below in table 1 and table 2 respectively:

Table 1: Reference sensitivity for UE power class 5

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -92.6 | -89.6 | -86.6 | -83.6 |
| n258 | -92.8 | -89.8 | -86.8 | -83.8 |
| n259 | -90.5 [Note] | - | - | - |
| Note: The REFSENS is based on [2] |  |  |  |  |

The following was agreed by RF group regarding the EIS spherical coverage for band n259 for UE power class 5 [5]:

- Spherical coverage requirement shall be derived based on following gain drop:

    - 8 dB degradation @ 85th%ile point

Based on the above gain drop and the REFSENS value [3], the EIS spherical coverage for UE power class 5 for band n259 is shown in table 2.

Table 2: EIS spherical coverage for UE power class 5

| Operating band | EIS at 85th %-tile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -84.6 | -81.6 | -78.6 | -75.6 |
| n258 | -84.8 | -81.8 | -78.8 | -75.8 |
| n259 | -82.5 | - | - | - |

Following is observed:

- The REFSENS of n259 for PC5 is 2.3 dB higher than that of n258 for PC5.

- The EIS spherical coverage of n259 for PC5 is 2.3 dB higher than that of n258 for PC5.

Based on the above analysis the n259 for PC5 can be assigned to band group Q with the band notation goup: NR_TDD_FR2_Q as shown in table 3.

Table 3: NR frequency band groups for FR2

| Group | Band group notation | Operating bands |
| --- | --- | --- |
| A | NR_TDD_FR2_A | n2571, n2581, n2611 |
| B | NR_TDD_FR2_B | n2574, n2584, n2614 |
| C | NR_TDD_FR2_C |  |
| D | NR_TDD_FR2_D |  |
| E | NR_TDD_FR2_E |  |
| F | NR_TDD_FR2_F | n2604 |
| G | NR_TDD_FR2_G | n2601 |
| H | NR_TDD_FR2_H |  |
| I | NR_TDD_FR2_I |  |
| J | NR_TDD_FR2_J |  |
| K | NR_TDD_FR2_K | n2575, n2585 n2621 |
| L | NR_TDD_FR2_L | n2572, n2582, n2612 |
| M | NR_TDD_FR2_M |  |
| N | NR_TDD_FR2_N | n2624 |
| O | NR_TDD_FR2_O |  |
| P | NR_TDD_FR2_P |  |
| Q | NR_TDD_FR2_Q | n2595 |
| R | NR_TDD_FR2_R |  |
| S | NR_TDD_FR2_S |  |
| T | NR_TDD_FR2_T | n2573, n2583, n2613 |
| U | NR_TDD_FR2_U |  |
| V | NR_TDD_FR2_V |  |
| W | NR_TDD_FR2_W | n2622 |
| X | NR_TDD_FR2_X |  |
| Y | NR_TDD_FR2_Y | n2603 |
| Z | NR_TDD_FR2_Z |  |
| AA | NR_TDD_FR2_AA | n2593 |
| AB | NR_TDD_FR2_AB |  |
| AC | NR_TDD_FR2_AC |  |
| AD | NR_TDD_FR2_AD |  |
| AE | NR_TDD_FR2_AE | n2623 |
| NOTE 1: UE power class 1. NOTE 2: UE power class 2. NOTE 3: UE power class 3. NOTE 4: UE power class 4. NOTE 5: UE power class 5. |  |  |

- Summary

The RRM core and performance requirements for n259 for power class 5 comprising of minimum SSB_RP and minimum CSI-RS_RP levels for the conditions in the annex B of TS 38.133 can be derived based on REFSENS and EIS coverage. They are 2.3 dB higher than those for band n258 for PC5.

CR to 38.133 on core and RRM performance requirements defining band group and the conditions for band n259 for UE PC5 is provided in [4].

- References

- RP-210875,	“New WID on Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n259”.

- R4-2112871	Remaining core part requirement for FWA, Sony, Ericsson

- R4-2107839,	“WF on PC5 requirements in n259”, Qualcomm Incorporated

- R4-2114466, RRM requirements for FR2 FWA for band n259 in 38.133, Ericsson