3GPP TSG-RAN WG4 Meeting #100-e                                         R4-2114519

Electronic Meeting, August 16-27, 2021

Source: 	Huawei, HiSilicon

Title: 	Link level evaluation for Pi/2 BPSK

Agenda Item:	10.6.5

Document for:	Approval

# Introduction

Link level simulation is one of the study aspect based on discussion in last RAN4 meeting [1]:

The Table 1 below is the recommended parameter list for link level simulation.

Table 1: Recommended Parameter list

| Parameter | Value |
| --- | --- |
| Pulse shaping filter | Filter configuration conforms to 38.101-1 |
| Channel model | TDL-C300ns, TDL-A30, TDL-D30 |
| MCS | 0 |
| Code rate | 1/8, 1/4, 1/3, 1/2, 2/3 |
| Waveform | DFTS OFDM with pi/2 BPSK filtered by same filter as for Rel-16 DMRS |
| # of DMRS symbols/slot | 2 |
| # of data symbols/slot | 12 |
| # of RBs | [2, 4, 8, 16, 64] |
| TX/RX configuration | 1TX/4RX |
| BW | 100 MHz |
| UE speed | 3 km/h |
| SCS | 30 kHz |
| HARQ configuration | No retransmissions |

This contribution provides the our simulation results and corresponding proposals for the evaluation.

# Discussion

## Performance comparison of shaping filters

With the simulation assumptions in Table 1, Figure 1 shows the BLER performance for NLoS channel with narrow (100MHz, 8 PRB) and large bandwidth allocation (100MHz, 64 PRB), respectively.

- Small RB allocation (8PRB)						(b) Large RB allocation (64PRB)

Figure 1: BLER performance of different shaping filters

It can be observed that, in NLoS channel according to the simulation assumptions, FDSS causes non-negligible demodulation loss especially for narrow bandwidth case, however, the performance for different shaping filter implementations are similar.

Conceptually, the channel non-flatness caused by channel fading or FDSS filter will bring negative impact on equalization due to the noise enhancement. An FDSS filter with a relatively flat shape is better in terms of equalization loss. With increasing bandwidth allocation, better frequency diversity gain may compensate the loss and reduce the performance gap between different FDSS filters.

In addition, with FDSS filter optimization (e.g. truncated RRC filter), the demodulation performance loss will become smaller. However, the cost of demodulation performance improvement is PAPR. The FDSS filter with wide main-lobe width is preferred from the receiver demodulation perspective, while the PAPR improvement of such filter is limited. Therefore, the FDSS filter design is a trade-off between transmitter PAPR and receiver demodulation performance.

Observation 1: Shaping filter schemes suffer from demodulation degradation in fading channels, and the performance loss can be alleviated by bandwidth allocation or FDSS filter optimization.

Observation 2: The FDSS filter design trades off between the PAPR performance at the transmitter and the receiver demodulation performance.

Observation 3: The performance loss in fading channel for different shaping filter schemes are very similar.

Proposal 1: Shaping filter scheme is up to UE implementation, which should not be limited to certain type during the study

## Transparent vs non-transparent approach

As we discussed in [3], Pi/2 BPSK with transparent approach in the specification is adopted in Rel-15. The TRX diagram of spec transparent approach is shown in Figure 2 below.

Figure 2: TRX diagram of specification transparent approach

However, non-transparent approach was mentioned again during the discussion. As a comparison, Figure 3 shows the BLER results between these two approaches (100MHz, 64RB). In the following evaluation, Pi/2 BPSK based DMRS is adopted.It can be observed that the demodulation performance in the transparent approach is almost the same as that of the non-transparent approach, regardless the FDSS filter being used.

Figure 3: BLER performance of transparent and non-transparent approaches

Observation 4: Specification with transparent FDSS schemes, using pi/2-BPSK with certain FDSS filters, don’t hurt much the demodulation performance at the receiver

Proposal 2: No need to re-consider the non-transparent approach in the specification, and FDSS filter should not be specified in the specification, which is an implementation issue

# Conclusion

This contribution provides link level evaluation based on simulation assumptions discussed in last meeting. Based on the results, we have the following observations and proposals for the shaping filter schemes:

Observation 1: Shaping filter schemes suffer from demodulation degradation in fading channels, and the performance loss can be alleviated by bandwidth allocation or FDSS filter optimization.

Observation 2: The FDSS filter design trades off between the PAPR performance at the transmitter and the receiver demodulation performance.

Observation 3: The performance loss in fading channel for different shaping filter schemes are very similar.

Proposal 1: Shaping filter scheme is up to UE implementation, which should not be limited to certain type during the study

Observation 4: Specification with transparent FDSS schemes, using pi/2-BPSK with certain FDSS filters, don’t hurt much the demodulation performance at the receiver

Proposal 2: No need to re-consider the non-transparent approach in the specification, and FDSS filter should not be specified in the specification, which is an implementation issue

# References

[1] R4-2108018, WF on waveform configuration, parameters for link simulations and agreements, Qualcomm

[2] R4-2114518, Shaping filter characteristics for Pi/2 BPSK, Huawei, HiSilicon

[3] R4-2111449, On feasibility of power enhancement for Pi_2 BPSK, Huawei, HiSilicon