3GPP TSG-RAN WG4 Meeting # 100-e                                                        	R4-2114528

# Electronic Meeting, 16th August – 27th August, 2021

Agenda Item:	9.1.3.1

Source: 			Huawei, HiSilicon

Title:                 TP to TS38.151: on FR1 2x2 BS beam selection

Document for:	Approval

# 1	Introduction

As discussed in [1], cited as below. We propose to adopt same principle for FR1 2x2 BS beam selection to avoid any further confusion.

- FR1 BS beam selection for channel modelling

  - Update the rule for beam selection:

    - For 4x4 MIMO OTA, two strongest transmitting beams from a pre-defined beam grid are selected for each FR1 channel model. These beams should have different azimuth directions and can provide the highest receive power for UE.

# 2	Text Proposal to TS 38.151

--------------Start of text proposal -------------

C.2		FR1 Base Station beam configuration

The emulated BS beam configuration to be used for all emulation of channel models defined in Annex C.1 is specified in this clause.

The Base Station beam configuration includes basic antenna parameters and beamforming characteristic. The basic BS antenna parameters is defined in Table C.2-1.

Table C.2-1: BS Antenna Parameters

| Parameter description | Symbol | Parameter value |  |  |
| --- | --- | --- | --- | --- |
|  |  | FR1 ≤2.5GHz | FR1 >2.5GHz | FR2 |
| Antenna panels in vertical dimension | Mg | 1 | 1 | 1 |
| Antenna panels in horizontal dimension | Ng | 1 | 1 | 1 |
| Elements per panel in vertical dimension | Me | 4 | 8 | 8 |
| Elements per panel in horizontal dimension | Ne | 8 | 8 | 16 |
| Number of polarizations per panel | P | 2 | 2 | 2 |
| Element spacing in horizontal dimension () | dH | 0.5 | 0.5 | 0.5 |
| Element spacing in vertical dimension () | dV | 0.5 | 0.5 | 0.5 |

The beamforming characteristic of the FR1 BS pattern is defined as follow:

-	A code book of 60 fixed beams is constructed to a grid of five elevation angles from –20 to +20 with 10 steps and 12 azimuth angles from –80 to +80 with ~15 steps；

-	For 4x4 MIMO OTA, 2 strongest transmitting beams are selected from the pre-defined beam grid based on their proximity to the strong clusters of each FR1 channel model. For 2x2 MIMO OTA, 1 strongest transmitting beam is selected from the pre-defined beam grid which provides the highest received power for the UE based on the FR1 channel model.

--------------End of text proposal -------------

# Reference

[1] R4-2108085, WF on NR MIMO OTA, vivo, CAICT, TSG-RAN WG4 Meeting #99-e

[2]3GPP TS 38.151 v0.4.0