3GPP TSG-RAN WG4 Meeting #100-e	R4- 2114576

Online, August 16-27, 2021

Agenda item:	9.20.3.4

Source: 	Qualcomm Incorporated

Title: 	RRM relaxation enhancements for Reduced Capability UE

Document for:	Discussion

# Introduction

In RAN#90e meeting, a Rel-17 work item for support of reduced capability NR devices was approved and the WID was further updated in RAN#92e [1]. As per the work item, at least the following key objectives are likely to have direct impact on RRM requirements (impacts from other objectives are not precluded)

- Support for UE complexity reduction features

- Support for Extended DRX enhancements

- RRM relaxation for neighbor cells

Specify support for the following RRM measurement relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected [RAN2, RAN4]:

- Specify measurement (RSRP/RSRQ) based stationarity criterion and not-at-cell-edge criterion [RAN2]

    - Enabling/disabling of RRM measurement relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM measurement relaxation.

  - Specify UE requirements for RRM measurement relaxation [RAN4]

  - No RRM measurement relaxations are specified for the serving cell.

The following Extended DRX enhancements are agreed to be supported by a reduced capability UE according to the latest WID[1]:

In this section, we provide some of the aspects that can be considered in defining the UE requirements for RRM measurement relaxation.

# RRM relaxation methods

During Rel-16 NR, RAN4 specified the RRM measurement relaxation methods for RRC_Idle/Inactive modes when one or both of the following scenarios are fulfilled:

- Low mobility scenario

- Not-at-cell-edge scenario

For Rel-17 RedCap UEs, RAN2 has agreed to specify at-least measurement-based stationarity criterion in addition to the above-mentioned Rel-16 criteria. Network may independently configure one or more of these criteria.

Observation 1: RAN2 has agreed to specify an RSRP/RSRQ based Rel-17 stationarity criterion in addition to the Rel-16 low mobility and not-at-cell-edge criteria.

Observation 2: Network may choose to configure one or more of Rel-16 and Rel-17 criteria

Observation 3: RAN4 defines a measurement relaxation factor, K = 3 when one of the Rel-16 criteria is met and a relaxation of up-to one hour when both Rel-16 criteria are met.

Since stationarity criterion is more stringent than the other two, and a stationary UE is not likely to perform cell-reselection very frequently, it makes sense to provide longer measurement relaxations. Furthermore, a stationary UE which is not at the cell edge is not likely to perform cell re-selection for a very long duration, so measurements could be further relaxed in this case.

Proposal 1: During Idle/Inactive mode, when a UE is configured with and fulfils the stationarity criterion, then irrespective of other criteria being configured and/or fulfilled, it may relax the neighbour cell measurements by at-least one hour. Exact value is FFS.

Proposal 2: During Idle/Inactive mode, when a UE is configured with and fulfils the stationarity criterion and additionally fulfils Rel 16 and/or Rel-17 (if defined by RAN2) not-at-cell-edge criteria, the measurement relaxation could be greater than the one defined for a UE fulfilling only stationarity criterion.

# Conclusion

Observation 1: RAN2 has agreed to specify an RSRP/RSRQ based Rel-17 stationarity criterion in addition to the Rel-16 low mobility and not-at-cell-edge criteria.

Observation 2: Network may choose to configure one or more of Rel-16 and Rel-17 criteria

Observation 3: RAN4 defines a measurement relaxation factor, K = 3 when one of the Rel-16 criteria is met and a relaxation of up-to one hour when both Rel-16 criteria are met.

Proposal 1: During Idle/Inactive mode, when a UE is configured with and fulfils the stationarity criterion, then irrespective of other criteria being configured and/or fulfilled, it may relax the neighbour cell measurements by at-least one hour. Exact value is FFS.

Proposal 2: During Idle/Inactive mode, when a UE is configured with and fulfils the stationarity criterion and additionally fulfils Rel 16 and/or Rel-17 (if defined by RAN2) not-at-cell-edge criteria, the measurement relaxation could be greater than the one defined for a UE fulfilling only stationarity criterion.

# References

[1] RP-211574, “Revised WID on support of reduced capability NR devices”, Ericsson, RAN#92e

[2] R4-2108359, “WF on RedCap RRM requirements”, Ericsson, RAN4#99e