3GPP TSG RAN WG4 Meeting #100-e	R4-2114839

Online, 16th – 27th August 2021

Agenda item:	8.19.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-11-21: EN-DC_8_n77-n79

Document for:	Approval

1	Introduction

EN-DC of 1B LTE and 2B NR of DC_8_n77-n79 was approved in RAN#92 [1]. This TP is to capture the basic aspects for the EN-DC.

2	Text Proposal

[Unchanged Parts Skipped]

6.X	DC_8_n77-n79

6.X.1	Operating bands for DC

Table 6.X.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n77-n79 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.X.2	Channel bandwidths per operating band for DC

Table 6.X.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n77A-n79A | DC_8A_n77A DC_8A_n79A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  |  |  | n77 |  | 15 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

6.X.3	Co-existence studies

Based on the co-existence studies of DC_8A-n77A and DC_8A-n79A, 2nd, 4th and 5th order IMD generated by dual uplink of Band 8 + Band n77 may also fall into own Rx of band n79 and 2nd order IMD generated by dual uplink of Band 8 + Band n79 may also fall into own Rx of band n77. But those IMDs are not occurred since the non-simultaneous Rx/Tx operation is considered for CA_n77-n79.

6.X.4	∆TIB and ∆RIB values

For DC_8_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n77-n79 | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 6.X.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n77-n79 | 8 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |

6.X.5	MSD

As mentioned in 6.X.3, there is no need to specify additional MSD requirement for this UL DC configuration.

[Unchanged Parts Skipped]

7.  Reference

[1]  RP-211043  Revised WID on DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) in Rel-17, LG Electronics