3GPP TSG-RAN WG4 Meeting #100e                                                             R4-2114877

Electronic Meeting, 16-27 Aug., 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 15.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.101-1: Inter-band NR CA Tx requirement including single carrier UL configuration. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2021-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As discussed in R4-2112904, Some of the Tx requirements are only defined for ‘nXA-nYA’ type of UL CA configurations, which means single carrier, i.e. ‘-’ type of UL CA configurations is missing. Also as mentioned in R4-2113021, the requirements for “Output power dynamics for CA” for “inter-band carrier aggregation with one uplink carrier” were defined in Rel-15 in Clause 6.3A. However, with introduction of more CA configurations in Rel-16, this part of requirements become a “hanging paragraph” which was not permitted by TS drafting rule defined in clause 5.2.4 in TS 21.801. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The definition/description of some Tx requirements are added to support single carrier, i.e. ‘-’ type of UL CA configurations. - Move the contents of hanging paragrap of requirments “inter-band carrier aggregation with one uplink carrier” in 6.3A into respective clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Incomplete inter-band UL CA requirements, meanwhile hanging paragrap which was not allowed by TS drafting rule remains in clause 6.3A. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.3, 6.2A.3.1.3, 6.3A, 6.3A.2.3, 6.3A.3.3, 6.4A.1.3, 6.4A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-1 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<< Start of change >>

6.2A.2	UE maximum output power reduction for CA

6.2A.2.1	Void

6.2A.2.2	Void

6.2A.2.3	UE maximum output power reduction for Inter-band CA

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the requirements in subclause 6.2.2 apply.

For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.2 apply for each uplink component carrier.

6.2A.3	UE additional maximum output power reduction for CA

6.2A.3.1.1	Void

6.2A.3.1.2	Void

6.2A.3.1.3	UE additional maximum output power reduction for Inter-band CA

Unless otherwise stated, for inter-band carrier aggregation with one uplink carrier assigned to one NR band, the requirements in subclause 6.2.3 apply, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.3 apply for each uplink component carrier.

<Next change>

6.3A	Output power dynamics for CA

6.3A.1	Minimum output power for CA

6.3A.1.1	Void

6.3A.1.2	Void

6.3A.1.3	Minimum output power for inter-band CA

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the minimum output power requirements in clause 6.3.1 apply.

For inter-band carrier aggregation with uplink assigned to two NR bands, the minimum output power is defined per carrier and the requirement is specified in clause 6.3.1.

6.3A.2	Transmit OFF power for CA

6.3A.2.1	Void

6.3A.2.2	Void

6.3A.2.3	Transmit OFF power for inter-band CA

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the transmit OFF power requirements in subclause 6.3.2 apply.

For inter-band carrier aggregation with uplink assigned to two NR bands, the transmit OFF power specified in clause 6.3.2.1 is applicable for each component carrier when the transmitter is OFF on all component carriers. The transmitter is considered to be OFF when the UE is not allowed to transmit on any of its ports.

6.3A.3	Transmit ON/OFF time mask for CA

6.3A.3.1	Void

6.3A.3.2	Void

6.3A.3.3	Transmit ON/OFF time mask for inter-band CA

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the transmit ON/OFF time mask requirements in subclause 6.3.3 apply.

For inter-band carrier aggregation with uplink assigned to two NR bands, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

<Next change>

6.4A	Transmit signal quality for CA

6.4A.1	Frequency error for CA

6.4A.1.1	Void

6.4A.1.2	Void

6.4A.1.3	Frequency error for inter-band CA

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the frequency error requirements in subclause 6.4.1 apply.

For inter-band carrier aggregation with uplink assigned to two NR bands, the frequency error requirements defined in clause 6.4.1 shall apply on each component carrier with all component carriers active.

6.4A.2	Transmit modulation quality for CA

6.4A.2.1	Void

6.4A.2.2	Void

6.4A.2.3	Transmit modulation quality for inter-band CA

For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the transmit modulation quality requirements in subclause 6.4.2 apply.

For inter-band carrier aggregation with uplink assigned to two NR bands, the transmit modulation quality requirements shall apply on each component carrier as defined in clause 6.4.2 with all component carriers active: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

<< End change >>