3GPP TSG-RAN WG4 Meeting #100-e	R4-2114904

16 – 27 August 2021

Source:	Qualcomm Incorporated

Title:	Calculation of MSD for n79 for V2X_n79A-n47A and V2X_n79A-47A and accompanying TP

Agenda item:	8.25.2

Work Item:	          NR_LTE_V2X_PC5_combos-Core

Document for:	Approval

# 1.	Introduction

In RAN4#98bis-e new inter-band V2X combinations V2X_n79A-n47A and V2X_n79A-47A were introduced [1,2] and is RAN4#99-e MSDs for n47 and B47 were agreed for these V2X combinations [3]. However, MSD for band n79 has yet to be decided. In this paper we present the justification for the required MSD for n79 for these V2X band combinations.

# 2. 	Discussion

In [1,2] the inter-band combinations V2X_n79A-n47A and V2X_n79A-47A were introduced. Though the frequency separation between these bands is larger than 800MHz analysis has shown that there is extensive PA noise coupling from the TX of n47/B47 into the RX of band n79. The TX noise coupling into the RX bands causes RX sensitivity degradation. Below the PA noise and associated TX to RX filtering budget is detailed using the performance of commercially available components and practically achievable isolation numbers.

Table 1: TX to RX Noise budget

|  | CV2x n47/B47 PA-->n79 Rx |
| --- | --- |
| BandWidth(MHz) | 40 |
| 3GPP Sensitivity(dBm) | -89.6 |
| Interfering PA Noise dBm/Hz | -112 |
| Tx-Rx Antenna Isolation | -20 |
| Tx Filter Rejection+ Triplexer Rej | -33 |
| Total PA noise (dBm) | -89 |
| 3GPP sensitivity+ PA noise (dBm) | -86.3 |
| De-Sense(dB) | 3.3 |

As can be seen from the above table the PA noise from n47/B47 leads to approximately 3.3 dB of desensitization of the n79 RX bands. Therefore, it is proposed that the following MSD value be adopted for n79 in band combinations V2X_n79A-n47A and V2X_n79A-47A:

Table 2 MSD for band n79 for V2X_n79A-47A

| V2X inter-band con-current band Combination | E-UTRA or NR Band | MSD [dB] |
| --- | --- | --- |
| V2X_n79-47 | n79 | 3.3 |

Table 3 MSD for n79 V2X_n79A-n47A

| V2X inter-band con-current band Combination | E-UTRA or NR Band | MSD [dB] |
| --- | --- | --- |
| V2X_n79-n47 | n79 | 3.3 |

We also note that these MSDs are optional and designs that have better performance do not need to adopt them. We make the following proposals:

Proposal 1: The MSD value of 3.3 dB given in table 2 be adopted for band n79 in V2X_n79A-47A

Proposal 2: The MSD value of 3.3 dB given in table 3 be adopted for band n79 V2X_n79A-n47A

Also, it is our understanding that the MSDs for bands n79, n47 and B47 in the CA combinations V2X_n79A-47A and  V2X_n79A-n47A  should apply for all BWs and SCS combinations.

Proposal 3: MSDs for n79, n47 and B47 in CA combinations V2X_n79A-47A and  V2X_n79A-n47A  apply for all BWs and SCS combinations.

# Conclusion

We present the analysis for MSD for inter-band combinations V2X_n79A-47A and V2X_n79A-n47A and make the following proposals. Also, proposals on the optionality of supporting inter-band V2X combinations are outlined.

Proposal 1: The MSD value of 3.3 dB given in table 2 be adopted for band n79 in V2X_n79A-47A

Table 2 MSD for band n79 for V2X_n79A-47A

| V2X inter-band con-current band Combination | E-UTRA or NR Band | MSD [dB] |
| --- | --- | --- |
| V2X_n79-47 | n79 | 3.3 |

Proposal 2: The MSD value of 3.3 dB given in table 3 be adopted for band n79 V2X_n79A-n47A

Table 3 MSD for n79 V2X_n79A-n47A

| V2X inter-band con-current band Combination | E-UTRA or NR Band | MSD [dB] |
| --- | --- | --- |
| V2X_n79-n47 | n79 | 3.3 |

Proposal 3: MSDs for n79, n47 and B47 in CA combinations V2X_n79A-47A and  V2X_n79A-n47A  apply for all BWs and SCS combinations.

# Annex - TP for TR 37.875 on MSD for n79 for V2X_n79A-n47A and V2X_n79A_47A

## --Start of changes1

#### 6.2.4.4	MSD

The REFSENS exception due to cross band isolation can be observed for V2X_n79A-n47A. One MSD test configuration is specified as below. The MSD values given in tables 6.2.4.1-1and 6.2.4.1-2 shall apply for all n79 and n47 BWs and SCSs combinations given in table 6.2.4.2-1

Table 6.2.4.1-1: Reference sensitivity exceptions (MSD) due to cross band isolation for V2X

| UL band | SL reception band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of SL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | n47 | 15 | 216 | 40 | 50 | 10 | 3.3 |

Table 6.2.4.1-2: Reference sensitivity exceptions (MSD) due to cross band isolation for Uu

| SL transmission band | DL band | SCS of UL/DL band (kHz) | LCRB of SL transmission band | SL transmission BW(MHz) | LCRB of DL band | DL band BW (MHz) | MSD value of DL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n47 | n79 | 15 | 50 | 10 | 216 | 40 | 3.3 |
| SL transmission band | DL band | SCS of UL/DL band (kHz) | LCRB of SL transmission band | SL transmission BW(MHz) | LCRB of DL band | DL band BW (MHz) | MSD value of DL band (dB) |
| n47 | n79 | 15 | 50 | 10 | 216 | 40 | 2.2 |

## --End of changes1

## --Start of changes2

#### 6.3.4.4	MSD

The REFSENS exception due to cross band isolation can be observed for V2X_n79A_47A. One MSD test configuration is specified as below. The MSD values given in tables 6.3.4.4-1 and 6.3.4.4-2 shall apply for all n79 and 47 BWs and SCSs combinations given in table 6.3.3.2-1

Table 6.3.4.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR V2X

| UL aggressor band | SL Victim band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of DL band | DL band BW (MHz) | MSD value of DL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | 47 | 15 | 216 | 40 | 50 | 10 | 3.3 |

Table 6.3.4.4-2: Reference sensitivity exceptions (MSD) due to cross band isolation for Uu

| SL transmission band | DL band | SCS of UL/DL band (kHz) | LCRB of SL transmission band | SL transmission BW(MHz) | LCRB of DL band | DL band BW (MHz) | MSD value of DL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 47 | n79 | 15 | 50 | 10 | 216 | 40 | 3.3 |
| SL transmission band | DL band | SCS of UL/DL band (kHz) | LCRB of SL transmission band | SL transmission BW(MHz) | LCRB of DL band | DL band BW (MHz) | MSD value of DL band (dB) |
| 47 | n79 | 15 | 50 | 10 | 216 | 40 | 2.2 |

--End of changes2

# References

[1] R4-2105367, CR for TS 38.101-1, Introduce new band combination of V2X_n79A-n47A, RAN4#98-bis, Apr. 12-20 2021

[2] R4-2105368, CR for TS 38.101-3, Introduce new band combination of V2X_n79A-47A, RAN4#98-bis, Apr. 12-20 2021

[3] R4-2107815, Discussion and TP for TR 38.375 on SMD for V2X_n79A-n47A and V2X_n79A_47A, RAN#99-e, May 19-28 2021