3GPP TSG-RAN WG4 Meeting # 100-e                             R4-2114905

Electronic Meeting, 16– 27 August, 2021

Source: 	Huawei, HiSilicon, Qualcomm

Title: 	WF on Type 2 UE RX Imbalance Requirement

Agenda Item:	6.1.9.2

Document for:	Approval

Background

Based on the approved WF [1] in last RAN4 meeting, RAN4 need to specify the requirements for Type 2 UE with DL carrier power imbalance. In this meeting, companies provided some analysis and proposals [2][3]. In this paper, we’d like to list all the options for further discussion.

Way forward

Proposal 1: The table format is agreeable. There are some options on how to specify the requirements for Type 2 UE with DL carrier power imbalance.

- Option 1a:

| Carriers | Power in transmission bandwidth configuration (dBm) | channel bandwidth | Frequency relationship |
| --- | --- | --- | --- |
| Wanted carrier | REFSENS + 14 dB | BWwanted ≤ BWanother | Place two DL carriers as close as possible |
| Another carrier with overlapping DL bands | Power of wanted carrier + 31.5 dB |  |  |
| Wanted carrier | REFSENS + 14 dB | BWwanted > BWanother |  |
| Another carrier with overlapping DL bands | Power of wanted carrier + 31.5 – 10*log10(BWwanted /BWanother) dB |  |  |

- Option 1b:

| Carriers | Power in transmission bandwidth configuration (dBm) | channel bandwidth | Frequency relationship |
| --- | --- | --- | --- |
| Wanted carrier | REFSENS + 1 dB | BWwanted ≤ BWanother | Place two DL carriers as close as possible |
| Another carrier with overlapping DL bands | Power of wanted carrier + 25 dB |  |  |
| Wanted carrier | REFSENS + 1 dB | BWwanted > BWanother |  |
| Another carrier with overlapping DL bands | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) dB |  |  |

- Option 2 (Gap between DL carriers ≥ 50MHz):

| Carriers | Power in transmission bandwidth configuration (dBm) | channel bandwidth | Frequency relationship |
| --- | --- | --- | --- |
| Wanted carrier | REFSENS + 1 dB | BWwanted ≤ BWanother | max (5/2*another DL BW, 50MHz) |
| Another carrier with overlapping DL bands | Power of wanted carrier + 25 dB |  |  |
| Wanted carrier | REFSENS + 1 dB | BWwanted > BWanother |  |
| Another carrier with overlapping DL bands | Power of wanted carrier + 25 dB – 10*log10(BWwanted /(5*min(BWanother, 20MHz))) |  |  |

- Option 3: Combination of option 1 and option 2

- Option 4: Other parameters are not precluded.

Proposal 2: To verify impact on other RX requirements with imbalanced DL carrier by next meeting, i.e. November meeting.

- References

[1]	R4-2107784	WF on Rx Requirements for Type 2 UE, Qualcomm Incorporated

[2] R4-2111732	Imbalance Requirement for Type 2 UE RX, Qualcomm Incorporated

[3] R4-2113403	Discussion on type 2 UE requirements, Huawei, HiSilicon