3GPP TSG-WG RAN4 Meeting #100-e	R4-2114931

Electronic meeting, August 16-27, 2021

WF on UE PC2 dutycycle SAR solutions

China Telecom

Background

The contributions submitted in RAN4 #100e meeting for SAR solutions and UE maximum power:

- R4-2111766, A way to increase UE maximum power for NR uplink inter band CA, Nokia, Nokia Shanghai Bell

- R4-2112047, Discussion on increasing maximum output power for UE PC2 CA, Mediatek India Technology Pvt.

- R4-2112382, Clarifications on NR FR1 inter-band UL CA power class, Apple

- R4-2112490, Discussion on how to introduce SAR schemes for UE power class 2 NR inter-band CA and SUL configurations, China Telecom

- R4-2112491, CR to 38.101-1 Introduce SAR solution for UE power class 2 NR inter-band CA and SUL configurations, China Telecom

- R4-2112492, LS on UE capability for UE power class 2 NR inter-band CA and SUL configurations, China Telecom

- R4-2112998, Further discussion on the dutycycle threshold calculation for HPUE with 2UL inter-band CA, vivo

- R4-2113305, Discussion on increasing UE maximum power high limit, Xiaomi

- R4-2113903, R17 Discussion on UE power class high limit, OPPO

- R4-2113904, R17 Inter band CA HPUE SAR, OPPO

- R4-2114209, Further Discussion on Higher UE Power Limits for Inter-band CA/DC, Huawei, HiSilicon

Agreement on dutycycle SAR solution

- Agreement on dutycycle SAR solution

  - Agree the CR R4-2114932 CR to 38.101-1 Introduce SAR solution for UE power class 2 NR inter-band CA and SUL configurations

- Agreement on dutycycle capability reporting

  - The values and range is listed as below

    - {50%, 60%, 70%, 80%, 90%,100%}.

  - From network perspective, PC2 is supported regardless of duty cycle when the duty cycle signalling is absent.

  - From UE perspective,

    - UE does not falls back to PC3 when the duty cycle signalling is absent

    - UE will use P-MPR to meet SAR requirement when the duty cycle signalling is absent

  - Agree the LS R4-2114933 LS on UE capability for UE power class 2 NR inter-band CA and SUL configurations