3GPP TSG-RAN WG4 Meeting # 100-e 												         R4-2115202

Electronic Meeting, 16th – 27th Aug., 2021

Agenda item:			6.1.7

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [100-e][212]NR_RRM_Enh

Document for:	Information

# Introduction

The email discussion is intended to cover topics related to R16 NR RRM Enhancements (NR_RRM_Enh-Core/Perf) in AI 6.1.7.

# Topic #1: RRM core requirements (6.1.7.1)

## Companies’ contributions summary

Moderator note: R4-2112261 will be treated in this thread instead of #205.

| R4-2112117 | Correction on SMTC alignment for multiple SCell activation R16 | Apple, Qualcomm, Huawei, HiSilicon |
| --- | --- | --- |
| R4-2112118 | Correction on SMTC alignment for multiple SCell activation R17 | Apple, Qualcomm, Huawei, HiSilicon |
| R4-2112532 | Correction on the SRS carrier switching in EN-DC and NE-DC in R16 | MediaTek inc. |
| R4-2112533 | Correction on the SRS carrier switching in EN-DC and NE-DC in R17 | MediaTek inc. |
| R4-2112685 | CR for multiple Scell activation requirements (R16) | Apple |
| R4-2112686 | CR for multiple Scell activation requirements (R17) | Apple |
| R4-2112693 | Rel-16 Cat-A CR to FR1 Multiple SCell activation requirement for SSB-less and TCI activation | Qualcomm Incorporated |
| R4-2112694 | Rel-17 Cat-A CR to FR1 Multiple SCell activation requirement for SSB-less and TCI activation | Qualcomm Incorporated |
| R4-2112695 | Rel-16 Cat-F CR to FR1 Multiple SCell activation requirement for SSB-less and TCI activation | Qualcomm Incorporated |
| R4-2112696 | Rel-17 Cat-A CR to FR1 Multiple SCell activation requirement for SSB-less and TCI activation | Qualcomm Incorporated |
| R4-2113635 | draftCR on TS38.133 mandatory gaps - r16 | Ericsson, Mediatek Inc. |
| R4-2113636 | draftCR on TS38.133 mandatory gaps - r17 | Ericsson, Mediatek Inc. |
| R4-2113850 | Discussion on mandatory gap pattern in R-16 Proposal 1: Suggest to keep current 1 bit signalling for NR only measurement in LTE SA, EN-DC and NE-DC. | Huawei, HiSilicon |
| R4-2114211 | CR on RRC-based BWP switch on multiple CCs in Rel16 | Nokia, Nokia Shanghai Bell |
| R4-2114212 | CR on RRC-based BWP switch on multiple CCs in Rel17 - Cat A | Nokia, Nokia Shanghai Bell |
|  | On Mandatory GP Signaling |  |
| R4-2112261 | Proposal 1: Add new bitmaps for signaling the support of NR-only measurement GPs in LTE-SA following NR-SA and NR-DC with the capability being optional for all GPs including GP 2,3,11. Proposal 2: The bitmap for NR-only measurement gap patterns support in LTE-SA needs to be consistent with the support of NR-only measurement gap patterns in NR-SA and NR-DC. Otherwise, they should be all zero. Proposal 3: In EN-DC, since UE signals the bitmap for LTE-SA, UE can use one bit to indicate the support for gap patterns as signalled by the bitmaps for LTE-SA. Similarly, in NE-DC, since UE signals the bitmap for NR-SA/DC, UE can use one bit to indicate the support for gap patterns as signalled by the bitmaps for NR-SA/DC. Proposal 4: RAN4 takes one of the following action: - Update note for UE feature list item 9-3 as: Note: Agreements are provided in [TBD]. According to RAN4 agreement, a bitmap should be introduced for LTE-SA and a signle bit should be introduced for EN-DC and NE-DC. - Send a separate LS to RAN2 to ask for new UE capability signaling. | Qualcomm Incorporated |

## Open issues summary and companies view’s collection

### Sub-topic 1-1 Mandatory gap pattern signaling

Issue 1-1: Whether to keep 1 bit signalling for NR only measurement gap patten in LTE SA, EN-DC and NE-DC

- Option 1: Yes (Huawei)

- Option 2: No (Qualcomm):

  - Proposal 1: Add new bitmaps for signaling the support of NR-only measurement GPs in LTE-SA following NR-SA and NR-DC with the capability being optional for all GPs including GP 2,3,11.

  - Proposal 2: The bitmap for NR-only measurement gap patterns support in LTE-SA needs to be consistent with the support of NR-only measurement gap patterns in NR-SA and NR-DC. Otherwise, they should be all zero.

  - Proposal 3: In EN-DC, since UE signals the bitmap for LTE-SA, UE can use one bit to indicate the support for gap patterns as signalled by the bitmaps for LTE-SA. Similarly, in NE-DC, since UE signals the bitmap for NR-SA/DC, UE can use one bit to indicate the support for gap patterns as signalled by the bitmaps for NR-SA/DC.

  - Proposal 4: RAN4 takes one of the following action:

    - (1) Update note for UE feature list item 9-3 as:

Note: Agreements are provided in [TBD]. According to RAN4 agreement, a bitmap should be introduced for LTE-SA and a signle bit should be introduced for EN-DC and NE-DC.

    - (2) Send a separate LS to RAN2 to ask for new UE capability signaling.

.

- Recommended WF:

  - Further discussion.

| Company | Comments |
| --- | --- |
| Apple | No strong view. Existing capability design can work. We are also fine to update LTE-SA capability design to align with NR-SA. One question if RAN4 decides to agree on option 2: wouldn’t it be better to use one bit in NR-DC to align with other DC operation? If so, we can have a bitmap in LTE-SA and NR-SA but one bit for all the DC operation. |
| Ericsson | We prefer Option 1. Our concern is that Option 2 would imply ASN.1 changes to Rel-16. |
| MTK | We prefer to Option 1, to keep the current spec. The benefit and necessity to introduce the new bit map are unclear. 2 nd further comment @ QC: thank you for the clarification. We understand the new bitmap for LTE-SA can allow UE to report the supporting on NR-only measurement with different gaps separately, as NR SA. However, Option 2 applies different approaches in on GP 2,3,11@LTE-SA from the previous RAN4 agreement. The inconsistency would not be the main reason we need to revert RAN4’s previous 1-bit agreement and reverting the LS to RAN2, because it will have RAN2 spec impact, since this is already implemented in ASN.1. Besides, this 1-bit is optional capability for LTE-SA, so UE can decide whether to report it. |
| QC | To Apple: gap pattern support is the same for NR-SA and NR-DC. LTE-SA, NE-DC and EN-DC are different than NR-SA or NR-DC because there are LTE serving cells, and the support to these mandatory gap patterns under these scenarios becomes optional. To Ericsson: this is a backward compatible change, and the new capability signaling is added as an extension. In the previous meeting, RAN4 agreed HST capability change and was done in a similar fashion. To MTK: we listed two important benefits for this change in our contribution, could you clarify which of them is not clear? - In NR-SA, UE can report the support to each NR-only measurement GPs separately, while in LTE SA and EN-DC, UE capability reporting for NR-only measurement GP 2,3,11 support is tied together, and the support of other NR-only measurement GPs can not be signaled to eNB. Introducing this bitmap resolves the inconsistency. - While NR-only measurement GP 2,3,11 should be mandatory supported, in practice, UE still need these capability bits to signal whether these GPs can be supported, since UE may not want to signal the support of these GSs before these GPs has been tested in the field to ensure inter-operability. Without field and inter-operability tests, even if the UE can functionally support these measurement GPs, the UE may run into issues when using these GPs in the field. |
| Huawei | We prefer option1. The current 1 bit to support NR only gap pattern for LTE SA, ENDC, NEDC can work. In LTE SA, ENDC, NEDC, if UE can only support pattern #2, UE reports “0”. In addition, using one bit indicates to support full set of pattern #2,#3,#11 fits the motivation of the “mandatory gap pattern” . -From network perspective, according to the existing design, at most 3 gap patterns (i.e., #2,#3,#11) are supported and configured for NR only measurement for NEDC, SA LTE and ENDC scenario. If changing the 1 bit signalling to a bitmap (corresponds to 25 gap patterns), more gap patterns may be reported as “1”, then network shall adapt and configure appropriate gap pattern according to different UE capability of supported UE gap patterns. Then the result is that network would support all gap patterns finally. This is not what network would like to see. -From UE perspective, either supporting full set of pattern #2,#3,#11 or supporting none of them is simple and explicit to UE implementation. Moreover the benefit of supporting more gap patterns for NR only measurement is not understanding. |
| Intel | We prefer option 1. We think that one bit signalling can already work and it’s a little late to modify this in current stage. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2112117 Apple, Qualcomm, Huawei, HiSilicon | Title: Correction on SMTC alignment for multiple SCell activation R16 |
|  | Ericsson: OK |
|  | MTK: OK |
|  | Nokia: OK. |
| R4-2112532 MediaTek inc. | Title: Correction on the SRS carrier switching in EN-DC and NE-DC in R16 |
|  | Ericsson: OK |
|  | MTK: OK |
|  | Nokia: OK. |
| R4-2112685 Apple | Title: CR for multiple Scell activation requirements (R16) |
|  | Ericsson: Technically OK. Please correct typo: one instance of “period” is incorrectly spelled “perod”. |
|  | Nokia: We have related discussion paper on this topic in another agenda (6.1.4.2.2) R4-2114010. Once that discussion is settled, we can return to this. |
|  | Huawei: OK |
| R4-2112695 Qualcomm Incorporated | Title: Rel-16 Cat-F CR to FR1 Multiple SCell activation requirement for SSB-less and TCI activation |
|  | Apple: the new added paragraph for ‘activation delay with TCI settling’ shall be placed in between of the case of ‘not counted in N1’ and the case of ‘otherwise’; if not, the ‘otherwise’ part in the current spec would cause ambiguity. We propose to re-organize the paragraphs as below: If the SCell is unknown and belongs to FR1, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time_multiple_scells is: - TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells+Trs +5ms, if the SCell is not counted in N1 - The activation delay may be longer if SSB is not in the same half-frame on the SCell and the contiguous FR1 known cell or contiguous FR1 active serving cell - if none of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, Tactivation_time_multiple_scells is:…….. - otherwise, TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells+Trs*N1 +Trs +5ms |
|  | Ericsson: OK. (Also if rearranged as suggested by Apple.) |
|  | Qualcomm: Thanks Apple and Ericsson for the suggestion. We’ll revise the CR as bellow: If the SCell is unknown and belongs to FR1, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time_multiple_scells is: - TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells+Trs +5ms, if the SCell is not counted in N1 - The activation delay may be longer if SSB is not in the same half-frame on the SCell and the contiguous FR1 known cell or contiguous FR1 active serving cell - if none of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, Tactivation_time_multiple_scells is: - 6ms + TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells + Trs*N1 + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC_multiple_scells + TFineTiming + 2ms, Tuncertainty_SP_multiple_scells), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells + Trs*N1 + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC_multiple_scells + 5ms + TFineTiming, Tuncertainty_RRC_multiple_scells + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - otherwise, TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells+Trs*N1 +Trs +5ms If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time_multiple_scells is same as single SCell activation delay requirement as defined in clause 8.3.2. If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time_multiple_scells is same as single SCell activation delay requirement as defined in clause 8.3.2. |
|  | Nokia: Agreeable as such. However, ‘is’ -> ‘are’ in ‘if none of the following conditions are met’ Wording is a bit hard in terms of saying ‘none of the..’ and then ‘‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication’. So we’re wondering if the readability can be improved? E.g.: - if the following conditions are not met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, Tactivation_time_multiple_scells is:…, |
|  | Huawei: We are fine with the latest TP from QC, but it seems one “otherwise” as highlighted in yellow is missing. Below is our understanding on the logic, where “if” and “otherwise” on the same level are marked with same color. If the SCell is unknown and belongs to FR1, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time_multiple_scells is: - TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells+Trs +5ms, if the SCell is not counted in N1 - The activation delay may be longer if SSB is not in the same half-frame on the SCell and the contiguous FR1 known cell or contiguous FR1 active serving cell otherwise - if none of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, Tactivation_time_multiple_scells is: - 6ms + TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells + Trs*N1 + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC_multiple_scells + TFineTiming + 2ms, Tuncertainty_SP_multiple_scells), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells + Trs*N1 + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC_multiple_scells + 5ms + TFineTiming, Tuncertainty_RRC_multiple_scells + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - otherwise, TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells+Trs*N1 +Trs +5ms |
|  | Intel: Generally fine with Qualcomm’s updated version. |
|  | Qualcomm2: Thanks for the input. Based on all received comments, we can revise the CR as below: If the SCell is unknown and belongs to FR1, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time_multiple_scells is: - TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells+Trs +5ms, if the SCell is not counted in N1 - The activation delay may be longer if SSB is not in the same half-frame on the SCell and the contiguous FR1 known cell or contiguous FR1 active serving cell otherwise - if the following conditions are not met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, Tactivation_time_multiple_scells is: - 6ms + TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells + Trs*N1 + TL1-RSRP,measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC_multiple_scells + TFineTiming + 2ms, Tuncertainty_SP_multiple_scells), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells + Trs*N1 + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC_multiple_scells + 5ms + TFineTiming, Tuncertainty_RRC_multiple_scells + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - otherwise, TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells+Trs*N1 +Trs +5ms If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time_multiple_scells is same as single SCell activation delay requirement as defined in clause 8.3.2. If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time_multiple_scells is same as single SCell activation delay requirement as defined in clause 8.3.2. |
| R4-2113635 Ericsson, Mediatek Inc. | Title: draftCR on TS38.133 mandatory gaps - r16 |
|  | Nokia: OK |
| R4-2114211 Nokia, Nokia Shanghai Bell | Title: CR on RRC-based BWP switch on multiple CCs in Rel16 |
|  | Ericsson: OK |
|  | MTK: OK |
|  | Intel: OK |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1 | Issue 1-1: Whether to keep 1 bit signalling for NR only measurement gap patten in LTE SA, EN-DC and NE-DC Tentative agreement: No. Candidate options: - Option 1: 4 company support (Ericsson, MTK, Huawei, Intel) - Option 2: 1 company support (Qualcomm) Recommendations for 2nd round: further discussion. Can proponent of option 2 compromise to option 1? |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2112117 Apple, Qualcomm, Huawei, HiSilicon | Agreeable |
| R4-2112532 MediaTek inc. | Agreeable |
| R4-2112685 Apple | Return to |
| R4-2112695 Qualcomm Incorporated | Revised |
| R4-2113635 Ericsson, Mediatek Inc. | Agreeable |
| R4-2114211 Nokia, Nokia Shanghai Bell | Agreeable |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

|  | Status summary |
| --- | --- |

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

# Topic #2: RRM performance requirements (6.1.7.2)

## Companies’ contributions summary

| R4-2112081 | On test applicability for mandatory gap patterns Observation 1: according to existing test applicability, UE is allowed to skip some test with GP#0 if UE can survive more demanding test, such as test with GP#4 in FR1. Observation 2: with the same assumption in R15 test design, if the UE can successfully pass the new test case configured new mandatory gap pattern, it can also survive the corresponding test case with “legacy” MG pattern. Observation 3: besides newly introduced tests configured with #2, #3 and #17, there are still quite a lot of existing test cases configured with “legacy” MG patterns. Since only one sub-test in A.6.6.2.1 will be skipped according to option 2, we don’t see any issue from test coverage perspective. Proposal 1: the following test applicability shall be introduced in R16: - For FR1 test: - UE capable of per-FR gap and GP#4 needs to pass both A.6.6.2.1 and A.6.6.2.9. - UE not capable of either per-FR gap or GP#4 needs to pass A.6.6.2.9 and is allowed to skip A.6.6.2.1 - For FR2 test: - If the UE can pass A.7.6.2.9, it is allowed to skip A.7.6.2.1. | Apple |
| --- | --- | --- |
| R4-2112082 | CR for test applicability for mandatory gap patterns (R16) | Apple |
| R4-2112083 | CR for test applicability for mandatory gap patterns (R17) | Apple |
| R4-2112265 | On Mandatory GP Test Proposal: Define the following applicability rule - For FR1 test: - UE capable of per-FR gap and GP#4 needs to pass both A.6.6.2.1 and A.6.6.2.9. - UE not capable of either per-FR gap or GP#4 needs to pass A.6.6.2.9 and is allowed to skip A.6.6.2.1 - For FR2 test: - If the UE can pass A.7.6.2.9, it is allowed to skip A.7.6.2.1. | Qualcomm, Inc. |
| R4-2114015 | Discussion on test cases for new mandatory GPs - No legacy test cases are skipped due to passing new Rel-16 test cases. A Rel-16 UE shall pass all existing Rel-15 related measurement gap test cases and new Rel-16 defined measurement gap test cases. | Nokia, Nokia Shanghai Bell |
| R4-2114163 | On testing in R16 of R15 mandatory gaps Observation 1: Test cases in A.6.6.2.1 and A.6.6.2.9 are using different MGLs. Substitution of MGPs within each respective test case is between MGPs with same MGL but different MGRP. Observation 2: The proposal in Option 2 concerns substitution between test cases with different MGLs. This is very different from the MGP substitution policy used so far in test cases. Observation 3: Test cases in A.7.6.2.1 and A.7.6.2.9 are using different MGLs. Proposal 1: R15 NR FR1 test cases on mandatory gap patterns in shall be inherited completely to R16 specifications, and R16 UEs shall pass all test cases. Proposal 2: R15 NR FR2 test cases on mandatory gap patterns in shall be inherited completely to R16 specifications, and R16 UEs shall pass all test cases. | Ericsson |

## Open issues summary and companies view’s collection

### Sub-topic 2-1 Test case design of mandatory gap pattern

Issue 2-1: Test case design of mandatory gap pattern for Rel-16

- Option 1(Nokia, Ericsson):

  - For both FR1 and FR2, R15 test cases on mandatory gap patterns shall be inherited completely to R16 specifications, and R16 UEs shall pass all test cases.

- Option 2 (Apple, Qualcomm):

  - For FR1 test:

    - UE capable of per-FR gap and GP#4 needs to pass both A.6.6.2.1 and A.6.6.2.9.

    - UE not capable of either per-FR gap or GP#4 needs to pass A.6.6.2.9 and is allowed to skip A.6.6.2.1

  - For FR2 test:

    - If the UE can pass A.7.6.2.9, it is allowed to skip A.7.6.2.1.

- Recommended WF:

  - Further discussion.

| Company | Comments |
| --- | --- |
| Apple | Support option 2. To address concern from R4-2114015 on FR1 test: “Additionally, if skipping A.6.6.2.1 completely would mean that for FR1 without SSB time index detection when DRX is not used, the 6ms MGL would be left untested.”. We don’t see any issue to skip this one sub-test in A.6.6.2.1. One on hand, as mentioned in our contribution, there are still many other test cases configured with GP#0. So we don’t need to worry about the performance of GP#0. On the other hand, as also mentioned in our contribution, the sub-test 1 in A.6.6.2.1 is already allowed to be skipped if UE support per-FR gap and GP#4, which means this sub-test 1 is something cannot be skipped. To address concern from R4-2114015 on FR2 test: Test case for FR2 It seems that the proposal for FR2 would lead to that the new test case A.7.6.2.9 would allow UE to skip both A.7.6.2.1 and A.7.6.2.5. This would clearly reduce the test coverage as we see it. The proposal would allow UE to skip 2 Rel-15 TCs based on 1 new Rel-16 TC. Looking at A.7.6.2.5 test 1, this test is different than the newly introduced test A.7.6.2.9, as there is PCell in FR1. Hence, this test can therefore not be skipped.. Seems the main concern is on A.7.6.2.5. However, option 2 only allows UE to skip A.7.6.2.1, not A.7.6.2.5, which we believe can be compromised solution to move forward. Some response on observation from R4-2114163: Observation 1: Test cases in A.6.6.2.1 and A.6.6.2.9 are using different MGLs. Substitution of MGPs within each respective test case is between MGPs with same MGL but different MGRP. [Apple]: we raised substitution of MGP in existing A.6.6.2.1 is not to borrow exactly same substitution rule here, but just to demonstrate that sub-test 1 in A.6.6.2.1 is not something cannot be skipped (to reply to the concern from companies in previous RAN4 meetings). Observation 2: The proposal in Option 2 concerns substitution between test cases with different MGLs. This is very different from the MGP substitution policy used so far in test cases. [Apple]: even though the substitution rule is different here, the fundamental spirit is the same: if UE can survive some more demanding test, it is rational to assume UE can survive the less demanding test. Observation 3: Test cases in A.7.6.2.1 and A.7.6.2.9 are using different MGLs. [Apple]: this is exactly the point. They have same MGRP but different MGLs. In A.7.6.2.1 GP#13 is used (5.5ms MGL, 40ms MGRP) while in A7.6.2.9 GP#17 is used (3.5ms MGL, 40ms MGRP). If UE can finish measurement successfully in 3.5ms MGL, it is rational to assume UE can of course finish measurement in 5.5ms. |
| Ericsson | We support Option 1. We do not agree with Apple on that just because UE can finish successfully for one MGL, it will do so for another longer MGL. In our view, there is likely a larger chance for errors when configuring different MGLs, than when configuring different MGRPs for the same MGL. Hence we are not willing to reduce the test coverage by substituting between different MGLs. |
| QC | We support option 2. |
| Nokia | Option 1. We recognise that there may be some overlap between the test A.6.6.2.1 and A.6.6.2.9. But for FR2 the proposal would allow UE to skip 2 Rel-15 TCs based on 1 new Rel-16 TC. But as discussed in previous meeting our view is that the new test cases defined for testing the new mandatory GPs are not copying any existing test cases directly. They do cover different test parameters setting to ensure broader test coverage and with the purpose of testing the new GPs. |
| OPPO | Option 2 is perferred |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2112082 Apple | Title: CR for test applicability for mandatory gap patterns (R16) |
|  | Ericsson: The CR is not agreeable. We have technical concern as well as concern on impact on already deployed networks. |
|  | Nokia: Not agreeable. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1 | Issue 2-1: Test case design of mandatory gap pattern for Rel-16 Tentative agreement: No. Candidate options: - Option 1: 2 company support (Ericsson, Nokia) - Option 2: 3 company support (Apple, Qualcomm, OPPO) Recommendations for 2nd round: further discussion. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2112082 Apple | Return to |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

|  | Status summary |  |
| --- | --- | --- |
| CR/TP/LS/WF number |  | T-doc Status update recommendation |

# Recommendations for Tdocs

## 1st round

Moderator note: since these are draft CR,

Existing tdocs

| Tdoc number | Title |  | Source |  | Recommendation |  |  | Comments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-2112117 | Correction on SMTC alignment for multiple SCell activation R16 | Apple, Qualcomm, Huawei, HiSilicon | Agreeable |  |  |  |
| R4-2112532 | Correction on the SRS carrier switching in EN-DC and NE-DC in R16 | MediaTek inc. | Agreeable |  |  |  |
| R4-2112685 | CR for multiple Scell activation requirements (R16) | Apple | Return to |  | Nokia: We have related discussion paper on this topic in another agenda (6.1.4.2.2) R4-2114010. Once that discussion is settled, we can return to this. |  |
| R4-2112695 | Rel-16 Cat-F CR to FR1 Multiple SCell activation requirement for SSB-less and TCI activation | Qualcomm Incorporated | Revised |  |  |  |
| R4-2113635 | draftCR on TS38.133 mandatory gaps - r16 | Ericsson, Mediatek Inc. | Agreeable |  |  |  |
| R4-2114211 | CR on RRC-based BWP switch on multiple CCs in Rel16 | Nokia, Nokia Shanghai Bell | Agreeable |  |  |  |
| R4-2112082 | CR for test applicability for mandatory gap patterns (R16) | Apple | Return to |  |  |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Apple | Qiming Li | li_qiming@apple.com |
| Apple | Jerry Cui | jie_cui@APPLE.COM |
| Ericsson | Joakim Axmon | joakim.axmon[at]ericsson.com |
| MediaTek Inc. | Hsuanli Lin | Hsuanli.Lin@mediatek.com |
| Nokia | Lars Dalsgaard | lars.dalsgaard@nokia.com |
| Intel | Hua Li | hua.li@intel.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)