3GPP TSG-RAN4 Meeting #100-e	R4-2115210

Electronic Meeting, August 16-27, 2021

Agenda item:			9.10

Source:	Moderator (Apple)

Title:	Email discussion summary for [99e][220] NR_RRM_enh2_1

Document for:	Information

# Introduction

This email discussion summary includes general (9.10.1) and SRS antenna port switching (9.10.2.1).

# Topic #1: SRS antenna port switching (9.10.2.1)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2111926 | CATT | Proposal 1: Don't define the scheduling restriction on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols. Proposal 2: Only defining the requirements for the scenario that the SRS resources of a set are transmitted in the same slot with consecutive SRS transmission. Proposal 3: The aperiodic SRS antenna port switching can impact on L1-RSRP/L1-SINR and RLM/BFD/CBD measurement requirements if the IE txSwitchImpactToRx include the working band for FDD. Proposal 4: Option 1 is agreed, i.e. No impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized. Proposal 5: Option 1 is agreed, i.e. the requirement for handover, BWP switching, SCell activation/deactivation should not be impacted by SRS antenna port switching. Proposal 6: If it is necessary to check, RAN4 can make requirements and test for the collision cases of SRS and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH according to the rules defined in clause 6.2.1 in RAN1specification[3]. Proposal 7: Option 2 can be agreed, i.e. txSwitchImpactToRx indicates the SRS antenna port switching impact to DL only, and txSwitchWithAnotherBand indicates the SRS antenna port switching impact to UL only. Proposal 8: The interruption requirements are defined only based on IEs of txSwitchImpactToRx or txSwitchWithAnotherBand indication. It is no need to clarify the relationship of the IEs with per-FR MG capability in RAN4 specification. Proposal 9: It is preferred the interruption requirement can differentiate between sync and async cases to reduce the unnecessary loss of sync cases. Proposal 10: No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case. Proposal 11: The interruption requirements will be defined based on slot level, i.e. no requirement for the case of interruption on flexible slot. Proposal 12: It is not necessary to further discuss on components of making interruption if the interruption requirements are defined based on slot level. Proposal 13: It may be reasonable that the interruption slot number is calculated based on the time of 6 SRS symbols and two 15µs of switching transient period. Proposal 14: The interruption requirements are proposed defined as table 1 and table 2. Table 1. Interruption (slot number) requirement for Synchronized network Victim cell SCS; Aggressor cell SCS, number SRS symbols on other antenna port. 15kHz; 30kHz; 60kHz. 1; 2; 4. 15kHz or 30kHz; 1; 1; 1; 1; 1. 60kHz; 1; 1; 2; 1; 1. Table 2. Interruption (slot number) requirement for asynchronized network Victim cell SCS; Aggressor cell SCS, number SRS symbols on other antenna port. 15kHz; 30kHz; 60kHz. 1; 2; 4. 15kHz or 30kHz; 2; 2; 2; 2; 2. 60kHz; 2; 2; 3; 2; 2. |
| R4-2111927 | CATT | Draft CR R4-2111927 will not be treated |
| R4-2112124 | Apple | Proposal 1: RAN4 to agree that one OFDM symbol before and after the SRS antenna port switching shall be introduced as scheduling restriction for FR1, that is, UE has scheduling restriction to not transmit PUCCH/PUSCH/SRS or not receive SSB/PDCCH/PDSCH/TRS/CSI-RS for CQI on 1 data symbol before SRS transmission and 1 data symbol after SRS transmission. Proposal 2: RAN4 to define a generic requirement regardless of SRS resource configuration. Proposal 3: In NR-SA, NR-DC, EN-DC and NE-DC, no impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized - The above-mentioned NR measurement requirements includes serving/neighboring cell L3 measurement requirements, L1-RSRP/L1-SINR and RLM/BFD/CBD measurement requirements Proposal 4: RAN4 to clarify that requirement for handover, BWP switching, SCell activation/deactivation applies when no SRS antenna port switching occurs during those activities. Proposal 5: Scheduling of SRS antenna switching should avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report. If collision happens, it is considered as an error case and no UE requirement is imposed. Proposal 6: SRS antenna switching interruptions on both DL and UL applies to the band combinations signaled in txSwitchImpactToRx or txSwitchWithAnotherBand. Proposal 7: No need to differentiate the requirement for the UE with or without capability of per-FR gap for SRS antenna port switching in RAN4. But the requirement applicability needs to be clarified that “for interruption caused by SRS antenna port switching, the victim CC would be based on indication of txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability”. Proposal 8: No need to differentiate interruption requirements between sync and async cases. Interruption requirement is based on the async case for the minimum requirement. Proposal 9: No need to differentiate txSwitchImpactToRx applicability for intra-band contiguous CA and intra-band non-contiguous CA case. Proposal 10: Interruption requirement of SRS antenna port switching shall be defined based on slot level for NR victim CC and based on subframe level for LTE victim CC respectively. Proposal 11: The components within interruption time of SRS antenna port switching in FR1 include: - 6 symbols, and - Antenna switching time before and after SRS transmission occasion. Proposal 12: Interruption time is specified based on 2 antenna switching time (2*15us) and 6 symbol time. Proposal 13: the interruption requirement of SRS antenna port switching is summarized as: Victim CC SCS(kHz); Aggressor CC SCS (kHz). 15; 30; 60. 15 (NR or LTE); 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3. Unit of interruption requirement is slot for NR and subframe for LTE. |
| R4-2112177 | vivo | Observation 1 The guard period between SRS and PUSCH/PUCCH is missing in RAN1 spec. Observation 2 Based on FR1 RF spec, the transient period between SRS and PUSCH/PUCCH is actually a window that allows EVM relaxation, and network can still schedule UE without 1-symbol gaps if it can endure EVM degradation. Observation 3 For the case where antenna port is switched, guard period between SRS is specified, but the guard period between SRS and PUSCH/PUCCH is not specified either in RAN1 spec or FR1 RF spec. Observation 4 From network perspective, if no scheduling restriction is defined, 1-symbol gap may not be always guaranteed even if the transient period is clearly defined in RF specs. Proposal 1 For FR1, specify scheduling restriction before and after the symbol(s) for SRS transmission, at least when the antenna port is switched, for the cell with SRS antenna port switching in R17. Proposal 2 RAN4 further discuss whether the scheduling restriction on the same carrier is specified in TS 38.133 or in RAN1 specs via LS to RAN1. Proposal 3 If RAN4 concludes necessity of clarifying the position of the transient period, it should be the 1 symbol before and the 1 symbol after the symbol(s) used for SRS transmission. Proposal 4 For SRS antenna switching, no impact to requirements for L1-RSRP/L1-SINR and RLM/BFD/CBD measurements in NR SA. Proposal 5 Define dropping rules for NR SRS antenna switching, at least for the case it collides with other NR RRM measurements. In this case the interruption requirements does not apply. Proposal 6 For SRS antenna switching, no impact to the requirement for handover, BWP switching, SCell activation/deactivation in NR CA/DC scenario. Proposal 7 Send LS to RAN1 to check the prioritization rule for SRS antenna switching, especially for the case in CA/DC operation. Proposal 8 The interruption requirement is preferred to be defined without differentiating sync and async case, at least in R17. Proposal 9 If UE indicates that in the corresponding band the Rx or Tx is impacted by antenna port switching, then only the corresponding Rx or Tx in that band is allowed to be interrupted when UE is configured to switch SRS antenna port. Proposal 10 The interruption requirement is preferred to be defined based on slot level. Proposal 11 The mis-alignment in the UL frame boundary between anchor and victim cells needs to be further discussed. Proposal 12 For interruption requirements, the interruption time is preferred to include power adjustment period, antenna switching time and SRS transmission time. Proposal 13 RAN4 can consider to define interruption requirements for a limited set of SRS configuration in R17. Proposal 14 If needed, RAN4 can study whether and how network can obtain the interrupted symbol information, when SRS antenna port switching is performed in another band. |
| R4-2112256 | Qualcomm, Inc. | Proposal 1: Do not specify the scheduling restriction on the transient periods since the interruption requirement already covers the transient period. Proposal 2: SRS antenna switching interruptions on both DL and UL applies to the band combinations signaled in txSwitchImpactToRx or txSwitchWithAnotherBand. Proposal 3: No impact on NR RLM/BFD/CBD measurement requirements due to NR SRS antenna switching, as NR RLM/BFD/CBD measurements are always prioritized. Proposal 4: RRM measurement and SRS antenna switching prioritization in NR-DC follows NR-SA. Proposal: Do not specify the requirement for SRS antenna switching colliding with handover, BWP switching, SCell activation/deactivation. Proposal 5: Do not specify the requirement for SRS antenna switching colliding with handover, BWP switching, SCell activation/deactivation. Proposal 6: Scheduling of SRS antenna switching should avoid collision to all reference signals, including CSI-IM, except DMRS, and UCI containing CSF report. If the collision happens, it is considered as an error case and no UE requirement is imposed. Proposal 7: Interruption time is specified in the unit of a slot. Proposal 8: Interruption requirement is the same for synchronized and asynchronized carriers. Observation 1: If gNB can’t utilize the SRS symbol transmission time between transient or guard periods, the SRS symbol transmission time is part of the interruption duration. Proposal 9: Interruption time is the summation of the 2 transient period and 6 symbol time. Proposal 10: SRS antenna switch interruption is specified as Table 2-2 for NR SA. In EN-DC, interruption on LTE carrier is the same as victim SCS = 15kHz case in NR SA. Interruption Length (slots). Victim SCS (kHz); 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3. |
| R4-2112414 | Xiaomi | Proposal 1: The SRS antenna switching interruptions on DL and UL should be defined based on the indication of txSwitchImpactToRx and txSwitchWithAnotherBand respectively. Proposal 2: Interruption requirements of SRS antenna port switching would have no relationship to per-UE or per-FR gap capability. Proposal 3: RAN4 to define unified interruption requirements for synchronous and asynchronous cases. Proposal 4: No need to further clarify for intra-band case. Proposal 5: RAN4 to define the interruption requirement based on slot level. Proposal 6: The interruption time of SRS antenna port switching would include the SRS antenna port switching time and the SRS transmission time. Proposal 7: The interruption time would be specified based on the antenna switching to and switching back time (2*15us) and the SRS transmission time of 6 OFDM symbols of the aggressive CC. |
| R4-2112512 | CMCC | Observation 1: RF has defined the transient period between PUSCH/PUCCH and SRS, in which there is no data transmission/reception. Proposal 1: in addition to the transient period, no need to define further scheduling restriction before and after SRS transmission. Proposal 2: the interruption time includes transient period before and after SRS transmission and SRS transmission time. |
| R4-2112522 | MediaTek inc. | Proposal 1: A carrier is considered as a victim carrier due to SRS antenna carrier switch if it is indicated by any one of the reported capabilities in txSwitchImpactToRx and txSwitchWithAnotherBand. Proposal 2: For the txSwitchImpactToRx, not to further differentiate the requirements for contiguous and non-contiguous CA cases. Proposal 3: For the txSwitchImpactToRx and txSwitchWithAnotherBand, interruption requirement of SRS antenna port switching will not depend on per-UE or per-FR gap capability. Proposal 4: Define the interruption requirement for SRS antenna port switching based on slot level. Observation 1: The max number of symbols for SRS in one slot is 6, including SRS resource(s) and guard period for switching among SRS ports. Proposal 5: The SRS antenna switching time is 15us. Proposal 6: The SRS antenna switching interruption time should be sum of - SRS Transmission time (up to 6 symbols). - 2 * 15us Proposal 7: RAN4 to define one single requirement to cover the synchronous and asynchronous scenarios with or without UL TA. Proposal 8: The SRS antenna switching interruption requirement should be specified as follows. Table 2. Interruption length (slots) due to SRS antenna switch Victim cell SCS(KHz); Aggressor Cell SCS (KHz). 15; 30; 60; 120. 15; 2; 2; 2; 2. 30; 2; 2; 2; 2. 60; 3; 2; 2; 2. 120; 5; 3; 3; 2. Proposal 9: For EN-DC, UE is not required to perform SRS antenna port switching when SRS resource and the L1 measurement, e.g., L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the SCG. Proposal 10: For NR-SA, UE is not required to perform SRS antenna port switching when SRS resource and the L1 measurement, e.g., L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol. Proposal 11: For NE-DC, UE is not required to perform SRS antenna port switching when SRS resource and the L1 measurement, e.g., L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the MCG. Proposal 12: For NR-DC, UE is not required to perform SRS antenna port switching when SRS resource and the L1 measurement, e.g., L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the same CG. Proposal 13: For SRS antenna port switching, define the interruption requirement for the case not covered by TS 38.214, e.g., add a note “if the case has no priority rule defined in TS 38.214, UE is allowed to cause the interruption on the other CC(s)”. Proposal 14: For SRS antenna port switching (FR1 only), RAN4 needs to further study the corresponding UE behavior when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol: Option1: Two different SRS resources having the same time domain behavior to be scheduled on the same OFDM symbol are not expected. Option2: Up to UE implementation. |
| R4-2112675 | LG Electronics Inc. | - Observation 1: After transmitting the last SRS resource in the slot, the switching back to the main Tx antenna is performed. Additional time for switching back to the main Tx antenna is required. - Observation 2: In case the SRS resource is configured in the last symbol of the slot and the next slot is downlink, the Tx path will change to Rx path within NTA_offset time, so there is no need to switch back to the main Tx antenna. Based on observations, we propose - Proposal 1: Introduce scheduling restriction for one OFDM symbol before and after SRS antenna port switching for FR1, and no scheduling restriction after SRS antenna port switching is needed in case of the SRS resource is configured in the last symbol of the slot and the next slot is downlink. - Proposal 2: The same interruption length for both synchronous and asynchronous cases could be defined, and the interruption length should be reduced by one slot when the slot after SRS antenna post switching is downlink in synchronous case. - Proposal 3: Slot based interruption requirement could be used in synchronous and asynchronous case, and only UL symbols based interruption should be defined when SRS antenna port switching is configured in the flexible slot in synchronous case. - Proposal 4: The interruption time is - Antenna port switching time (15usec) before the first SRS transmission - Time from the first SRS transmission to the last SRS transmission - Antenna port switching back time (15usec) after the last SRS transmission - It should be excluded when the SRS resource is configured in the last symbol of a slot and the next slot is downlink - Proposal 5: Interruption requirements for SRS antenna port switching are shown in Table 1. Table 1 Proposed interruption requirements Victim cell SCS [kHz]; Interruption length [slot]. Aggressor cell SCS [kHz]. 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. Note 1: The interruption length should be reduced by one slot when the slot after SRS antenna post switching is downlink in synchronous case. Note 2: If SRS antenna port switching is configured in the flexible slot in synchronous case, the interruption applies to only uplink symbols in that slot.. |
| R4-2112877 | Nokia, Nokia Shanghai Bell | Observation #1: For the scenario in Fig.1 (a), the guard period defined in RAN1 does not apply and shall not be taken into account when defining the requirements. Observation #2: For the scenario in Fig.1 (b), RAN4 requirements shall be defined taking into account the guard period in-between the SRS resources. Observation #3: For the scenario in Fig.1 (c), RAN4 requirements shall be not defined before the guard period gets clarified in RAN1. Proposal #1: RAN4 shall define the requirements for the following scenarios in Rel17 where - The SRS resources of a set are transmitted NOT in the same slot, or - The SRS resources of a set are transmitted in the same slot with consecutive SRS transmission Proposal #2: RAN4 do not define the requirements if the SRS resources of a set are transmitted in the same slot with non-consecutive SRS transmission, before the guard period in this scenario gets clarified in RAN1. Proposal #3: The DL and UL interruption shall be defined separately on different groups of victim cells based on the indication of txSwitchImpactToRx and txSwitchWithAnotherBand. Observation #4: One OFDM symbol is allowed to be blanked due to transient period only in case of 60kHz SCS. Proposal #4: The transient period before and after the SRS resource will cause UL interruption of 1 OFDM symbol at 60kHz. At 15kHz or 30kHz SCS, RAN4 should discuss if and how to capture the transient periods from RF session in the RRM specifications. Proposal #5: The SRS transmission will cause UL interruption on the other carriers if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE. Proposal #6: When the SRS resources of a set are not configured in a slot, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE, where the uplink interruption comprises at least the SRS transmission period. Proposal #7: When the SRS resources of a set are configured in a slot with consecutive SRS transmission, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE, where the uplink interruption comprises at least the SRS transmission periods and the guard period in-between. Proposal #8: The position of the transient period needs to be clarified before defining the interruption and/or the scheduling restriction in RRM spec. Proposal #9: Add one note indicating the DL may be affected due to SRS antenna switching if txSwitchImpactToRx is configured. |
| R4-2113138 | Intel Corporation | Proposal 1: SRS antenna switching interruptions on both DL and UL applies to the band combinations signaled in txSwitchImpactToRx or txSwitchWithAnotherBand. Proposal 2: Since txSwitchImpactToRx can’t differentiate the intra-contiguous CA and non-contiguous CA case, there are two solutions: Option 1: Send LS to RAN2 to modify the signaling. Option 2: Solve the issue in RAN4 by updating the applicability. Proposal 3: The interruption requirement don’t need to differentiate between sync and async cases. Proposal 4: The interruption requirement is defined based on slot level. |
| R4-2113275 | OPPO | Proposal 1: The scheduling restriction shall be defined before and after SRS transmission considering the 15 us SRS antenna switching time. Proposal 2: For NR-DC, no impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized. Proposal 3: Consider the interruption requirement based on the band group/combination regardless of cross CGs if txSwitchImpactToRx is configured. Proposal 4: It can be maintained by network scheduling to avoid the case when SRS antenna port switching is colliding with L1 reference signals, e.g., for L1-RSRP/L1-SINR and RLM/BFD/CBD measurement, CSI-IM except DMRS and UCI containing CSF report. Proposal 5: The requirements for handover, BWP switching, SCell activation/deactivation should not be impacted by SRS antenna port switching. Proposal 6: SRS antenna switching interruptions on both DL and UL applies to the band combinations signaled in txSwitchImpactToRx or txSwitchWithAnotherBand. Proposal 7: No need to differentiate the requirement for the UE with or without capability of per-FR gap for SRS antenna port switching in RAN4. Proposal 8: The requirement applicability needs to be clarified that “for interruption caused by SRS antenna port switching, the victim CC would be based on indication of txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability”. Proposal 9: Define one single interruption requirement, which can be based on the async case for the minimum requirement. Proposal 10: Interruption time is specified based on 2 antenna switching time (2*15us) and 6 symbol time. |
| R4-2114139 | Huawei, Hisilicon | Proposal 1: The impact of SRS AS on aggressor CC shall be considered based on the SRS AS resource instead of the fixed 6 OFDM symbols. Observation 1: There is no need to restrict scheduling on symbols before and after SRS AS resource in addition to RF requirements where consecutive scheduling is already considered. Proposal 2: No need to define scheduling restriction on SRS AS carrier on symbols before and after SRS AS resource. Performance degradation on these symbols could be expected. Proposal 3: For NR-SA, NR measurement are always prioritized including serving/neighbouring cell L3 measurement requirements, L1-RSRP/L1-SINR and RLM/BFD/CBD measurement. Proposal 4: For NR-DC, NR measurement shall always be prioritized within the same CG. Observation 2: There are no prioritizing rules for the existing requirements and procedures considering all the combinations. Proposal 5: RAN4 not to define prioritizing rules of SRS antenna switching and other specific RRM requirements except for the NR measurements. Observation 3: There is no precedent on defining the rules whether the interruptions could be applied when certain transmission will be interrupted, and it is over complicated from both UE and NW side. Proposal 6: Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement. Observation 4: The definition of txSwitchImpactToRx and txSwitchWithAnotherBand should not be modified in RAN4 only from test perspective. Proposal 7: txSwitchImpactToRx indicates the SRS antenna port switching impact to DL only, and txSwitchWithAnotherBand indicates the SRS antenna port switching impact to UL only. Proposal 8: The interruption requirements are defined in slot level based on 2*15us and 6 symbol time. Table I Interruption requirement in number of slots Aggressor CC SCS(kHz). Victim CC SCS (kHz); 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 2. Proposal 9: Define the interruption requirements as Table I above. |
| R4-2114174 | Ericsson | Proposal 1: Scheduling restrictions are to be introduced for the case where there is no gap between PUSCH and SRS. Proposal 2: RAN4 to further discuss whether collisions between PUCCH carrying L1 measurement report and aperiodic SRS transmission needs to be handled within this work item, or whether it is already covered in the specification. The justification being that the RAN1 riule on how to handle conflicts is based on the SRS transmission as such, rather than on whether there is SRS antenna port switching. Proposal 3: There shall be no impact from SRS antenna port switching within a CG, as per previous agreement on prioritizing NR measurements over NR SRS antenna switching. Add a note indicating the DL in one CG may be affected by SRS antenna switching in another CG when txSwitchImpactToRx has been reported by UE. Proposal 4: Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels is to be handled by RAN1. If anything is unclear, RAN4 shall send LS to RAN1 and ask for clarification. Proposal 5: Interruption requirement for SRS antenna port switching shall not depend on per-UE or per-FR gap capability. Hence no need to further clarify UE reported capability. Proposal 6: Define separate interruption requirements for SRS antenna port switching in synchronous and asynchronous scenarios, respectively. Proposal 7: Interruption requirements for SRS antenna port switching shall be defined in OFDM symbol granularity. Proposal 8: The components within the interruption requirement shall include SRS transmission time for the configured number of symbols, as well as SRS antenna switching time before and after the SRS transmission. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Scope of SRS antenna switching requirement

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1-1: whether scheduling restriction requirement would be defined in RRM for SRS antenna port switching

- Proposals

  - Option 1 (CATT, QC, CMCC): Don't define the scheduling restriction on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols.

    - Option 1a (Huawei):

      - The impact of SRS AS on aggressor CC shall be considered based on the SRS AS resource instead of the fixed 6 OFDM symbols.

      - No need to define scheduling restriction on SRS AS carrier on symbols before and after SRS AS resource. Performance degradation on these symbols could be expected.

  - Option 2: Yes

    - Option 2a (Apple): RAN4 to agree that one OFDM symbol before and after the SRS antenna port switching shall be introduced as scheduling restriction for FR1, that is, UE has scheduling restriction to not transmit PUCCH/PUSCH/SRS or not receive SSB/PDCCH/PDSCH/TRS/CSI-RS for CQI on 1 data symbol before SRS transmission and 1 data symbol after SRS transmission.

    - Option 2b (vivo):

      - For FR1, specify scheduling restriction before and after the symbol(s) for SRS transmission, at least when the antenna port is switched, for the cell with SRS antenna port switching in R17.

      - RAN4 further discuss whether the scheduling restriction on the same carrier is specified in TS 38.133 or in RAN1 specs via LS to RAN1.

      - If RAN4 concludes necessity of clarifying the position of the transient period, it should be the 1 symbol before and the 1 symbol after the symbol(s) used for SRS transmission.

    - Option 2c (LG): Introduce scheduling restriction for one OFDM symbol before and after SRS antenna port switching for FR1, and no scheduling restriction after SRS antenna port switching is needed in case of the SRS resource is configured in the last symbol of the slot and the next slot is downlink.

    - Option 2d (OPPO): The scheduling restriction shall be defined before and after SRS transmission considering the 15 us SRS antenna switching time.

    - Option 2e (Ericsson): Scheduling restrictions are to be introduced for the case where there is no gap between PUSCH and SRS.

- Agreements in GTW(Aug 17):

  - Do not define the scheduling restriction on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols in Rel-17

    - Performance degradation on these symbols can be expected

    - FFS how to capture this in TS 38.133

- Option 1 (QC, CATT, vivo): Capture the GTW agreement in WF without changing specification.

- Option 2 (OPPO, Nokia): FFS how to capture this in TS 38.133

- Recommended WF

  - Based on GTW agreement, please companies discuss above option 1 and option 2. The proponents of option 2 may propose how to capture GTW agreement into TS 38.133. Continue the discussion in 2nd round. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | Support Option 2a. RAN1 has defined the guard period (1 OFDM symbol) due to the port switching. Following similar logic, RAN4 should define scheduling restriction before and after SRS transmission because port switching may occur during that transient period (the time before and after SRS transmission). |
| Apple | We propose option 2a to capture that scheduling restriction in TS38.133, but we could compromise to option 1 in case companies also think RF spec has already defined such transient period. |
| LG | Considering antenna switching time before and after SRS transmission, 1 symbol scheduling restriction should be considered. However, if the SRS resource is configured in the last symbol of a slot and the next slot is DL, no scheduling restriction after the SRS transmission should apply since Tx-Rx switching is performed after the SRS transmission. So, option 2c is preferred. |
| CMCC | We support Option 1. Our consideration is that RF has defined the transient period due to SRS antenna switching, which includes the transient period between PUSCH/PUCCH and SRS. No need to specify scheduling restriction in RRM. It is up to network scheduling. If UE is scheduled in the transient period, performance degradation can be expected. |
| Xiaomi | Support Option2a. |
| Huawei | Support option 1a. The consecutive PUSCH/PUCCH and SRS are already considered in transient period requirements. Prefer not to further restrict scheduling on these symbols. |
| OPPO | As agreed in GTW，RAN4 RRM do not to define scheduling restriction. And how to capture the performance degradation can be further discussed, especially considering the case when transient period will be taken in the blanked SRS symbol, i.e., no scheduling is allowed in these exact symbols. Figure 6.3.3.6-4: Consecutive SRS time mask for the case when power change is required and when 120kHz SCS is used in FR2 |

| Intel | Fine with option 1a. further discuss how to specify the performance degradation. |
| --- | --- |

| ZTE | Support option 1. |
| --- | --- |
| QC | As we commented on GTW, no requirement on EVM already implies performance degradation. Therefore, we can capture the GTW agreement on WF without changing specification. |
| Nokia | With the agreements on GTW, no scheduling restriction will be defined due to transient period. Considering the performance degradation, we may add a note like some performance degradation may be expected on these symbols. The texts can be further discussed. |
| vivo | As commented in GTW, we can compromise to no scheduling restriction defined in R17 RRM session. But we do not think leaving performance degradation is a good approach. This issue still needs to be solved. Since 38.101-1 is clear, we do not think further capturing the performance degradation in 38.133 is necessary, but open to discuss. |
| CATT | Follow the GTW agreement. Agree with QC to capture the agreement in WF without spec change. |

Issue 1-1-2: RAN4 requirement scope with different SRS resource configuration

- Proposals

  - Option 1 (CATT):

    - Only defining the requirements for the scenario that the SRS resources of a set are transmitted in the same slot with consecutive SRS transmission.

  - Option 2 (Nokia, Ericsson):

    - RAN4 shall define the requirements for the following scenarios in Rel17 where

      - The SRS resources of a set are transmitted NOT in the same slot, or

      - The SRS resources of a set are transmitted in the same slot with consecutive SRS transmission

    - RAN4 do not define the requirements if the SRS resources of a set are transmitted in the same slot with non-consecutive SRS transmission, before the guard period in this scenario gets clarified in RAN1.

  - Option 3 (Apple, Xiaomi, Huawei, QC, vivo, CATT):

    - RAN4 to define a generic requirement regardless of SRS resource configuration.

      - Option 3a: define interruption requirements for a limited set of SRS configurations.

- Recommended WF

  - Continue the discussion in 2nd round. 6 companies supported option 3, but other companies had different views, and 3 companies suggested to wait conclusions from issue 1-4-1. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | Pending on the conclusion in Issue 1-4-1 (slot level v.s. symbol level) |
| Apple | Option 3. We prefer to define a generic requirement based on the worst case regardless of SRS resource configuration. For instance, we use 6 symbols as the SRS transmission occasion for the worst case, and also for multiple slots where SRS antenna port switching is used, the multiple interruptions would be expected based on those slots who has SRS antenna port switching, but the baseline requirement for single interruption shall be identical. We could follow same method as in SRS carrier switching case in R16. |
| LG | Depending on decision of symbol or slot based requirements, limited scenario for interruption requirements could be discussed. However, if slot based interruption requirement is considered, generic requirements could apply regardless of SRS resource configuration |
| Xiaomi | Support Option 3. |
| Huawei | For the interruption requirements, we support option 3. |
| LG | Depending on decision of symbol or slot based requirements, limited scenario for interruption requirements could be discussed. However, if slot based interruption requirement is considered, generic requirements could apply regardless of SRS resource configuration |
| Intel | Suggest to focus about whether slot lever or symbol level is defined. The granularity will have impact on this issue. |
| Ericsson | Our preference is Option 2. No case is ruled out, but prefer waiting for clarification from RAN1 regarding guard period for non-consecutive SRS transmission. |
| QC | Support option 3. |
| Nokia | Option 2. As the guard period in RAN1 is specified for limited scenarios, we need make it clear where the requirements to be defined will apply. The scenarios need to be aligned in the beginning to avoid misunderstanding. And we don’t think there is dependency on slot-level or symbol-level discussion. The scenarios need to be aligned before identifying the interruption; then the interruption details can be discussed. The final step may be discussing the possibility to round to slot-level. |
| vivo | Support option 3. However, we still prefer to define interruption requirements for a limited set of SRS configurations. This can be clarified in the requirement applicability. |
| CATT | Support option 1. But we can compromise to option 3 to define a generic requirement. |

### Sub-topic 1-2: Impact of SRS antenna port switching to other requirements

Sub-topic description:

Open issues and candidate options before e-meeting:

Last meeting agreement (R4-2108343):

- Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA - Agreement: No impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized. - The above-mentioned NR measurement requirements includes serving/neighboring cell L3 measurement requirements - FFS on L1-RSRP/L1-SINR and RLM/BFD/CBD measurement requirements - Issue 1-2-2: Impact of SRS antenna port switching to RRM requirements in EN-DC or NE-DC - Agreement - In EN-DC and NE-DC operation, - NR SRS antenna switching colliding with E-UTRA measurement - Interruptions on E-UTRA measurement in the interrupted carrier group are allowed due to NR SRS antenna switching, but NOT allowed due to NR SRS antenna switching for the carriers not in the interrupted carrier group. - Additional delay can be expected on E-UTRA measurement in the interrupted carrier group when UE is configured to perform NR SRS antenna switching. - NR SRS antenna switching is allowed to be dropped when colliding with E-UTRA measurement in the interrupted carrier group. - Issue 1-2-5: Impact of SRS antenna port switching to timing requirements - Agreement: - New option 4: No need to consider impact to timing requirements (non-positioning related) for SRS antenna switching. - Issue 1-2-6: Impact of SRS antenna port switching to positioning related requirements - Agreement: The impact of SRS antenna switching on positioning related measurement will not be discussed in this Rel-17 FeRRM.

Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA

- Proposals

  - Option 1 (Apple, vivo, QC, MTK, HW): No impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized.

    - Option 1a: The above-mentioned NR measurement requirements including serving/neighboringcell L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD measurement

    - Option  1b (Nokia): The above-mentioned NR measurement requirements including serving/ neighbouring cell L3 measurement and RLM/BFD/CBD measurement

    - Option 1c (HW): The above-mentioned NR measurement requirements including serving/ neighbouring cell L3 measurement and RLM/BFD/CBD measurement. FFS on L1-RSRP/L1-SINR.

    - Option 1d (moderator added):

      - For NR-SA, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol.

      - For NR-SA, UE is not required to perform SRS antenna port switching when P/SP SRS resource and the AP L1-RSRP/L1-SINR measurement, are scheduled in the same OFDM symbol.

  - Option 2 (CATT): The aperiodic SRS antenna port switching can impact on L1-RSRP/L1-SINR and RLM/BFD/CBD measurement requirements if the IE txSwitchImpactToRx include the working band for FDD.

  - Option 3 (Ericsson): RAN4 to further discuss whether collisions between PUCCH carrying L1 measurement report and aperiodic SRS transmission needs to be handled within this work item, or whether it is already covered in the specification. The justification being that the RAN1 rule on how to handle conflicts is based on the SRS transmission as such, rather than on whether there is SRS antenna port switching.

  - Option 4 (OPPO): It can be maintained by network scheduling to avoid the case when SRS antenna port switching is colliding with L1 reference signals, e.g., for L1-RSRP/L1-SINR and RLM/BFD/CBD measurement, CSI-IM except DMRS and UCI containing CSF report.

- Recommended WF

Moderator: In TS38.214 section 6.2.1.3, the prioritization for CSI reporting and SRS carrier switching is summarized as below,

| PUCCH/PUSCH for CSI (L1-RSRP/L1-SINR) reporting | SRS transmission (including interruption) | Prioritization |
| --- | --- | --- |
| P/SP CSI | AP SRS | AP SRS |
| AP CSI | AP SRS | AP SRS |
| AP CSI | P/SP SRS | AP CSI |
| P/SP CSI | P/SP SRS | P/SP SRS |

Please companies check the above prioritization and discuss:

- Whether the CSI reporting prioritization could be equivalent to CSI measurement prioritization?

- Whether SRS carrier switching prioritization could be reused for SRS antenna port switching prioritization?

If ‘yes’ to both above questions, only when AP CSI colliding with P/SP SRS, the AP CSI shall be prioritized, otherwise SRS shall be prioritized. This is the new added option 1d.

  - Continue the discussion in 2nd round. New options have been added under option 1. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | Support Option 1. Following the similar logic as SRS carrier switching. |
| Apple | Option 1. |
| LG | Support Option 1 |
| Xiaomi | Support Option1. |
| Huawei | Support option 1 and option 3 for L1 report and AP SRS, which should be FFS |
| OPPO | Option 1 is OK. Option 4 proposed by us seems aligned with option 1. |
| Intel | Fine with option 1. |
| ZTE | Support option 1. |
| Ericsson | Option 3. Our understanding is that conflict handling pertaining to SRS transmissions is already specified by RAN1. |
| QC | Support option 1. |
| Nokia | We agree with Option 1 apart from L1-RSRP/L1-SINR, so would propose a new sub-bullet: - Option 1x: The above-mentioned NR measurement requirements including serving/ neighbouring cell L3 measurement and RLM/BFD/CBD measurement We agree that RLM/BFD/CBD measurement can be prioritized over SRS switching. But for L1-RSRP, RAN1 specifies the priority handling between CSI report carrying L1-RSRP and SRS. We believe SRS switching also follows the priority principle for general SRS transmission hence no absolute priority for L1-RSRP measurements. |
| vivo | Support option 1 Regarding option 3, we have some discussion in our paper. Firstly we think SRS AS are normally periodic or semi-persistent. Secondly, gNB should be able to schedule aperiodic SRS away from PUSCH/PUCCH carrying CSF. Therefore, we think option 1 can be acceptable. And some applicability rule can be considered for the case of aperiodic SRS case. |
| CATT | Option 2. We think RAN1 has defined that the aperiodic SRS transmission can impact on L1-RSRP/L1-SINR and RLM/BFD/CBD measurement requirements. And in our understanding, the SRS transmission should include the transmission with antenna port switching and without antenna port switching. |
| Huawei2 | For L1-RSRP and L1-SINR, RAN4 should further discuss. Option 1 seems contradict with RAN1 spec now. |

Issue 1-2-2: Impact of SRS antenna port switching to RRM requirements in EN-DC or NE-DC

- Proposals

  - Option 1 (Apple, LG, Xiaomi, OPPO, QC, vivo, CATT): For EN-DC and NE-DC, no impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized.

    - The above-mentioned NR measurement requirements including serving/neighboring cell L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD measurement

  - Option 2 (MTK, Apple , LG, Xiaomi, HW, OPPO, QC, vivo):

    - For EN-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the SCG.

    - For NE-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the MCG.

  - Option 3 (OPPO): It can be maintained by network scheduling to avoid the case when SRS antenna port switching is colliding with L1 reference signals, e.g., for L1-RSRP/L1-SINR and RLM/BFD/CBD measurement, CSI-IM except DMRS and UCI containing CSF report.

  - Option 4 (Nokia, Ericsson, HW):

    - For EN-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the SCG.

    - For NE-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the MCG.

    - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1)

- Recommended WF

  - Continue the discussion in 2nd round. Companies to check whether option 4 is agreeable or not. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | Support Option2. Following the similar logic as SRS carrier switching, the clarification “in the SCG” and “in the MCG” should be defined in specification clearly. Besides, we would like to clarify our proposal as follows to make it clearer. - For EN-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the SCG. - For NE-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the MCG. |
| Apple | Option 1. Option 1 and Option 2 are similar, but we think L3 NR measurement shall also be prioritized over NR SRS antenna port switching in option 2. |
| LG | We have similar view with Apple. |
| Xiaomi | Both Option1 and Option2 are fine with us. |
| Huawei | Support option 1, which is similar as the revised proposal in MTK’s comments. |
| OPPO | Support NR L3 and L1 measurements to be prioritized over NR SRS antenna port switching |
| Intel | With the clarification of MTK. We think that both option 1 and updated option 2 are similar. The wording of updated Option 2 is more aligned with the spec. |
| QC | Support option 1 and believe option 1 and 2 are the same. |
| Nokia | We think the same principle for NR-SA can be applied to EN-DC or NE-DC, but only within the MCG or SCG where SRS switching occurs. Hence as commented to Issue 1-2-1, we are fine with Option 1 by removing L1-RSRP/L1-SINR and also option 2 by adding L3 measurements. The new proposal from Mediatek is good by combining both option 1 and 2, we are fine with it by removing L1-RSRP/L1-SINR: Option x: - For EN-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the SCG. - For NE-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the MCG. |
| Vivo | Support option 1 and 2. |
| CATT | Support option 1. |
| Huawei2 | Same comments as issue 1-2-1 about L1-RSRP and L1-SINR |
| Ericsson2 | Support Option x originating from Nokia and MTK. |

Issue 1-2-3: Impact of SRS antenna port switching to RRM requirements in NR-DC

- Proposals

  - Option 1 (Apple, OPPO, CATT, QC, LG): No impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized.

    - Option 1a (Apple, QC): the above-mentioned NR measurement requirements in option 1 including serving/neighboring cell L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD measurement

    - Option 1b (OPPO): It can be maintained by network scheduling to avoid the case when SRS antenna port switching is colliding with L1 reference signals, e.g., for L1-RSRP/L1-SINR and RLM/BFD/CBD measurement, CSI-IM except DMRS and UCI containing CSF report.

  - Option 2 (): For NR-DC, NR measurement shall always be prioritized within the same CG.

    - Option 2a (MTK, Apple, Xiaomi, OPPO, Intel,): UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the same CG.

    - Option 2b(Ericsson): Add a note indicating the DL in one CG may be affected by SRS antenna switching in another CG when txSwitchImpactToRx has been reported by UE.

  - Option 3 (vivo): Define dropping rules for NR SRS antenna switching, at least for the case when it collides with other NR RRM measurements. In this case the interruption requirements does not apply.

  - Option 4 (Nokia, HW, Ericsson):

    - For NR-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the same CG.

    - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1)

- Recommended WF

  - Continue the discussion in 2nd round. Companies to check whether option 4 is agreeable or not. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | Support Option 2a. Following the similar logic as SRS carrier switching, the clarification “in the same CG” should be defined in specification clearly. Besides, we would like to clarify our proposal as follows to make it clearer. - Option 2a (MTK): UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the same CG. |
| Apple | Prefer Option 1a as a generic rule, but we could compromised to option 2a which is further clarified by MTK if majority companies agree on that as well. |
| LG | Since NR measurements are prioritized, we support Option 1. |
| Xiaomi | Taking the requirements for SRS carrier switching as baseline, we prefer Option 2a. |
| Huawei | Support option 2 following the same principle of SRS carrier switching. |
| OPPO | The option 2a updated by MTK is fine to us. |
| Intel | We are fine with updated option 2a provided by MTK. |
| Nokia | We think the same principle for NR-SA can be applied to NR-DC but within the MCG/SCG only where SRS switching occurs. Hence as commented to Issue 1-2-1, we are fine with Option 1a by removing L1-RSRP/L1-SINR and also option 2a by adding L3 measurements. The new proposal from Mediatek is good by combining both option 1 and 2, we are fine with it by removing L1-RSRP/L1-SINR: Option x: UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the same CG. |
| Vivo | Support option 1 and 1a, but fine to compromise to option 2a. To clarify, the intention of option 3 is the same as 1 or 1a. Not sure whether the principle of SRS carrier switching can be re-used here directly. Since the actual interrupted band is indicated in “txSwitchImpactToRx or txSwitchWithAnotherBand”, the interruption may also happen in the NR-DC case. This is not the same as SRS carrier switching. But according to RAN-P guidance, there will be no FR1+FR1 NR-DC discussion in R17, we are also fine with option 2a. |
| CATT | Fine with option 2a to clarify in this same CG. |
| Huawei2 | Similar views with Nokia. L1-RSRP/SINR should be further discussed to avoid contradiction with RAN1 spec. |
| Ericsson2 | We are fine with Option x originating from Nokia and MTK. |

Issue 1-2-4: Impact of SRS antenna port switching to other specific RRM requirements

- Proposals

  - Option 1 (CATT, vivo, OPPO, MTK, ZTE, vivo): The requirement for handover, BWP switching, SCell activation/deactivation should not be impacted by SRS antenna port switching.

  - Option 2 (Apple, QC, MTK, LG, Xiaomi, OPPO, ZTE, vivo): RAN4 to clarify that legacy requirement for handover, BWP switching, SCell activation/deactivation only applies when no SRS antenna port switching occurs during those activities.

  - Option 3 (Huawei, Xiaomi, Ericsson, Nokia): RAN4 not to define prioritizing rules of SRS antenna switching and other specific RRM requirements except for the NR measurements.

  - Option 4 (moderator added):

    - RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities.

    - RAN4 not to define prioritizing rules of SRS antenna switching and other specific RRM requirements except for the NR measurements.

- Recommended WF

  - Moderator: please proponents of option 2 and 3 check if option 4 is acceptable, because ‘not to define prioritization rule’ also means those existing specific RRM requirement applies when we don’t consider SRS antenna switching colliding with them.

  - Continue the discussion in 2nd round. Companies to check whether option 4 is agreeable or not. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | Both Option 1 and 2 are fine to us. In our understanding, for Option 1, the priority of HO, BWP switching and SCell activation/deactivation is higher than SRS antenna port switching (SRS ANPS), which means SRS resource(s) will not be transmitted if SRS resource(s) collide with HO, BWP switching and SCell activation/deactivation. Regarding Option 2, it only mentioned that the requirement is applied when not collided with HO, BWP switching and SCell activation/deactivation. |
| Apple | Option 2. It’s hard to say which one shall be prioritized since those UE behavior may not be directly related with specific NR measurement, e.g., BWP switching. So, we prefer to avoid mixing SRS antenna port switching with those UE behaviors in one requirement. |
| LG | Since there are no clear priority among the RRM requirement except for the NR measurement, Option 2 is slightly preferred. |
| Xiaomi | As the priority rule of SRS antenna switching and other specific RRM requirements are not clear, we prefer to Option 2. To make the requirements simple, Option3 is also fine with us. |
| Huawei | We support option 3. For option 1, we have following questions: Does it mean the priority of SRS AS is lower than HO, BWP switch and SCell activation but higher than all other procedures? And what is the priority rules of existing requirements (e.g. BWP switching and SCell activation)? |
| OPPO | Option 1 or 2 seems similar. Some clarification may be introduced as said in option 2. |
| ZTE | Fine with option 1 or option 2. |
| Ericsson | We support Option 3. |
| QC | Our understanding is that these procedures shouldn’t collide with SRS antenna switching, hence option 2 is our preference. |
| Nokia | We support Option 3. The collision between SRS switching and handover, BWP switching, SCell activation seems to be corner case scenarios. A reasonable network configuration would know how to handle it. There is no need to define the requirements for these cases. |
| vivo | Fine with option 1 or option 2. |
| CATT | Support option 1. We think the priority of HO, BWP switching and SCell activation/deactivation should be higher than SRS antenna port switching. |

Issue 1-2-5: Impact of SRS antenna port switching to CSF and other RS

- Proposals

  - Option 1 (CATT): If it is necessary to check, RAN4 can make requirements and test for the collision cases of SRS and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH according to the rules defined in clause 6.2.1 in RAN1specification.

  - Option 2 (Apple, QC, OPPO, vivo): Scheduling of SRS antenna switching should avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report. If collision happens, it is considered as an error case and no UE requirement is imposed.

  - Option 3 (vivo): Send LS to RAN1 to check the prioritization rule for SRS antenna switching, especially for the case in CA/DC operation.

    - Option 3a (Ericsson, Nokia): Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels is to be handled by RAN1. If anything is unclear, RAN4 shall send LS to RAN1 and ask for clarification.

  - Option 4 (Huawei, Ericsson, Nokia, CATT): Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement.

- Recommended WF

  - Moderator: since companies mentioned such prioritization was also defined in RAN1, please companies to check with your RAN1 colleagues and then discuss if option 3a is agreeable or not.

  - Continue the discussion in 2nd round. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | More discussion is needed. Could the companies supported Option2 explain why we need to preclude the DMRS and UCI containing CSF report? |
| Apple | Option 2. We think it means ‘Scheduling of SRS antenna switching should avoid collision to all reference signals including [CSI-IM except DMRS], and [UCI containing CSF report]’. DMRS is configured every slot and if we avoid configuring SRS colliding with DMRS, it almost has no room to schedule SRS transmission. |
| Huawei | Support option 4. We have question regarding option 2. Are we talking about when the RS and SRS AS are on the same carrier? If yes, we believe the situation has been defined in RAN1. |
| OPPO | Option 2. If we did not reach agreement in RAN4, an LS out to RAN1 is also good in this meeting. |
| Ericsson | Option 3a and Option 4. |
| QC | Our view on option 2 is that it includes collision with victim carriers. Since there is an interruption, periodic collision with the victim carriers still blocks UE from updating CSI for its demodulation and report to gNB. |
| Nokia | Option 3a and Option 4. We think this is up to RAN1 discussion and should not be discussed in RAN4. |
| vivo | Support option 3 but fine with option 2 if we do not consider FR1+FR1 NR-DC in R17. Also support to send the LS to RAN1. |
| CATT | We can support option 1and option 4. We think the collision case between SRS transmission and other signals should be defined in RAN1. And if the collision case is defined in RAN1, RAN4 can further consider to define requirement and test. |

### Sub-topic 1-3: Interruption requirement applicability

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-3-1: Interruption requirement applicability

- Proposals

  - Option 1 (Apple, QC, MTK, Intel, OPPO): SRS antenna switching interruptions on both DL and UL applies to the band combinations signaled in txSwitchImpactToRx or txSwitchWithAnotherBand.

  - Option 2 (CATT, vivo, Xiaomi, Nokia, HW, Apple, Intel, Ericsson): txSwitchImpactToRx indicates the SRS antenna port switching impact to DL only, and txSwitchWithAnotherBand indicates the SRS antenna port switching impact to UL only.

Discussion in GTW (Aug 17):

- Chair: Common understanding that txSwitchImpactToRx indicates the SRS antenna port switching impact to DL only, and txSwitchWithAnotherBand indicates the SRS antenna port switching impact to UL only. No common understanding if interruptions should be limited to one direction (DL or UL) or both DL and UL. Recommend to continue discussion.

- Recommended WF

  - Continue the discussion in 2nd round. Companies to check whether option 2 is agreeable or not. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | Support Option 1. From test case perspective, the ACK/NACK will be failed as long as one of UL or DL is impacted by SRS antenna port switching. |
| Apple | Support option 1 since Rx/Tx are always correlated. But if majority companies agree on option 2, we can also compromise to option 2 (UE would indicate both signalings when both Rx/Tx would be impacted ). |
| Xiaomi | We prefer Option2, to define the interruption requirements in the same way as the two IEs defined in TS 38.306. |
| Huawei | We support option 2. Proponents of option 1 argued that DL and UL interruptions could not be distinguished by current test procedure. We don’t think the definition of these two signals shall be changed for test purpose. |
| Intel | Can compromise to option 2. |
| Ericsson | Our understanding is in line with Option 2. |
| QC | As we commented on GTW, we would like to understand how network scheduling can be different between option 1 and 2. If we see the benefit in option 2, we are open to discuss compromised options. |
| Nokia | Option 2. After GTW discussion, there is consensus that different groups of carriers will be impacted due to SRS switching. We understood the UL and DL interruption shall be defined separately and respective test cases need to be defined. |
| vivo | Option 2. For option 1, we think the issue can be up-to test case design. For example, for the CA cases, NW may configure cross-carrier HARQ feedback, if only UL of a band is impacted, but DL is not. |
| CATT | Support option 2. Suggest keeping alignment with the signaling indication. |

Issue 1-3-2: Interruption requirement for UE with or without per-FR MG capability

- Proposals

  - Option 1 (CATT, Xiaomi, MTK, Ericsson, Nokia): The interruption requirements are defined only based on IEs of txSwitchImpactToRx or txSwitchWithAnotherBand indication. It is no need to clarify the relationship of the IEs with per-FR MG capability in RAN4 specification.

  - Option 2 (Apple, OPPO, MTK, Xiaomi, HW, Intel, Ericsson, vivo, CATT): No need to differentiate the requirement for the UE with or without capability of per-FR gap for SRS antenna port switching in RAN4. But the requirement applicability needs to be clarified that “for interruption caused by SRS antenna port switching, the victim CC would be based on indication of txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability”.

- Recommended WF

  - Continue the discussion in 2nd round. Please Nokia to check whether option 2 is agreeable. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | We can also support Option 2. |
| Apple | Option 2. We need clarification in the introduction section of interruption requirement to address the ambiguity as the existing introduction section only specified the interruption applicability for per-FR MG capability which cannot be used for SRS antenna port switching. |
| Xiaomi | Both Option1 and Option2 are fine with us. |
| Huawei | Support option 2. The clarification at beginning of 8.2 is needed. |
| OPPO | Option 2. |
| Intel | we are fine with option 2. |
| Ericsson | Seems there is no technical difference between Option 1 and Option 2. We are fine with either. The exact wording can be agreed when discussing draft CRs. |
| Nokia | Option 1. If we all agree to define the requirements based on the capability IEs, it seems unnecessary to further indicate per-FR capability is not followed. |
| vivo | Fine with option 2. |
| CATT | Fine with the both options. |

Issue 1-3-3: Would the interruption requirement differentiate between sync and async cases?

- Proposals

  - Option 1 (Apple, vivo, QC, Xiaomi, MTK, Intel, OPPO, HW, ZTE): No; one single requirement to cover the synchronous and asynchronous scenarios with or without UL TA.

    - Option 1a (Apple, OPPO): The unified interruption requirement is based on the async case for the minimum requirement.

    - Option 1b (LG): The same interruption length for both synchronous and asynchronous cases could be defined, and the interruption length should be reduced by one slot when the slot after SRS antenna post switching is downlink in synchronous case.

  - Option 2 (CATT, Ericsson, Nokia): Yes, the interruption requirement can differentiate between sync and async cases.

- Recommended WF

  - Moderator: Since this issue has been discussed for 3 meetings, could we compromise to start the work based on async case first (that’s the worst case for minimum requirement), and later if companies can prove there are big difference for sync case, we could define another set of interruption based on sync case.

  - Continue the discussion in 2nd round. Conclusions would be captured in the WF..

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | Support Option 1. One slot will be considered for async case if TA is considered. As a result, no difference between sync and async cases. |
| Apple | Option 1 and 1a. |
| LG | Generally, same requirement could be applied for sync and async. However, in sync case, when the slot after SRS transmission is DL, interruption does not occur in the DL slot since Tx-Rx switching is performed after SRS transmission. So, in this case, interruption length could be reduced by one slot. To reduce unnecessary interruption slot, option 1b should be considered. |
| Xiaomi | Support Option1. |
| Huawei | Option 1 |
| OPPO | Option 1 and 1a. |
| Intel | Support option 1 by considering the TA. |
| ZTE | Support option 1. |
| QC | Support option 1. |
| Nokia | Option 2. Same approach shall be followed as during SRS carrier based switching discussion. The requirements need to be derived for sync and async cases separately, and if the two can be merged can be further discussed afterward. |
| vivo | Support option 1. |
| CATT | Option 2. We think the TA cannot resolve the cross-slot interruption in asyn cases. And for the switching time after SRS transmission should not cause interruption for the next slot and the impact should be same as transient period. To reduce the interruption for sync case, the requirements should be differentiated for sync and async case. |
| Ericsson2 | Option 2, with same justification as provided by Nokia. First we derive how big interruption will result for each case, and then after that we look into whether to have separate or one common requirement. |

Issue 1-3-4: txSwitchImpactToRx for intra-band case

- Proposals

  - Option 1 (Intel):

    - Since txSwitchImpactToRx can’t differentiate the intra-contiguous CA and non-contiguous CA case, there are two solutions:

      - Option 1: Send LS to RAN2 to modify the signaling.

      - Option 2: Solve the issue in RAN4 by updating the applicability.

  - Option 2 (CATT, Apple, Xiaomi, MTK, OPPO, Nokia, CATT, Ericsson)

    - No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case.

- Recommended WF

  - Continue the discussion in 2nd round. Please Intel to check if can compromise to option 2. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | Support Option 2 to make the requirement simple. Because it is R15 feature, we slightly prefer not to change the R15 RAN2 specification anymore. |
| Apple | Option 2. |
| Xiaomi | Support Option2. |
| OPPO | Option 2 is fine. |
| Intel | We can compromise not to define different requirement for intra-band contiguous and non-contiguous case which is depending on txSwitchImpactToRx. However, we still have some concern as it actually has some UE implementation limitation. For example, as the following figure, Suppose carrier 1 is the PCell, carrier 2 is SCell. When UL switch happen in carrier 1, the interruption is defined on the other CC, which is carrier 2. If UE has different RXs, there may be no interruption to DL carrier 2. However, if only one requirement is defined for both contiguous and non-contiguous case, UE will always indicate that there is impact for carrier 2. It actually has limitation on the UE implementation. We understand that the worst case will be considered as some company comment. However, at least, I think that we should clarify this in spec, e.g. For intra-band case, the interruption always applied regardless of txSwitchImpactToRx, or UE will always indicate that there is impact to DL in txSwitchImpactToRx for intra-band case . |
| Nokia | Support Option 2. The proponent of Option 1 assumes “For intra-band contiguous CA, the interruption requirement is always applicable regardless of txSwitchImpactToRx.”. Could the company clarify the reasoning? |
| Intel | To Nokia, our proposal is to clarify for “intra-band case” which includes both contiguous and non-contiguous case. For intra-band contiguous CA, we assume that UE has only one set of RF chain, then if UL switch, it will have impact on the other DL SCCs. So for intra-band contiguous CA, UE will always indicate that there is impact to DL due to UL switch. For intra-band non-contiguous CA, UE has the flexibility to indicate whether there is impact to DL or not as it can have different RF chain. As commented from other companies, if we don’t want to differentiate the two cases and only consider the worst case, then UE can only indicate that there will always be impact to DL for two cases. Then there is UE implementation limitation for non-contiguous case. We suggest to clarify this in the spec. Option 1: For intra-band case, the interruption always applied. Option 2: UE will always indicate that there will be impact to DL in txSwitchImpactToRx for both contiguous and non-contiguous intra-band case . |
| CATT | Support option 2. Even for intra-band non-contiguous CA, UE may use the same antenna for the different receivers, in this case, the antenna port switching in one carrier can also cause interruption in the other carrier. |
| Ericsson2 | Preference for Option 2, as it is likely that the same antennas are used throughout the band. |

### Sub-topic 1-4: Interruption requirement design

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-4-1: The interruption requirement is defined based on slot level or symbol level

- Proposals

  - Option 1 (CATT, Apple, vivo, QC, Xiaomi, MTK, Intel, HW, OPPO): based on slot level

    - Option 1a (CATT): The interruption requirements will be defined based on slot level, i.e. no requirement for the case of interruption on flexible slot. It is not necessary to further discuss on components of making interruption if the interruption requirements are defined based on slot level.

    - Option 1b (vivo): The interruption requirement is preferred to be defined based on slot level. The mis-alignment in the UL frame boundary between anchor and victim cells needs to be further discussed.

    - Option 1c(LG): Slot based interruption requirement could be used in synchronous and asynchronous case, and only UL symbols based interruption should be defined when SRS antenna port switching is configured in the flexible slot in synchronous case.

  - Option 2 (Ericsson, Nokia): based on symbol level

    - Option 2a (Nokia): The transient period before and after the SRS resource will cause UL interruption of 1 OFDM symbol at 60kHz. At 15kHz or 30kHz SCS, RAN4 should discuss if and how to capture the transient periods from RF session in the RRM specifications.

- Recommended WF

  - Continue the discussion in 2nd round. If possible Moderator will recommend to discuss on 2nd round GTW. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | Support Option 1. The symbol level interruption is introduced due to the frequent switching within a slot for Tx switching. However, we do not see the reason to define SRS antenna switching based on symbol level. |
| Apple | Option 1. |
| LG | We support slot based interruption requirements. However, in sync case, only UL symbols would be interrupted when the SRS resources are configured in the flexible slot (DL symbols + UL symbols). So, to avoid interruption for DL symbols in the slot, option 1c should be considered. |
| Xiaomi | Support Option1 |
| Huawei | Option 1 |
| OPPO | Option 1. |
| Intel | Support option 1. It’s hard to verify the function in symbol level. |
| QC | We don’t think 1c is necessary. It is specifically for the case that all the victim carriers are with flexible slot when SRS carrier switching and we will have different set of requirements for different configurations which is not necessary. |
| Nokia | Option 2 and Option 2a. As the transient period may impact no more than 1 symbol, it is a waste to apply the interruption to the whole slot. We should not extend the interruption without any analysis. About Option 1, what is “flexible slot”? We are fine to analyze the impact due to timing alignment by starting on per-symbol basis. |
| vivo | Support option 1. But fine to discuss whether only limited set of SRS configuration is considered in the interruption requirements. For example, if network can prevent the scheduling of SRS at the slot boundary, the number of interrupted slot can be reduced, especially when the slot boundary is aligned and limited MRTD/MTTD is considered. |
| CATT | Option 1a. We think in the flexible slot, the decoding is based on symbol level, so the symbols not interrupted by SRS antenna port switching can still be scheduled i.e. in the flexible slot, the interruption should be symbol level. |
| Ericsson2 | Option 2 and Option 2a, at least until we have derived the interruption for each respective case. |

Issue 1-4-2: The components within interruption time of SRS antenna port switching in FR1

- Proposals

  - Option 1 (CATT, Apple, QC, Xiaomi, CMCC, OPPO, HW, MTK, Intel): The composites of interruption requirement for SRS antenna port switching in FR1 include:

    - SRS Transmission time (use 6 symbols as minimum requirement), and

    - Antenna switching time before and after SRS transmission occasion (2*15us).

  - Option 2 (vivo):

    - For interruption requirements, the interruption time is preferred to include power adjustment period, antenna switching time and SRS transmission time.

    - RAN4 can consider to define interruption requirements for a limited set of SRS configuration in R17.

  - Option 3 (LG): The composites of interruption requirement for SRS antenna port switching in FR1 include:

    - Antenna port switching time (15usec) before the first SRS transmission

    - Time from the first SRS transmission to the last SRS transmission

    - Antenna port switching back time (15usec) after the last SRS transmission

      - It should be excluded when the SRS resource is configured in the last symbol of a slot and the next slot is downlink

  - Option 4 (Nokia, Ericsson):

    - When the SRS resources of a set are not configured in a slot, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE, where the uplink interruption comprises at least the SRS transmission period.

    - When the SRS resources of a set are configured in a slot with consecutive SRS transmission, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE, where the uplink interruption comprises at least the SRS transmission periods and the guard period in-between.

    - The position of the transient period needs to be clarified before defining the interruption and/or the scheduling restriction in RRM spec.

    - Add one note indicating the DL may be affected due to SRS antenna switching if txSwitchImpactToRx is configured.

  - Option 5 (Ericsson): The components within the interruption requirement shall include SRS transmission time for the configured number of symbols, as well as SRS antenna switching time before and after the SRS transmission.

- Recommended WF

  - Continue the discussion in 2nd round. If possible Moderator will recommend to discuss on 2nd round GTW. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | Support Option 1. The reason please refer to our discussion paper. |
| Apple | Option 1. |
| LG | As commented above Issue 1-3-3 and 1-4-1, RAN4 needs further check applying the transient time before and after SRS transmission according to slot configuration |
| Xiaomi | Support Option 1. |
| Huawei | Option 1 |
| OPPO | Option 1 |
| Intel | Support option 1. |
| Ericsson | We support Option 4. |
| QC | Support option 1. We would like to understand which cases option 1 doesn’t cover in option 4, if we consider each slot separately if SRS resource occupying more than one slot. |
| Nokia | Option 4. For SRS transmission time, the UL transmission on other carriers is prohibited only if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE. Otherwise, the UL transmission is allowed hence no interruption shall be applied. Also it is a waste to apply interruption to the overall 6 OFDM symbols which is not efficient from network point of view. The switching time due to transient period needs to be discussed taking RF specification into account. For instance, the 15us impact is different in respective SCS. And the timing alignment between aggressor and victim cells can also be considered. |
| vivo | Option 2. For option 1. Do not think SRS transmission always need 6 symbols. For option 4, DL interruption requirement is necessary in our view. But it is reasonable to reduce the number of slots that could be interrupted. |
| CATT | Support option 1. |

Issue 1-4-3: Interruption requirement proposals

- Proposals

  - Option 1 (CATT): The interruption requirements are proposed defined as table 1 and table 2.

Table 1. Interruption (slot number) requirement for Synchronized network

| Victim cell SCS | Aggressor cell SCS, number SRS symbols on other antenna port |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 15kHz |  |  | 30kHz | 60kHz |
|  | 1 | 2 | 4 |  |  |
| 15kHz or 30kHz | 1 | 1 | 1 | 1 | 1 |
| 60kHz | 1 | 1 | 2 | 1 | 1 |

Table 2. Interruption (slot number) requirement for asynchronized network

| Victim cell SCS | Aggressor cell SCS, number SRS symbols on other antenna port |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 15kHz |  |  | 30kHz | 60kHz |
|  | 1 | 2 | 4 |  |  |
| 15kHz or 30kHz | 2 | 2 | 2 | 2 | 2 |
| 60kHz | 2 | 2 | 3 | 2 | 2 |

  - Option 2 (Apple, QC, MTK): the interruption requirement of SRS antenna port switching is summarized as:

| Victim CC SCS(kHz) | Aggressor CC SCS (kHz) |  |  |
| --- | --- | --- | --- |
|  | 15 | 30 | 60 |
| 15 (NR or LTE) | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 3 |

Unit of interruption requirement is slot for NR and subframe for LTE.

  - Option 3 (Huawei):

|  | Aggressor CC SCS(kHz) |  |  |
| --- | --- | --- | --- |
| Victim CC SCS (kHz) | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| 120 | 5 | 3 | 2 |

  - Option 4 (LG): Interruption requirements for SRS antenna port switching are shown in Table 1.

Table 1 Proposed interruption requirements

| Victim cell SCS [kHz] | Interruption length [slot] |  |  |
| --- | --- | --- | --- |
|  | Aggressor cell SCS [kHz] |  |  |
|  | 15 | 30 | 60 |
| 15 | 2 | 2 | 2 |
| 30 | 2 | 2 | 2 |
| 60 | 3 | 2 | 2 |
| Note 1: The interruption length should be reduced by one slot when the slot after SRS antenna post switching is downlink in synchronous case. Note 2: If SRS antenna port switching is configured in the flexible slot in synchronous case, the interruption applies to only uplink symbols in that slot. |  |  |  |

- Recommended WF

  - Up to conclusions from other issues. Continue the discussion in 2nd round. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | Support Option 2. |
| Apple | Option 2. But we could wait more conclusions from other issues. |
| LG | Support recommended WF |
| Xiaomi | Support recommended WF |
| Huawei | Suggest to focus on the other issues first before discussing the detailed requirements. |
| OPPO | Support recommended WF |
| ZTE | Agree with the recommended WF. |
| Ericsson | Agree with the recommended WF. |
| Nokia | Agree with the recommended WF. |
| vivo | Support recommended WF |
| CATT | Fine with the recommended WF. |

### Sub-topic 1-5: Others

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-5-1: Network to obtain the interrupted symbol info

- Proposals

  - Option 1 (vivo): If needed, RAN4 can study whether and how network can obtain the interrupted symbol information, when SRS antenna port switching is performed in another band.

  - Option 2 (MTK, Apple, HW, QC, CATT) : option 1 is not needed.

  - Option 3: Do not need to discuss this issue in R17 FeRRM WI.

- Recommended WF

  - Tentative agreements:

    - No need to discuss this issue in R17 FeRRM WI.

  - Companies to confirm on tentative agreement. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | More discussion is needed. For R17, we do not prefer to extend the WI scope to have this study. But we are ok discuss this in R18. |
| Apple | Option 2. In RAN4, interruption is always defined as an uncertainty part out of network control. |
| Huawei | Prefer option 2. Similar views as Apple |
| OPPO | Open to discuss. |
| QC | Same view and apple and Huawei. |
| Nokia | We would like to understand this issue better. Interruption requirements are defined so that both UE and network knows when/where the interruption occurs. What additional information does the network need to know? |
| vivo | To Nokia, our intention is that in some cases, network is able to locate the interrupted symbols slightly better than some other cases. For example, if NW can ensure slot boundary alignment, (Note UL/DL does not necessarily align), and based on MRTD and MTTD and maximum TA, some rough information of the interrupted symbols can be derived at network side. In this case, at least the interrupted symbols at slot boundary is not necessary. However we also understand the concern from MTK and we are fine to further discuss in future release. |
| CATT | Option 2. Same view as Apple. |

Issue 1-5-2: Interruption requirement related with prioritization rule in RAN1

- Proposals

  - Option 1 (MTK, Apple, OPPO, vivo):

    - For SRS antenna port switching, define the interruption requirement for the case not covered by TS 38.214, e.g., add a note “if the case has no priority rule defined in TS 38.214, UE is allowed to cause the interruption on the other CC(s)”.

  - Option 2 (Nokia, Ericsson) :

    - The SRS transmission will cause UL interruption on the other carriers if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE.

  - Option 3 (Huawei, Ericsson, CATT):

    - Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement.

- Recommended WF

  - Continue the discussion in 2nd round. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | Support Option 1. As far as we know, some cases are not covered by TS 38.214 and it might be related to Issue 1-2-2 and 1-2-3. For all physical channel not prioritized in RAN1 and in Issue 1-2-2 and 1-2-3, we suggest to define the interruption requirement. |
| Apple | Option 1 is generally fine, but if we agreed that NR measurement shall be also prioritized, we shall also include that in the condition, i.e., if the case has no priority rule defined in TS 38.214 and not colliding with any NR measurements, UE is allowed to cause the interruption on the other CC(s) |
| Huawei | The first principle is that the UE behavior already defined in RAN1 shall not be changed, which may cause NBC issue. If such unclear case is identified, it is preferred to be clarified in RAN1. |
| OPPO | Option 1 is fine. |
| Ericsson | Agree with Option 2 and Option 3. |
| QC | Our understanding is SRS antenna switching scheduling and execution should follow RAN1 and RAN4 agreed rules. Once UE checked these rules and performed SRS antenna switching, the interruption requirement applies. This is quite clear and we don’t see the need of additional clarification. Option 2 is related to SRS transmission itself, not SRS antenna switching. It’s out of scope for this WI. |
| Nokia | Our proposal in Option 2 is more relevant to the interruption applicability. That is, the UL interruption on other carrier applies only if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE. Regarding to the priority rules, we need follow the principles defined by RAN1 and no need to specify/clarify further in RAN4 spec. In RAN4, it is sufficient to discuss the collision cases between SRS and DL measurements as in Issue 1-2. |
| vivo | Support option 1. |
| CATT | Option 3. We think option 2 can be addressed by priority rules defined in RAN1. |

Issue 1-5-3: Two SRS colliding on same symbol

- Proposals (MTK):

  - For SRS antenna port switching (FR1 only), RAN4 needs to further study the corresponding UE behavior when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol:

    - Option1 (Apple): Two different SRS resources having the same time domain behavior to be scheduled on the same OFDM symbol are not expected.

    - Option2: Up to UE implementation.

    - Option3 (Nokia): This is up to RAN1 discussion.

- Recommended WF

  - Continue the discussion in 2nd round. Companies to check if option 3 can be a compromise. Conclusions would be captured in the WF.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| MediaTek | According to current specification, it is possible that two SRS resource having same time behavior are transmitted in the same OFDM symbol. However, no priority rule in RAN1/4 spec to cover this issue. Thus, we suggest RAN4 to further discuss this issue. |
| Apple | Option 1 is preferred. |
| Huawei | The related issue is under discussion in RAN1 FeMIMO as least for AP SRS collision. Suggest to wait for more RAN1 conclusions on this. |
| Ericsson | If anything is unclear with RAN1 specification, then it is likely better that we send an LS to RAN1 to get clarification before defining additional rules. |
| Nokia | The priority handling between PUCCH/PUSCH/SRS and SRS is specified in RAN1. And some priority handling is under discussion also in RAN1. Could we add one option below? Option x: This is up to RAN1 discussion. |
| vivo | For AP SRS collision, more discussion is needed. |
| CATT | Same view as Huawei. If RAN1 is discussing the collision, we should wait for the conclusion. |

## Companies views’ collection for 1st round

### Open issues

Comments are collected in section 1.2

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Sub-topic 1-1: Scope of SRS antenna switching requirement

|  | Status summary |
| --- | --- |
| Issue 1-1-1: whether scheduling restriction requirement would be defined in RRM for SRS antenna port switching | Tentative agreements: - Agreements in GTW(Aug 17): - Do not define the scheduling restriction on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols in Rel-17 - Performance degradation on these symbols can be expected - FFS how to capture this in TS 38.133 Candidate options: - Option 1 (QC, CATT, vivo): Capture the GTW agreement in WF without changing specification. - Option 2 (OPPO, Nokia): FFS how to capture GTW agreement into TS 38.133 Recommendations for 2nd round: Based on GTW agreement, please companies discuss above option 1 and option 2. The proponents of option 2 may propose how to capture GTW agreement into TS 38.133. Continue the discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-1-2: RAN4 requirement scope with different SRS resource configuration | Tentative agreements: Candidate options: - Option 1 (CATT): - Only defining the requirements for the scenario that the SRS resources of a set are transmitted in the same slot with consecutive SRS transmission. - Option 2 (Nokia, Ericsson): - RAN4 shall define the requirements for the following scenarios in Rel17 where - The SRS resources of a set are transmitted NOT in the same slot, or - The SRS resources of a set are transmitted in the same slot with consecutive SRS transmission - RAN4 do not define the requirements if the SRS resources of a set are transmitted in the same slot with non-consecutive SRS transmission, before the guard period in this scenario gets clarified in RAN1. - Option 3 (Apple, Xiaomi, Huawei, QC, vivo, CATT): - RAN4 to define a generic requirement regardless of SRS resource configuration. - Option 3a: define interruption requirements for a limited set of SRS configurations. Recommendations for 2nd round: Continue the discussion in 2nd round. 6 companies supported option 3, but other companies had different views, and 3 companies suggested to wait conclusions from issue 1-4-1. Conclusions would be captured in the WF. |

Sub-topic 1-2: Impact of SRS antenna port switching to other requirements

|  | Status summary |
| --- | --- |
| Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA | Tentative agreements: Candidate options: - Option 1 (Apple, vivo, QC, MTK, HW, LG, xiaomi, HW, OPPO, Intel, ZTE, Nokia): No impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized. - Option 1a: The above-mentioned NR measurement requirements including serving/neighboringcell L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD measurement - Option 1b (Nokia): The above-mentioned NR measurement requirements including serving/ neighbouring cell L3 measurement and RLM/BFD/CBD measurement - Option 1c (HW): The above-mentioned NR measurement requirements including serving/ neighbouring cell L3 measurement and RLM/BFD/CBD measurement. FFS on L1-RSRP/L1-SINR. - Option 1d (moderator added): - For NR-SA, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol. - For NR-SA, UE is not required to perform SRS antenna port switching when P/SP SRS resource and the AP L1-RSRP/L1-SINR measurement, are scheduled in the same OFDM symbol. - Option 2 (CATT): The aperiodic SRS antenna port switching can impact on L1-RSRP/L1-SINR and RLM/BFD/CBD measurement requirements if the IE txSwitchImpactToRx include the working band for FDD. - Option 3 (Ericsson, HW): RAN4 to further discuss whether collisions between PUCCH carrying L1 measurement report and aperiodic SRS transmission needs to be handled within this work item, or whether it is already covered in the specification. The justification being that the RAN1 rule on how to handle conflicts is based on the SRS transmission as such, rather than on whether there is SRS antenna port switching. - Option 4 (OPPO): It can be maintained by network scheduling to avoid the case when SRS antenna port switching is colliding with L1 reference signals, e.g., for L1-RSRP/L1-SINR and RLM/BFD/CBD measurement, CSI-IM except DMRS and UCI containing CSF report. Recommendations for 2nd round: Moderator: In TS38.214 section 6.2.1.3, the prioritization for CSI reporting and SRS carrier switching is summarized as below, PUCCH/PUSCH for CSI (L1-RSRP/L1-SINR) reporting; SRS transmission (including interruption); Prioritization. P/SP CSI; AP SRS; AP SRS. AP CSI; AP SRS; AP SRS. AP CSI; P/SP SRS; AP CSI. P/SP CSI; P/SP SRS; P/SP SRS. Please companies check the above prioritization and discuss: - Whether the CSI reporting prioritization could be equivalent to CSI measurement prioritization? - Whether SRS carrier switching prioritization could be reused for SRS antenna port switching prioritization? If ‘yes’ to both above questions, only when AP CSI colliding with P/SP SRS, the AP CSI shall be prioritized, otherwise SRS shall be prioritized. This is the new added option 1d. Continue the discussion in 2nd round. New options have been added under option 1. Conclusions would be captured in the WF. |
| Issue 1-2-2: Impact of SRS antenna port switching to RRM requirements in EN-DC or NE-DC | Tentative agreements: Candidate options: - Option 1 (Apple, LG, Xiaomi, OPPO, QC, vivo, CATT): For EN-DC and NE-DC, no impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized. - The above-mentioned NR measurement requirements including serving/neighboring cell L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD measurement - Option 2 (MTK, Apple , LG, Xiaomi, OPPO, QC, vivo): - For EN-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the SCG. - For NE-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the MCG. - Option 3 (OPPO): It can be maintained by network scheduling to avoid the case when SRS antenna port switching is colliding with L1 reference signals, e.g., for L1-RSRP/L1-SINR and RLM/BFD/CBD measurement, CSI-IM except DMRS and UCI containing CSF report. - Option 4 (Nokia, Ericsson, Huawei): - For EN-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the SCG. - For NE-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the MCG. - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1) Recommendations for 2nd round: Continue the discussion in 2nd round. Companies to check whether option 4 is agreeable or not. Conclusions would be captured in the WF. |
| Issue 1-2-3: Impact of SRS antenna port switching to RRM requirements in NR-DC | Tentative agreements: Candidate options: - Option 1 (Apple, OPPO, CATT, QC, LG): No impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized. - Option 1a (Apple, QC): the above-mentioned NR measurement requirements in option 1 including serving/neighboring cell L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD measurement - Option 1b (OPPO): It can be maintained by network scheduling to avoid the case when SRS antenna port switching is colliding with L1 reference signals, e.g., for L1-RSRP/L1-SINR and RLM/BFD/CBD measurement, CSI-IM except DMRS and UCI containing CSF report. - Option 2 (): For NR-DC, NR measurement shall always be prioritized within the same CG. - Option 2a (MTK, Apple, Xiaomi, OPPO, Intel,): UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the same CG. - Option 2b(Ericsson): Add a note indicating the DL in one CG may be affected by SRS antenna switching in another CG when txSwitchImpactToRx has been reported by UE. - Option 3 (vivo): Define dropping rules for NR SRS antenna switching, at least for the case when it collides with other NR RRM measurements. In this case the interruption requirements does not apply. - Option 4 (Nokia, HW, Ericsson): - For NR-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the same CG. - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1) Recommendations for 2nd round: Continue the discussion in 2nd round. Companies to check whether option 4 is agreeable or not. Conclusions would be captured in the WF. |
| Issue 1-2-4: Impact of SRS antenna port switching to other specific RRM requirements | Tentative agreements: Candidate options: - Option 1 (CATT, vivo, OPPO, MTK, ZTE, vivo): The requirement for handover, BWP switching, SCell activation/deactivation should not be impacted by SRS antenna port switching. - Option 2 (Apple, QC, MTK, LG, Xiaomi, OPPO, ZTE, vivo): RAN4 to clarify that legacy requirement for handover, BWP switching, SCell activation/deactivation only applies when no SRS antenna port switching occurs during those activities. - Option 3 (Huawei, Xiaomi, Ericsson, Nokia): RAN4 not to define prioritizing rules of SRS antenna switching and other specific RRM requirements except for the NR measurements. Recommendations for 2nd round: Moderator: please proponents of option 2 and 3 check if option 4 is acceptable, because ‘not to define prioritization rule’ also means those existing specific RRM requirement applies when we don’t consider SRS antenna switching colliding with them. Continue the discussion in 2nd round. Companies to check whether option 4 is agreeable or not. Conclusions would be captured in the WF. |
| Issue 1-2-5: Impact of SRS antenna port switching to CSF and other RS | Tentative agreements: Candidate options: - Option 1 (CATT): If it is necessary to check, RAN4 can make requirements and test for the collision cases of SRS and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH according to the rules defined in clause 6.2.1 in RAN1specification. - Option 2 (Apple, QC, OPPO, vivo): Scheduling of SRS antenna switching should avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report. If collision happens, it is considered as an error case and no UE requirement is imposed. - Option 3 (vivo): Send LS to RAN1 to check the prioritization rule for SRS antenna switching, especially for the case in CA/DC operation. - Option 3a (Ericsson, Nokia): Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels is to be handled by RAN1. If anything is unclear, RAN4 shall send LS to RAN1 and ask for clarification. - Option 4 (Huawei, Ericsson, Nokia, CATT): Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement. Recommendations for 2nd round: Moderator: since companies mentioned such prioritization was also defined in RAN1, please companies to check with your RAN1 colleagues and then discuss if option 3a is agreeable or not. Continue the discussion in 2nd round. Conclusions would be captured in the WF. |

Sub-topic 1-3: Interruption requirement applicability

|  | Status summary |
| --- | --- |
| Issue 1-3-1: Interruption requirement applicability | Tentative agreements: Candidate options: - Option 1 (Apple, QC, MTK, Intel, OPPO): SRS antenna switching interruptions on both DL and UL applies to the band combinations signaled in txSwitchImpactToRx or txSwitchWithAnotherBand. - Option 2 (CATT, vivo, Xiaomi, Nokia, HW, Apple, Intel, Ericsson): txSwitchImpactToRx indicates the SRS antenna port switching impact to DL only, and txSwitchWithAnotherBand indicates the SRS antenna port switching impact to UL only. Recommendations for 2nd round: Continue the discussion in 2nd round. Companies to check whether option 2 is agreeable or not. Conclusions would be captured in the WF. |
| Issue 1-3-2: Interruption requirement for UE with or without per-FR MG capability | Tentative agreements: Candidate options: - Option 1 (CATT, Xiaomi, MTK, Ericsson, Nokia): The interruption requirements are defined only based on IEs of txSwitchImpactToRx or txSwitchWithAnotherBand indication. It is no need to clarify the relationship of the IEs with per-FR MG capability in RAN4 specification. - Option 2 (Apple, OPPO, MTK, Xiaomi, HW, Intel, Ericsson, vivo, CATT): No need to differentiate the requirement for the UE with or without capability of per-FR gap for SRS antenna port switching in RAN4. But the requirement applicability needs to be clarified that “for interruption caused by SRS antenna port switching, the victim CC would be based on indication of txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability”. Recommendations for 2nd round: Continue the discussion in 2nd round. Please Nokia to check whether option 2 is agreeable. Conclusions would be captured in the WF. |
| Issue 1-3-3: Would the interruption requirement differentiate between sync and async cases? | Tentative agreements: Candidate options: - Option 1 (Apple, vivo, QC, Xiaomi, MTK, Intel, OPPO, HW, ZTE): No; one single requirement to cover the synchronous and asynchronous scenarios with or without UL TA. - Option 1a (Apple, OPPO): The unified interruption requirement is based on the async case for the minimum requirement. - Option 1b (LG): The same interruption length for both synchronous and asynchronous cases could be defined, and the interruption length should be reduced by one slot when the slot after SRS antenna post switching is downlink in synchronous case. - Option 2 (CATT, Ericsson, Nokia): Yes, the interruption requirement can differentiate between sync and async cases. Recommendations for 2nd round: Moderator: Since this issue has been discussed for 3 meetings, could we compromise to start the work based on async case first (that’s the worst case for minimum requirement), and later if companies can prove there are big difference for sync case, we could define another set of interruption based on sync case. Continue the discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-3-4: txSwitchImpactToRx for intra-band case | Tentative agreements: Candidate options: - Option 1 (Intel): - Since txSwitchImpactToRx can’t differentiate the intra-contiguous CA and non-contiguous CA case, there are two solutions: - Option 1: Send LS to RAN2 to modify the signaling. - Option 2: Solve the issue in RAN4 by updating the applicability. - Option 2 (CATT, Apple, Xiaomi, MTK, OPPO, Nokia, CATT, Ericsson) - No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case. Recommendations for 2nd round: Continue the discussion in 2nd round. Please Intel to check if can compromise to option 2. Conclusions would be captured in the WF. |

Sub-topic 1-4: Interruption requirement design

|  | Status summary |
| --- | --- |
| Issue 1-4-1: The interruption requirement is defined based on slot level or symbol level | Tentative agreements: Candidate options: - Option 1 (CATT, Apple, vivo, QC, Xiaomi, MTK, Intel, HW, OPPO): based on slot level - Option 1a (CATT): The interruption requirements will be defined based on slot level, i.e. no requirement for the case of interruption on flexible slot. It is not necessary to further discuss on components of making interruption if the interruption requirements are defined based on slot level. - Option 1b (vivo): The interruption requirement is preferred to be defined based on slot level. The mis-alignment in the UL frame boundary between anchor and victim cells needs to be further discussed. - Option 1c(LG): Slot based interruption requirement could be used in synchronous and asynchronous case, and only UL symbols based interruption should be defined when SRS antenna port switching is configured in the flexible slot in synchronous case. - Option 2 (Ericsson, Nokia): based on symbol level - Option 2a (Nokia): The transient period before and after the SRS resource will cause UL interruption of 1 OFDM symbol at 60kHz. At 15kHz or 30kHz SCS, RAN4 should discuss if and how to capture the transient periods from RF session in the RRM specifications. Recommendations for 2nd round: Continue the discussion in 2nd round. If possible Moderator will recommend to discuss on 2nd round GTW. Conclusions would be captured in the WF. |
| Issue 1-4-2: The components within interruption time of SRS antenna port switching in FR1 | Tentative agreements: Candidate options: - Option 1 (CATT, Apple, QC, Xiaomi, CMCC, OPPO, HW, MTK, Intel): The composites of interruption requirement for SRS antenna port switching in FR1 include: - SRS Transmission time (use 6 symbols as minimum requirement), and - Antenna switching time before and after SRS transmission occasion (2*15us). - Option 2 (vivo): - For interruption requirements, the interruption time is preferred to include power adjustment period, antenna switching time and SRS transmission time. - RAN4 can consider to define interruption requirements for a limited set of SRS configuration in R17. - Option 3 (LG): The composites of interruption requirement for SRS antenna port switching in FR1 include: - Antenna port switching time (15usec) before the first SRS transmission - Time from the first SRS transmission to the last SRS transmission - Antenna port switching back time (15usec) after the last SRS transmission - It should be excluded when the SRS resource is configured in the last symbol of a slot and the next slot is downlink - Option 4 (Nokia, Ericsson): - When the SRS resources of a set are not configured in a slot, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE, where the uplink interruption comprises at least the SRS transmission period. - When the SRS resources of a set are configured in a slot with consecutive SRS transmission, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE, where the uplink interruption comprises at least the SRS transmission periods and the guard period in-between. - The position of the transient period needs to be clarified before defining the interruption and/or the scheduling restriction in RRM spec. - Add one note indicating the DL may be affected due to SRS antenna switching if txSwitchImpactToRx is configured. - Option 5 (Ericsson): The components within the interruption requirement shall include SRS transmission time for the configured number of symbols, as well as SRS antenna switching time before and after the SRS transmission. Recommendations for 2nd round: Continue the discussion in 2nd round. If possible Moderator will recommend to discuss on 2nd round GTW. Conclusions would be captured in the WF. |
| Issue 1-4-3: Interruption requirement proposals | Tentative agreements: Candidate options: Options in section 1.2.4. Recommendations for 2nd round: Up to conclusions from other issues. Continue the discussion in 2nd round. Conclusions would be captured in the WF. |

1.2.5	Sub-topic 1-5: Others

|  | Status summary |
| --- | --- |
| Issue 1-5-1: Network to obtain the interrupted symbol info | Tentative agreements: No need to discuss this issue in R17 FeRRM WI. Candidate options: - Option 1 (vivo): If needed, RAN4 can study whether and how network can obtain the interrupted symbol information, when SRS antenna port switching is performed in another band. - Option 2 (MTK, Apple, HW, QC, CATT) : option 1 is not needed. - Option 3: No need to discuss this issue in R17 FeRRM WI. Recommendations for 2nd round: Companies to confirm on tentative agreement. Conclusions would be captured in the WF. |
| Issue 1-5-2: Interruption requirement related with prioritization rule in RAN1 | Tentative agreements: Candidate options: - Proposals - Option 1 (MTK, Apple, OPPO, vivo): - For SRS antenna port switching, define the interruption requirement for the case not covered by TS 38.214, e.g., add a note “if the case has no priority rule defined in TS 38.214, UE is allowed to cause the interruption on the other CC(s)”. - Option 2 (Nokia, Ericsson) : - The SRS transmission will cause UL interruption on the other carriers if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE. - Option 3 (Huawei, Ericsson, CATT): - Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement. Recommendations for 2nd round: Continue the discussion in 2nd round. Conclusions would be captured in the WF. |
| Issue 1-5-3: Two SRS colliding on same symbol | Tentative agreements: Candidate options: - For SRS antenna port switching (FR1 only), RAN4 needs to further study the corresponding UE behavior when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol: - Option1 (Apple): Two different SRS resources having the same time domain behavior to be scheduled on the same OFDM symbol are not expected. - Option2: Up to UE implementation. - Option3 (Nokia): This is up to RAN1 discussion. Recommendations for 2nd round: Continue the discussion in 2nd round. Companies to check if option 3 can be a compromise. Conclusions would be captured in the WF. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Issue 1-1-1: whether scheduling restriction requirement would be defined in RRM for SRS antenna port switching

- Proposals

- Agreements in GTW(Aug 17):

  - Do not define the scheduling restriction on symbols before and after SRS transmission for the cell with SRS antenna port switching and on SRS transmit symbols in Rel-17

    - Performance degradation on these symbols can be expected

    - FFS how to capture this in TS 38.133

- Option 1 (QC, CATT, vivo): Capture the GTW agreement in WF without changing specification.

- Option 2 (OPPO, Nokia): FFS how to capture this in TS 38.133

- Recommended WF

  - Based on GTW agreement, please companies discuss above option 1 and option 2. The proponents of option 2 may propose how to capture GTW agreement into TS 38.133. Continue the discussion in 2nd round. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-1-2: RAN4 requirement scope with different SRS resource configuration

- Proposals

  - Option 1 (CATT):

    - Only defining the requirements for the scenario that the SRS resources of a set are transmitted in the same slot with consecutive SRS transmission.

  - Option 2 (Nokia, Ericsson):

    - RAN4 shall define the requirements for the following scenarios in Rel17 where

      - The SRS resources of a set are transmitted NOT in the same slot, or

      - The SRS resources of a set are transmitted in the same slot with consecutive SRS transmission

    - RAN4 do not define the requirements if the SRS resources of a set are transmitted in the same slot with non-consecutive SRS transmission, before the guard period in this scenario gets clarified in RAN1.

  - Option 3 (Apple, Xiaomi, Huawei, QC, vivo, CATT):

    - RAN4 to define a generic requirement regardless of SRS resource configuration.

      - Option 3a: define interruption requirements for a limited set of SRS configurations.

- Recommended WF

  - Continue the discussion in 2nd round. 6 companies supported option 3, but other companies had different views, and 3 companies suggested to wait conclusions from issue 1-4-1. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-2-1: Impact of SRS antenna port switching to RRM requirements in NR-SA

- Proposals

  - Option 1 (Apple, vivo, QC, MTK, HW): No impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized.

    - Option 1a: The above-mentioned NR measurement requirements including serving/neighboringcell L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD measurement

    - Option  1b (Nokia): The above-mentioned NR measurement requirements including serving/ neighbouring cell L3 measurement and RLM/BFD/CBD measurement

    - Option 1c (HW): The above-mentioned NR measurement requirements including serving/ neighbouring cell L3 measurement and RLM/BFD/CBD measurement. FFS on L1-RSRP/L1-SINR.

    - Option 1d (moderator added):

      - For NR-SA, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol.

      - For NR-SA, UE is not required to perform SRS antenna port switching when P/SP SRS resource and the AP L1-RSRP/L1-SINR measurement, are scheduled in the same OFDM symbol.

  - Option 2 (CATT): The aperiodic SRS antenna port switching can impact on L1-RSRP/L1-SINR and RLM/BFD/CBD measurement requirements if the IE txSwitchImpactToRx include the working band for FDD.

  - Option 3 (Ericsson): RAN4 to further discuss whether collisions between PUCCH carrying L1 measurement report and aperiodic SRS transmission needs to be handled within this work item, or whether it is already covered in the specification. The justification being that the RAN1 rule on how to handle conflicts is based on the SRS transmission as such, rather than on whether there is SRS antenna port switching.

  - Option 4 (OPPO): It can be maintained by network scheduling to avoid the case when SRS antenna port switching is colliding with L1 reference signals, e.g., for L1-RSRP/L1-SINR and RLM/BFD/CBD measurement, CSI-IM except DMRS and UCI containing CSF report.

- Recommended WF

Moderator: In TS38.214 section 6.2.1.3, the prioritization for CSI reporting and SRS carrier switching is summarized as below,

| PUCCH/PUSCH for CSI (L1-RSRP/L1-SINR) reporting | SRS transmission (including interruption) | Prioritization |
| --- | --- | --- |
| P/SP CSI | AP SRS | AP SRS |
| AP CSI | AP SRS | AP SRS |
| AP CSI | P/SP SRS | AP CSI |
| P/SP CSI | P/SP SRS | P/SP SRS |

Please companies check the above prioritization and discuss:

- Whether the CSI reporting prioritization could be equivalent to CSI measurement prioritization?

- Whether SRS carrier switching prioritization could be reused for SRS antenna port switching prioritization?

If ‘yes’ to both above questions, only when AP CSI colliding with P/SP SRS, the AP CSI shall be prioritized, otherwise SRS shall be prioritized. This is the new added option 1d.

  - Continue the discussion in 2nd round. New options have been added under option 1. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-2-2: Impact of SRS antenna port switching to RRM requirements in EN-DC or NE-DC

- Proposals

  - Option 1 (Apple, LG, Xiaomi, OPPO, QC, vivo, CATT): For EN-DC and NE-DC, no impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized.

    - The above-mentioned NR measurement requirements including serving/neighboring cell L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD measurement

  - Option 2 (MTK, Apple , LG, Xiaomi, OPPO, QC, vivo):

    - For EN-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the SCG.

    - For NE-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the MCG.

  - Option 3 (OPPO): It can be maintained by network scheduling to avoid the case when SRS antenna port switching is colliding with L1 reference signals, e.g., for L1-RSRP/L1-SINR and RLM/BFD/CBD measurement, CSI-IM except DMRS and UCI containing CSF report.

  - Option 4 (Nokia, Ericsson, Huawei):

    - For EN-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the SCG.

    - For NE-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the MCG.

    - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1)

- Recommended WF

  - Continue the discussion in 2nd round. Companies to check whether option 4 is agreeable or not. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-2-3: Impact of SRS antenna port switching to RRM requirements in NR-DC

- Proposals

  - Option 1 (Apple, OPPO, CATT, QC, LG): No impact to NR measurement requirements relevant to measurements based on SSB/CSI-RS due to NR SRS antenna switching, as NR measurements are always prioritized.

    - Option 1a (Apple, QC): the above-mentioned NR measurement requirements in option 1 including serving/neighboring cell L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD measurement

    - Option 1b (OPPO): It can be maintained by network scheduling to avoid the case when SRS antenna port switching is colliding with L1 reference signals, e.g., for L1-RSRP/L1-SINR and RLM/BFD/CBD measurement, CSI-IM except DMRS and UCI containing CSF report.

  - Option 2 (): For NR-DC, NR measurement shall always be prioritized within the same CG.

    - Option 2a (MTK, Apple, Xiaomi, OPPO, Intel,): UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement, L1-RSRP/L1-SINR and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the same CG.

    - Option 2b(Ericsson): Add a note indicating the DL in one CG may be affected by SRS antenna switching in another CG when txSwitchImpactToRx has been reported by UE.

  - Option 3 (vivo): Define dropping rules for NR SRS antenna switching, at least for the case when it collides with other NR RRM measurements. In this case the interruption requirements does not apply.

  - Option 4 (Nokia, HW, Ericsson):

    - For NR-DC, UE is not required to perform SRS antenna port switching when SRS resource and the NR measurement, i.e., L3 measurement and RLM/BFD/CBD, are scheduled in the same OFDM symbol in the same CG.

    - FFS on L1-RSRP/L1-SINR (follow the conclusion from issue 1-2-1)

- Recommended WF

  - Continue the discussion in 2nd round. Companies to check whether option 4 is agreeable or not. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-2-4: Impact of SRS antenna port switching to other specific RRM requirements

- Proposals

  - Option 1 (CATT, vivo, OPPO, MTK, ZTE, vivo): The requirement for handover, BWP switching, SCell activation/deactivation should not be impacted by SRS antenna port switching.

  - Option 2 (Apple, QC, MTK, LG, Xiaomi, OPPO, ZTE, vivo): RAN4 to clarify that legacy requirement for handover, BWP switching, SCell activation/deactivation only applies when no SRS antenna port switching occurs during those activities.

  - Option 3 (Huawei, Xiaomi, Ericsson, Nokia): RAN4 not to define prioritizing rules of SRS antenna switching and other specific RRM requirements except for the NR measurements.

  - Option 4 (moderator added):

    - RAN4 to clarify that other specific RRM requirements except for the NR measurements only applies when no SRS antenna port switching occurs during those RRM activities.

    - RAN4 not to define prioritizing rules of SRS antenna switching and other specific RRM requirements except for the NR measurements.

- Recommended WF

  - Moderator: please proponents of option 2 and 3 check if option 4 is acceptable, because ‘not to define prioritization rule’ also means those existing specific RRM requirement applies when we don’t consider SRS antenna switching colliding with them.

  - Continue the discussion in 2nd round. Companies to check whether option 4 is agreeable or not. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-2-5: Impact of SRS antenna port switching to CSF and other RS

- Proposals

  - Option 1 (CATT): If it is necessary to check, RAN4 can make requirements and test for the collision cases of SRS and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH according to the rules defined in clause 6.2.1 in RAN1specification.

  - Option 2 (Apple, QC, OPPO, vivo): Scheduling of SRS antenna switching should avoid collision to all reference signals including CSI-IM except DMRS and UCI containing CSF report. If collision happens, it is considered as an error case and no UE requirement is imposed.

  - Option 3 (vivo): Send LS to RAN1 to check the prioritization rule for SRS antenna switching, especially for the case in CA/DC operation.

    - Option 3a (Ericsson, Nokia): Prioritization between scheduling of SRS antenna switching and transmission of certain signals and channels is to be handled by RAN1. If anything is unclear, RAN4 shall send LS to RAN1 and ask for clarification.

  - Option 4 (Huawei, Ericsson, Nokia, CATT): Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement.

- Recommended WF

  - Moderator: since companies mentioned such prioritization was also defined in RAN1, please companies to check with your RAN1 colleagues and then discuss if option 3a is agreeable or not.

  - Continue the discussion in 2nd round. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-3-1: Interruption requirement applicability

- Proposals

  - Option 1 (Apple, QC, MTK, Intel, OPPO): SRS antenna switching interruptions on both DL and UL applies to the band combinations signaled in txSwitchImpactToRx or txSwitchWithAnotherBand.

  - Option 2 (CATT, vivo, Xiaomi, Nokia, HW, Apple, Intel, Ericsson): txSwitchImpactToRx indicates the SRS antenna port switching impact to DL only, and txSwitchWithAnotherBand indicates the SRS antenna port switching impact to UL only.

Discussion in GTW (Aug 17):

- Chair: Common understanding that txSwitchImpactToRx indicates the SRS antenna port switching impact to DL only, and txSwitchWithAnotherBand indicates the SRS antenna port switching impact to UL only. No common understanding if interruptions should be limited to one direction (DL or UL) or both DL and UL. Recommend to continue discussion.

- Recommended WF

  - Continue the discussion in 2nd round. Companies to check whether option 2 is agreeable or not. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-3-2: Interruption requirement for UE with or without per-FR MG capability

- Proposals

  - Option 1 (CATT, Xiaomi, MTK, Ericsson, Nokia): The interruption requirements are defined only based on IEs of txSwitchImpactToRx or txSwitchWithAnotherBand indication. It is no need to clarify the relationship of the IEs with per-FR MG capability in RAN4 specification.

  - Option 2 (Apple, OPPO, MTK, Xiaomi, HW, Intel, Ericsson, vivo, CATT): No need to differentiate the requirement for the UE with or without capability of per-FR gap for SRS antenna port switching in RAN4. But the requirement applicability needs to be clarified that “for interruption caused by SRS antenna port switching, the victim CC would be based on indication of txSwitchImpactToRx or txSwitchWithAnotherBand regardless of per-FR MG capability”.

- Recommended WF

  - Continue the discussion in 2nd round. Please Nokia to check whether option 2 is agreeable. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-3-3: Would the interruption requirement differentiate between sync and async cases?

- Proposals

  - Option 1 (Apple, vivo, QC, Xiaomi, MTK, Intel, OPPO, HW, ZTE): No; one single requirement to cover the synchronous and asynchronous scenarios with or without UL TA.

    - Option 1a (Apple, OPPO): The unified interruption requirement is based on the async case for the minimum requirement.

    - Option 1b (LG): The same interruption length for both synchronous and asynchronous cases could be defined, and the interruption length should be reduced by one slot when the slot after SRS antenna post switching is downlink in synchronous case.

  - Option 2 (CATT, Ericsson, Nokia): Yes, the interruption requirement can differentiate between sync and async cases.

- Recommended WF

  - Moderator: Since this issue has been discussed for 3 meetings, could we compromise to start the work based on async case first (that’s the worst case for minimum requirement), and later if companies can prove there are big difference for sync case, we could define another set of interruption based on sync case.

  - Continue the discussion in 2nd round. Conclusions would be captured in the WF..

| Company | Comments |
| --- | --- |

Issue 1-3-4: txSwitchImpactToRx for intra-band case

- Proposals

  - Option 1 (Intel):

    - Since txSwitchImpactToRx can’t differentiate the intra-contiguous CA and non-contiguous CA case, there are two solutions:

      - Option 1: Send LS to RAN2 to modify the signaling.

      - Option 2: Solve the issue in RAN4 by updating the applicability.

  - Option 2 (CATT, Apple, Xiaomi, MTK, OPPO, Nokia, CATT, Ericsson)

    - No need to have clarification for txSwitchImpactToRx with intra-band contiguous CA and intra-band non-contiguous CA case.

- Recommended WF

  - Continue the discussion in 2nd round. Please Intel to check if can compromise to option 2. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-4-1: The interruption requirement is defined based on slot level or symbol level

- Proposals

  - Option 1 (CATT, Apple, vivo, QC, Xiaomi, MTK, Intel, HW, OPPO): based on slot level

    - Option 1a (CATT): The interruption requirements will be defined based on slot level, i.e. no requirement for the case of interruption on flexible slot. It is not necessary to further discuss on components of making interruption if the interruption requirements are defined based on slot level.

    - Option 1b (vivo): The interruption requirement is preferred to be defined based on slot level. The mis-alignment in the UL frame boundary between anchor and victim cells needs to be further discussed.

    - Option 1c(LG): Slot based interruption requirement could be used in synchronous and asynchronous case, and only UL symbols based interruption should be defined when SRS antenna port switching is configured in the flexible slot in synchronous case.

  - Option 2 (Ericsson, Nokia): based on symbol level

    - Option 2a (Nokia): The transient period before and after the SRS resource will cause UL interruption of 1 OFDM symbol at 60kHz. At 15kHz or 30kHz SCS, RAN4 should discuss if and how to capture the transient periods from RF session in the RRM specifications.

- Recommended WF

  - Continue the discussion in 2nd round. If possible Moderator will recommend to discuss on 2nd round GTW. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-4-2: The components within interruption time of SRS antenna port switching in FR1

- Proposals

  - Option 1 (CATT, Apple, QC, Xiaomi, CMCC, OPPO, HW, MTK, Intel): The composites of interruption requirement for SRS antenna port switching in FR1 include:

    - SRS Transmission time (use 6 symbols as minimum requirement), and

    - Antenna switching time before and after SRS transmission occasion (2*15us).

  - Option 2 (vivo):

    - For interruption requirements, the interruption time is preferred to include power adjustment period, antenna switching time and SRS transmission time.

    - RAN4 can consider to define interruption requirements for a limited set of SRS configuration in R17.

  - Option 3 (LG): The composites of interruption requirement for SRS antenna port switching in FR1 include:

    - Antenna port switching time (15usec) before the first SRS transmission

    - Time from the first SRS transmission to the last SRS transmission

    - Antenna port switching back time (15usec) after the last SRS transmission

      - It should be excluded when the SRS resource is configured in the last symbol of a slot and the next slot is downlink

  - Option 4 (Nokia, Ericsson):

    - When the SRS resources of a set are not configured in a slot, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE, where the uplink interruption comprises at least the SRS transmission period.

    - When the SRS resources of a set are configured in a slot with consecutive SRS transmission, the UE is allowed an uplink interruption on any of the active serving cells if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE, where the uplink interruption comprises at least the SRS transmission periods and the guard period in-between.

    - The position of the transient period needs to be clarified before defining the interruption and/or the scheduling restriction in RRM spec.

    - Add one note indicating the DL may be affected due to SRS antenna switching if txSwitchImpactToRx is configured.

  - Option 5 (Ericsson): The components within the interruption requirement shall include SRS transmission time for the configured number of symbols, as well as SRS antenna switching time before and after the SRS transmission.

- Recommended WF

  - Continue the discussion in 2nd round. If possible Moderator will recommend to discuss on 2nd round GTW. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-4-3: Interruption requirement proposals

- Recommended WF

  - Up to conclusions from other issues. Continue the discussion in 2nd round. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-5-1: Network to obtain the interrupted symbol info

- Proposals

  - Option 1 (vivo): If needed, RAN4 can study whether and how network can obtain the interrupted symbol information, when SRS antenna port switching is performed in another band.

  - Option 2 (MTK, Apple, HW, QC, CATT) : option 1 is not needed.

  - Option 3: Do not need to discuss this issue in R17 FeRRM WI.

- Recommended WF

  - Tentative agreements:

    - No need to discuss this issue in R17 FeRRM WI.

  - Companies to confirm on tentative agreement. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-5-2: Interruption requirement related with prioritization rule in RAN1

- Proposals

  - Option 1 (MTK, Apple, OPPO, vivo):

    - For SRS antenna port switching, define the interruption requirement for the case not covered by TS 38.214, e.g., add a note “if the case has no priority rule defined in TS 38.214, UE is allowed to cause the interruption on the other CC(s)”.

  - Option 2 (Nokia, Ericsson) :

    - The SRS transmission will cause UL interruption on the other carriers if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by the UE.

  - Option 3 (Huawei, Ericsson, CATT):

    - Not to define the rules to avoid collisions except what has been define in RAN1 and NR measurement.

- Recommended WF

  - Continue the discussion in 2nd round. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

Issue 1-5-3: Two SRS colliding on same symbol

- Proposals (MTK):

  - For SRS antenna port switching (FR1 only), RAN4 needs to further study the corresponding UE behavior when two SRS resources having the same time domain behavior, i.e., both SRS resources are periodic, semi-persistent or aperiodic, are scheduled on the same OFDM symbol:

    - Option1 (Apple): Two different SRS resources having the same time domain behavior to be scheduled on the same OFDM symbol are not expected.

    - Option2: Up to UE implementation.

    - Option3 (Nokia): This is up to RAN1 discussion.

- Recommended WF

  - Continue the discussion in 2nd round. Companies to check if option 3 can be a compromise. Conclusions would be captured in the WF.

| Company | Comments |
| --- | --- |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on further RRM enhancement for NR and MR-DC - SRS antenna port switching | Apple |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Apple | Jie Cui | Jie_cui@apple.com |
| LG | Jin-Yup Hwang | jinyup.hwang@lge.com |
| Ericsson | Joakim Axmon | joakim.axmon[at]ericsson.com |
| vivo | Yanliang SUN | yanliang.sun@vivo.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)