3GPP TSG RAN WG4 Meeting #100-e		R4-2115286

Electronic Meeting, August 16th - 27th, 2021

Agenda item:       6.1.1.6

Source: 	Nokia, Nokia Shanghai Bell

Title: 	WF on NR-U RRM performance requirements

Document for:     Discussion and Decision

# 1	Introduction

This document contains the agreements and discussions in RAN4#100-e in the email thread [100-e][207] NR_unlic_RRM_2 for the topics concerning general and test case specific RRM performance test configurations. The summary containing the complete discussion on that topic is found on the document [1].

Background information and WF documents from previous meetings are in the references.

The following colour code is used to highlight the agreements in this meeting:

Agreements in the GTW session

Agreements in the 1st round

Agreements in the 2nd round

# 2	CCA models

Avoiding LMAX in test cases with DRX

- Option 1: L increases on per DRX basis. At the end of each DRX cycle, if one of SMTC is not successfully transmitted during this DRX cycle, increase L by 1.

- Option 2: Improvements on CCA model for DRX might be still be discussed, while solution is not reached set the following parameters for test cases with DRX:

  - LCCA_DL=LCCA_UL=Not configured

  - WCCA_DL=WCCA_UL=Not configured

# 3	Test case specific configurations

## 3-1: RRC Connection Release with Redirection

Configuration of LCCA and WCCA  for RRC connection release with redirection test cases

- Configure CCA model with LCCA_DL=8 and WCCA_DL=Tidentify-NR_CCA for the test cases of RRC connection release with redirection under CCA.

## 3-2: 	SCell activation/deactivation

Configuration of LCCA and WCCA  for SCell activation/deactivation

- Configure CCA model with LCCA_DL=2 and WCCA_DL= Tactivation_time_withCCA for the test cases of SCell activation and deactivation test cases in non-DRX mode.

# References

- R4-2115381, Email discussion summary: [100-e][207] NR_unlic_RRM_2, Nokia, Nokia Shanghai Bell, RAN4#100-e.

- R4-2017089, “WF on NR-U RRM performance,” Nokia, Nokia Shanghai Bell, RAN4#97-e.

- R4-2103518, “WF on general test configurations for NR-U RRM performance requirements”, Nokia, Nokia Shanghai Bell, RAN4#98-e.

- R4-2104088    WF on LBT models for NR-U RRM performance requirements, Qualcomm, RAN4#98-e

- R4-2105709, “WF on general test configurations for NR-U RRM performance requirements”, Nokia, Nokia Shanghai Bell, RAN4#98-bis-e

- R4-2105710, WF on LBT models for NR-U RRM performance requirements, Qualcomm, RAN4#98-bis-e

- R4-2108259, WF on general test configurations for NR-U RRM performance requirements, Nokia, RAN4#99-e

- R4-2108260, WF on LBT models for NR-U RRM performance requirements, Qualcomm, RAN4#99-e