3GPP TSG RAN WG4 Meeting #100-e	                                                                  R4-2115340

Electronic Meeting, August 16-27, 2021

Agenda item:			9.11.2.1

Source:	Intel Corporation

Title:	WF on R17 NR MG enhancements – Pre-configured MG

Document for:	Information

# Agreements

Legends:

Agreements in the 1st round/GTW

Agreements in the 2nd round

Still open after 2nd round discussion

|  | Proposals |
| --- | --- |
| Issue#0-1 | #### Whether can the pre-MG be used for PRS measurement? - It is feasible to config Pre-MG for PRS measurements. - The exact configuration of Pre-MG used for PRS measurement can be FFS - FFS on whether UE assumes the Pre-MG is always activated - FFS if UE should always indicate serving cell about the PRS measurement when it is configured with pre-MG. - It is up to NW to configure either Pre-MG which shall be always activated or legacy MG for PRS measurement |
| Issue#0-2 | #### Whether can the pre-MG be used for CSI-RS L3 measurement? - It is feasible to configure a Pre-MG for CSI-RS L3 inter-frequency measurement. - The exact configuration of Pre-MG used for CSI-RS L3 inter-frequency measurement can be FFS - FFS on whether UE assumes the Pre-MG is always activated - It is up to NW to configure either Pre-MG which shall be always activated or legacy MG for CSI-RS L3 inter-frequency |
| Issue#1-1-1 | Parameter to differentiate the legacy MG and Pre-MG - The RRC parameter(s) used to differentiate Pre-MG with the legacy MG are needed when Pre-MG is being configured - The exact signalling can be designed by RAN2 |
| Issue#1-1-2/1-2 | #### Parameter to indicate the status (activation/deactivation) of Pre-MG at or during configuration - NW can control activation/deactivation of pre-configured MG - RRC based activation/deactivation method is supported - Network can indicate activation/deactivation status per BWP - Details of signalling are FFS - FFS if MAC CE based activation/deactivation method is supported - Companies are encouraged bring inputs on the conditions when NW based activation/deactivation and explicit rules for activation/deactivation are used. Note: This issue is related to the following agreement at RAN4#99-e, “Status of pre-configured MG is not fixed at RRC configuration”. |
| Issue#1-3 | Relation of pre-configured MG and with the current legacy MG - Without considering concurrent MG and on top of the RRC parameter(s) used to differentiate Pre-MG with the legacy MG, - FFS whether additional mechanism is needed to switch the configured pre-MG to serve as the same functionaries as the legacy MG (MG is always on). |
| Issue#2-1 | Criteria of activation/deactivation pre-configured MG - The explicit rules for pre-configured MG autonomous activation/deactivation shall be defined for the case when signalling is not provided - The general principle to define Pre-MG autotmous activation/deactivation criteria can be: - If MG is not required by any of the configured measurements, the MG is deactivated. - If MG is required by one or more of the configured measurements, the MG is activated - FFS the detailed criteria for pre-MG autonomous activation/deactivation - Option 1: - MO’s needs on the gap for the measurements(inter-f, inter-RAT, intra-f with gap) changed because of any operations below - BWP switching - adding/removing any measurement object(s), - adding/releasing/changing a PSCell, - activating/de-activating any Scell(s) - Option 2 : - The Pre-MG status (activation/deactivation) is triggered by active BWP switching as follows: - Pre-MG is activated if the BW of the SSB configured for measurement is not fully within the BW of the active BWP. - Pre-MG is deactivated if the BW of the SSB configured for measurement is fully within the BW of the active |
| Issue#2-1a | Conditions for pre-configured MG activation/deactivation - FFS if additional conditions beside BWP switching for pre-configured MG activation/deactivation shall be considered - Option 1: pre-configured MG activation/deactivation can be triggered by finishing the following network commands and procedures: BWP switching, adding/removing any measurement object(s), adding/releasing/changing a PSCell, activating/de-activating any SCell(s). - Option 2: pre-configured MG activation/deactivation is triggered ONLY by the DCI/Timer based BWP switch in accordance with the WID objective # 1. - Other options not precluded |
| Issue#2-2 | How pre-configured MGs can be activated/deactivated - The pre-configured MG activation/deactivation is triggered by the DCI/Timer based BWP switch - FFS if additional conditions for pre-configured MG activation/deactivation shall be considered - NW can control activation/deactivation of pre-configured MG - RRC based activation/deactivation method is supported - Network can indicate activation/deactivation status per BWP - Details of signalling are FFS - FFS if MAC CE based activation/deactivation method is supported - Additional explicit rules for pre-configured MG autonomous activation/deactivation shall be defined for the case when signalling is not provided - UE capability on the support of NW-controlled and autonomous pre-configured MG activation/deactivation mechanisms can be further discussed - Companies are encouraged bring inputs on the conditions when NW based activation/deactivation and explicit rules for activation/deactivation are used |
| Issue#3-1 | Activation/Deactivation Delay Additional transition time (T = TBD) on top of the BWP switching delay shall be included in the pre-configured MG activation/deactivation time. |
| Issue#3-2 | Measurement period FFS in the future meetings for the general principle to define the measurement period e.g.: - Applicability - Main components which shall be included in the measurement report period - Others |
| Issue 3-3 | UE behavior after deactivation of pre-configured MG Without considering concurrent gap, after Pre-MG being deactivated, UE shall perform measurement without MG and be able to receive and transmit in the serving cell. |
| Issue#4-1 | MG patterns used for the pre-configured MG mechanism All existing MG patterns #0~25 in Rel-16 are applicable for the pre-configured MG |