3GPP TSG-RAN WG4 Meeting # 100-e 						R4-2115355

Electronic Meeting, 16th – 27th Aug, 2021

Agenda item:			9.9.4

Source:	Samsung

Title:	WF for FeMIMO RRM

Document for:	Approval

Introduction

This contribution is to capture the agreements for the email discussion for Rel-17 FeMIMO RRM in RAN4 #100 meeting

Way-forward

RRM requirements impact

- No impact on RRM requirement for

  - Enhancements for PDCCH, PUCCH and PUSCH for multi-TRP

  - CSI enhancement

- No RRM requirement will be defined for 8 RX antenna ports

- Further study whether to define RRM requirements and RRM impacts for simultaneous reception of channel/RS with different QCL type D

  - RAN4 is supposed to conclude whether to define RRM requirements and RRM impacts for simultaneous reception of channel/RS with QCL type D in RAN4 #101-e

Unified TCI

- Specify TCI switching delay requirements for

  - Joint TCI with UL and DL

  - Separate TCI for UL

  - FFS: TCI for DL

- Specify the requirements for PL-RS update under unified TCI framework

Inter-cell beam management

- RAN4 needs to specify the intra-frequency L1-RSRP measurement requirements for non-serving cells

- RAN4 needs to specify the intra-frequency L1-RSRP measurement accuracy requirements for non-serving cells

- For non-serving L1-RSRP measurement of single panel FR2 UE, requirements will be applied if UE only measure L1-RSRP from one single cell at a time.

- To guarantee UE’s mobility performance, RAN4 shall agree that PCell/PSCell’s L3-RSRP measurement delay shall not be impacted by NSC measurements. FFS UE measurement behaviour for L1-RSRP.

- RAN4 will further study if UE only performs L1-RSRP measurements on the identified non-serving cell(s)

QCL definition

- RAN4 will further study QCL definition update for PUCCH and PUSCH in applicability of requirements