3GPP TSG-RAN WG4 Meeting #100-e	R4-2115361

Electronic Meeting, August 16-27, 2021

Title: 	Simulation assumptions for RedCap RLM and BFD performance

Source: 	Ericsson

Agenda item:	9.20.3.2

Document for:	Approval

# Introduction

The work item on NR support of reduced capability NR devices has been approved [1]. This work item contains an objective on UE complexity reduction as shown below [1].

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz. - Reduced minimum number of Rx branches: - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands. - A means shall be specified by which the gNB can know the number of Rx branches of the UE. - ….. - Duplex operation: - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

This contribution contains simulation assumptions to evaluate the RLM and BFD performance for release 17 RedCap UE with 1 Rx.

# Simulation Assumption for RLM and BFD evaluations

Table 1: RLM and BFD simulation parameters for SS block based on RLM and BFD

| Parameters | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Carrier frequency | 4GHz | 30GHz |
| Subcarrier Spacing for SS block | 15kHz, 30kHz | 120kHz |
| Number of transmitted SS block within a SS burst set period (K) | 1 |  |
| Reference signals | SSS |  |
| Transmission period of SS block | 20ms |  |
| Number of samples in L1 evaluation period | 5 samples (Same as Rel-15 RLM Qin and BFD) 10 samples (Same as Rel-15 RLM Qout) 20 samples Other samples are not precluded |  |
| Transmit antennas | 1 |  |
| Receive antennas | 1 |  |
| Propagation conditions | FR1: AWGN TDL-A 30ns TDL-B 100ns TDL-C 300ns FR2: AWGN TDL-A 30ns |  |
| UE speed | FR1: 30 km/h FR2: 3 km/h |  |
| CP type | Normal CP |  |
| SNR | [-14, -13, ., 1, 0] dB |  |

Table 2: RLM and BFD simulation parameters for CSI-RS based on RLM and BFD

| Parameter | Unit | Value |
| --- | --- | --- |
| Bandwidth |  | 24 RBs |
| SCS | kHz | 15, 30, 120 |
| Number of CSI-RS ports (X) |  | 1 |
| CDM Type |  | ‘No CDM’ |
| Density (ρ) |  | 3 |
| CSI-RS periodicity | slots | 20 for 15 kHz 40 for 30 kHz 80 for 120 kHz |
| Number of samples in L1 evaluation period |  | 10 samples (Same as Rel-15 RLM Qin and BFD) 20 samples (Same as Rel-15 RLM Qout) 40 samples Other samples are not precluded |
| EPRE ratio to SSS | dB | 0 |
| Propagation conditions |  | FR1: AWGN TDL-A 30ns TDL-B 100ns TDL-C 300ns FR2: AWGN TDL-A 30ns |
| UE speed |  | FR1: 30 km/h FR2: 3 km/h |
| CP type |  | Normal CP |
| Receive antennas |  | 1 |
| SNR | dB | [-14, -13, ., 1, 0] |

# Performance Metric

The UE performs signal and interference measurements based on RLM-RS or BFD-RS (i.e. SS block or CSI-RS) to derive a downlink link quality metric (e.g. SINR) as same as Rel-15 RLM/BFD.

The CDF curves are to be provided for measurement errors for downlink link quality metric:

•	Delta SINR = (estimated SINR – ideal SINR) 	[dB]

Interest companies are encouraged to evaluate the measurement accuracy impact due to 1Rx.

# Reference

- RP-211038, Revised WID on support of reduced capability NR devices, Ericsson