3GPP TSG-RAN WG4 Meeting # 100-e 												R4-2115385

Electronic Meeting, 16th – 27th August, 2021

Agenda item:			6.1.6.2.3

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [100-e][211] NR_pos_3

Document for:	Information

# Introduction

The document contains discussion related to the RRM performance requirements for gNB positioning measurements:

The document contains the following four main topics:

- Topic #1: General aspects (Agenda item: 6.1.6.2.3.1)

- Topic #2: SRS-RSRP requirements (Agenda item: 6.1.6.2.3.2)

- Topic #3: gNB Rx-Tx time difference requirements (Agenda item: 6.1.6.2.3.3)

# Topic #1: General aspects

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2114044 | Ericsson | Summary of link level simulation results of SRS RSRP and gNB TOA |

## Open issues summary

### Sub-topic 1-1: Link level simulation results

Issue 1-1-1: Link level simulation results for SRS-RSRP

- Proposal:

  - Option 1: Ericsson

    - Updated link simulation results

  - Option 2: None

- Recommended WF

  - Further discuss the PRS-RSRP results for updating accuracy

Issue 1-1-2: Link level simulation results for gNB Rx-Tx time difference

- Proposal:

  - Option 1: Ericsson

    - Updated link simulation results

  - Option 2: None

- Recommended WF

  - Further discuss the gNB Rx-Tx time difference link results for updating accuracy

## Companies views’ collection for 1st round

### Open issues

Sub-topic 1-1: Issue 1-1-1: Link level simulation results for SRS-RSRP

| Company | Comments |
| --- | --- |
| Ericsson | Updated link simulation results show no changes for derived SRS-RSRP measurement accuracies as depicted in our draft CR, where only brackets have been removed. |
| Huawei | Fine with option 1 and same observation as Ericsson above. |
| Nokia | We have also the same observation. |

Sub-topic 1-1: Issue 1-1-2: Link level simulation results for gNB Rx-Tx time difference

| Company | Comments |
| --- | --- |
| Ericsson | Updated link simulation results show some changes for derived gNB Rx-Tx time difference measurements. It is proposed to combine companies’ new results and derive final values implementing averaging and addition of additional margin as per outcome of issue 3-1-2. |
| Huawei | Fine with option 1 and same observation as Ericsson above. |
| Nokia | As informed on the server, we have included our simulation results for the new lower boundaries of some BW ranges in the simulation collection contribution. These results were already submitted to RAN4#99-e in R4-2110273 (before the agreement); for SCS=60 kHz, we did not observe a change of the results related to previous limits. We propose to take these added results into account. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic 1-1 | Issue 1-1-1: Link level simulation results for PRS-RSRP Tentative agreements: The latest companies’ link simulation results show that the existing SRS-RSRP measurement accuracies in TS 38.133 v16.8.0 do not need any modification. Recommendations for 2nd round: None. |

|  | Status summary |
| --- | --- |
| Sub-topic 1-1 | Issue 1-1-2: Link level simulation results for gNB Rx-Tx time difference Tentative agreements: The latest companies’ link simulation results shown that for some SRS BW ranges the existing gNB Rx-Tx time difference measurement accuracies in TS 38.133 v16.8.0 need to be updated. Recommendations for 2nd round: None. |

### CRs/TPs

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Topic #2: SRS-RSRP requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2114045 | Ericsson | - Simulation results collected in RAN4#99-e can be used to define SRS-RSRP measurement accuracy requirements, since no outliers can be identified in the further simulation results - RF calibration margin differs between gNB type 1-C and other gNB types: - X= 2.5 dB for gNB type 1-C - X= 4 dB for gNB type 1-H, 1-O and 2-O - Proposal 3 Table 13.3.2.2-1 gNB SRS-RSRP absolute accuracy requirements in FR1 for gNB type 1-C Accuracy; Conditions. SRS Ês/Iot; SRS bandwidth range. dB; dB; RB. ±4; Ês/Iot ≥ +3; 24 ≤ BW < 48. ±4; 48 ≤ BW < 132. ±4; 132 ≤ BW. ±6.5; Ês/Iot ≥ -13; 48 ≤ BW < 132. ±5.5; 132 ≤ BW. Table 13.3.2.2-2 gNB SRS-RSRP absolute accuracy requirements in FR1 for gNB type 1-H and 1-O Accuracy; Conditions. SRS Ês/Iot; SRS bandwidth range. dB; dB; RB. ±5.5; Ês/Iot ≥ +3; 24 ≤ BW < 48. ±5.5; 48 ≤ BW < 132. ±5.5; 132 ≤ BW. ±8; Ês/Iot ≥ -13; 48 ≤ BW < 132. ±7; 132 ≤ BW. Table 13.3.2.2-3 gNB SRS-RSRP absolute accuracy requirements in FR2 for gNB type 2-O Accuracy; Conditions. SRS Ês/Iot; SRS bandwidth range. dB; dB; RB. ±5.5; Ês/Iot ≥ +3; 32 ≤ BW < 64. ±5.5; 64 ≤ BW < 132. ±5.5; 132 ≤ BW. ±8; Ês/Iot ≥ -13; 64 ≤ BW < 132. ±7; 132 ≤ BW. |
| R4-2114047 | Ericsson | Cat A CR on Corrections to gNB SRS-RSRP requirements |
| R4-2114048 | Ericsson | - Cat F CR on Corrections to gNB SRS-RSRP requirements |
| R4-2114294 | Huawei, HiSilicon | Cat F CR: Remove [] in the SRS-RSRP requirements |
| R4-2114295 | Huawei, HiSilicon | Cat A CR: Remove [] in the SRS-RSRP requirements |

## Open issues summary

### Sub-topic 2-1: Updates to SRS-RSRP accuracy requirements

Issue 2-1-1: SRS BW grouping for defining SRS-RSRP accuracy requirements

- Proposals

  - Option 1: Ericsson, Huawei

    - SRS-RSRP accuracy requirements in TS 38.133 are updated as follows by removing brackets:

Table 13.3.2.2-1 gNB SRS-RSRP absolute accuracy requirements in FR1 for gNB type 1-C

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | SRS Ês/Iot | SRS bandwidth range |
| dB | dB | RB |  |
| ±4 | Ês/Iot ≥ +3 | 24 ≤ BW < 48 |  |
| ±4 |  | 48 ≤ BW < 132 |  |
| ±4 |  | 132 ≤ BW |  |
| ±6.5 | Ês/Iot ≥ -13 | 48 ≤ BW < 132 |  |
| ±5.5 |  | 132 ≤ BW |  |

Table 13.3.2.2-2 gNB SRS-RSRP absolute accuracy requirements in FR1 for gNB type 1-H and 1-O

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | SRS Ês/Iot | SRS bandwidth range |
| dB | dB | RB |  |
| ±5.5 | Ês/Iot ≥ +3 | 24 ≤ BW < 48 |  |
| ±5.5 |  | 48 ≤ BW < 132 |  |
| ±5.5 |  | 132 ≤ BW |  |
| ±8 | Ês/Iot ≥ -13 | 48 ≤ BW < 132 |  |
| ±7 |  | 132 ≤ BW |  |

Table 13.3.2.2-3 gNB SRS-RSRP absolute accuracy requirements in FR2 for gNB type 2-O

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | SRS Ês/Iot | SRS bandwidth range |
| dB | dB | RB |  |
| ±5.5 | Ês/Iot ≥ +3 | 32 ≤ BW < 64 |  |
| ±5.5 |  | 64 ≤ BW < 132 |  |
| ±5.5 |  | 132 ≤ BW |  |
| ±8 | Ês/Iot ≥ -13 | 64 ≤ BW < 132 |  |
| ±7 |  | 132 ≤ BW |  |

  - Option 2: None

- Recommended WF

  - Further discuss if option 1 is agreeable (removal of brackets around SRS-RSRP accuracy in 38.133)

## Companies views’ collection for 1st round

### Open issues

Sub-topic 2-1:  Updates to SRS-RSRP accuracy requirements

| Company | Comments |
| --- | --- |
| Ericsson | Option 1. |
| Huawei | Option 1. |
| Nokia | Option 1. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2114048 (Ericsson) | Agreeable |
|  | Nokia: Agreeable. On the title page, for the field ‘Consequences if not approved’: “Incorrect requirements” is not correct, it should be rather “incomplete requirements”, as we have pre-liminary ones. In the field ‘Clauses affected’: This should be only 13.3.2.2. |
| R4-2114294 (Huawei) | Agreeable |
|  | Nokia: Agreeable. On the title page for the field ‘Clauses affected’: This should be 13.3.2.2 rather than 13.3.2. Then there are spelling errors for “requirements” (3 occurences). |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic 2-1 | Updates to SRS-RSRP accuracy requirements: Tentative agreements: Existing SRS-RSRP accuracy requirements in TS 38.133 v16.8.0 are updated by removing the brackets. Recommendations for 2nd round: None |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2115317 (Rev of R4-2114294) (Huawei) | Nokia: The revised draft CR is agreeable. |

# Topic #3: gNB Rx-Tx time difference requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2114046 | Ericsson | Take into account new data points from simulation results for lowest RB number of bandwidth group to define gNB Rx-Tx measurement accuracy requirements Add half of standard deviation rounded to the next integer as additional margin Table 13.2.2.2-1: gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 1-C, 1-H and 1-O Accuracy; SRS Ês/Iot; SCS; SRS bandwidth range. Unit: Tc; Unit: dB; Unit: kHz; Unit: RB. [66]; ≥ -13; 15; 44 ≤ BW ≤ 84. [33]; 88 ≤ BW ≤ 168. [15]; 176 ≤ BW. 122; ≥ +3; 24 ≤ BW ≤ 40. [60]; 44 ≤ BW ≤ 84. [31]; 88 ≤ BW ≤ 168. [15]; 176 ≤ BW. 42; ≥ -13; 30; 48 ≤ BW ≤ 84. [15]; 88 ≤ BW ≤ 168. [8]; 176 ≤ BW. 32; ≥ +3; 48 ≤ BW ≤ 84. [15]; 88 ≤ BW ≤ 168. [8]; 176 ≤ BW. 21; ≥ -13; 60; 48 ≤ BW ≤ 84. [9]; 88 ≤ BW. 16; ≥ +3; 48 ≤ BW ≤ 84. [8]; 88 ≤ BW. Table 13.2.2.2-2: gNB Rx-Tx time difference absolute accuracy in FR2 for gNB type 2-O Accuracy; SRS Ês/Iot; SCS; SRS bandwidth range. Unit: Tc; Unit: dB; Unit: kHz; Unit: RB. 9; ≥ -13; 60; 132 ≤ BW ≤ 168. [6]; 176 ≤ BW. 9; ≥ +3; 132 ≤ BW ≤ 168. [6]; 176 ≤ BW. 22; ≥ -13; 120; 32 ≤ BW ≤ 40. [10]; 44 ≤ BW ≤ 84. [6]; 88 ≤ BW. 16; ≥ +3; 32 ≤ BW ≤ 40. [8]; 44 ≤ BW ≤ 84. [6]; 88 ≤ BW. |
| R4-2114049 | Ericsson | Cat A CR on Corrections to gNB SRS-RSRP requirements |
| R4-2114050 | Ericsson | - Cat F CR on Corrections to gNB Rx-Tx time requirements |
| R4-2114296 | Huawei, HiSilicon | Proposal 1: Update the gNB Rx-Tx accuracy requirements based on the simulation results for the lower bounds of SRS BW ranges (i.e. 44 RBs, 88 RBs, 176 RBs). Proposal 2: Add some margin in the accuracy requirements for cases where results from companies are with std deviation is >= [10]Tc. |
| R4-2114297 | Huawei, HiSilicon | Cat F CR to update gNB Rx-Tx time difference requirements |
| R4-2114298 | Huawei, HiSilicon | - Cat A CR to update gNB Rx-Tx time difference requirements |

## Open issues summary

### Sub-topic 3-1: Updates to gNB Rx-Tx accuracy requirements

Issue 3-1-1: SRS BW ranges for updating gNB Rx-Tx accuracy requirements

- Proposals: Companies proposals on list of BW ranges for SCS and at SRS Ês/Iot for which the existing gNB Rx-Tx accuracies in TS 38.133 are to be updated:

  - Option 1: Ericsson

Table 13.2.2.2-1: gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 1-C, 1-H and 1-O

| SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- |
| Unit: dB | Unit: kHz | Unit: RB |
| ≥ -13 | 15 | 44 ≤ BW ≤ 84 |
|  |  | 88 ≤ BW ≤ 168 |
| ≥ +3 |  | 24 ≤ BW ≤ 40 |
| ≥ -13 | 30 | 48 ≤ BW ≤ 84 |
| ≥ +3 |  | 48 ≤ BW ≤ 84 |
| ≥ -13 | 60 | 48 ≤ BW ≤ 84 |
|  |  | 88 ≤ BW |
| ≥ +3 |  | 48 ≤ BW ≤ 84 |

Table 13.2.2.2-2: gNB Rx-Tx time difference absolute accuracy in FR2 for gNB type 2-O

| SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- |
| Unit: dB | Unit: kHz | Unit: RB |
| ≥ -13 | 60 | 132 ≤ BW ≤ 168 |
| ≥ +3 |  | 132 ≤ BW ≤ 168 |
| ≥ -13 | 120 | 32 ≤ BW ≤ 40 |
|  |  | 44 ≤ BW ≤ 84 |
| ≥ +3 |  | 32 ≤ BW ≤ 40 |

  - Option 2: Huawei

Table 13.2.2.2-1: gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 1-C, 1-H and 1-O

| SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- |
| Unit: dB | Unit: kHz | Unit: RB |
| ≥ -13 | 15 | 44 ≤ BW ≤ 84 |
|  |  | 88 ≤ BW ≤ 168 |
| ≥ +3 |  | 24 ≤ BW ≤ 40 |
| ≥ -13 | 30 | 48 ≤ BW ≤ 84 |
|  |  | 88 ≤ BW ≤ 168 |
|  |  | 88 ≤ BW |

Table 13.2.2.2-2: gNB Rx-Tx time difference absolute accuracy in FR2 for gNB type 2-O

| SRS Ês/Iot | SCS | SRS bandwidth range |
| --- | --- | --- |
| ≥ -13 | 120 | 44 ≤ BW ≤ 84 |

- Recommended WF

  - Further discuss the proposals

Issue 3-1-2: Principles for updating gNB Rx-Tx accuracy requirements

- Proposals: Companies proposals on principles for deriving the updated gNB Rx-Tx accuracies:

  - Option 1: Ericsson

    - Take into account new data points from simulation results for lowest RB number of bandwidth group to define gNB Rx-Tx measurement accuracy requirements

    - Add half of standard deviation rounded to the next integer as additional margin.

    - Proposed accuracy figures in CR in R4-2114050.

  - Option 2: Huawei

    - Update the gNB Rx-Tx accuracy requirements based on the simulation results for the lower bounds of SRS BW ranges (i.e. 44 RBs, 88 RBs, 176 RBs).

    - Add some margin in the accuracy requirements for cases where results from companies are with std deviation is >= [10]Tc.

    - Proposed accuracy figures in CR in R4-2114297.

- Recommended WF

  - Further discuss the proposals

## Companies views’ collection for 1st round

### Open issues

Sub-topic 3-1:  Issue 3-1-1: SRS BW ranges for updating gNB Rx-Tx accuracy requirements

| Company | Comments |
| --- | --- |
| Ericsson | The SRS configurations that need to be updated per option 1 or option 2 respectively are based on the individual companies’ results. Therefore, we suggest compiling the results and check which values need to be updated accordingly based on averaged values (incl. or excl. additional margin based on proceedings in issue 3-1-2). Preliminarily, the compiled results show: Accuracy @ 90% (Tc), SINR >= -13dB; Accuracy @ 90% (Tc), SINR >= 3dB. FR; SCS(kHz); SRS BW (PRBs); Ericsson; Huawei; Nokia; Average; Std.Dev; Ericsson2; Huawei2; Nokia2; Average2; Std.Dev2. FR1; 15; 24; 2816; 110; 113; 128; 117; 10. FR1; 15; 44; 66; 157; 112; 65; 58; 57; 58; 1. FR1; 15; 88; 33; 56; 45; 17; 30; 29; 30; 1. FR1; 15; 176; 15; 15; 15; 0; 15; 14; 15; 1. FR1; 30; 48; 31; 48; 32; 37; 10; 30; 29; 32; 31; 2. FR1; 30; 88; 15; 27; 21; 9; 15; 15; 15; 0. FR1; 30; 176; 8; 8; 8; 0; 7; 8; 8; 1. FR1; 60; 48; 18; 21; 16; 19; 3; 15; 14; 16; 15; 1. FR1; 60; 88; 9; 13; 11; 3; 8; 7; 8; 1. FR2; 60; 132; 8; 7; 8; 8; 1; 8; 7; 8; 8; 1. FR2; 60; 176; 5; 4; 5; 1; 4; 4; 4; 0. FR2; 120; 32; 25; 15; 16; 19; 6; 15; 14; 16; 15; 1. FR2; 120; 44; 10; 18; 14; 6; 8; 7; 8; 1. FR2; 120; 88; 5; 6; 6; 1; 4; 4; 4; 0. |
| Huawei | Same comment as Ericsson above, i.e. we can check which cases are to be updated based on the averaged results, and we suggest to consider the margin for identifying the cases. |
| Nokia | We propose to take into account our results previously submitted to RAN4 #99-e, as commented for sub-topic 1-1. For the margin, we support going along option 1 and then discuss the few other cases separately. |
| Ericsson | Incorporating Nokia’s results as shared on the reflector we arrive at the following table for combined results: Accuracy @ 90% (Tc), SINR >= -13dB; Accuracy @ 90% (Tc), SINR >= 3dB. FR; SCS(kHz); SRS BW (PRBs); Ericsson; Huawei; Nokia; Average; Std.Dev; Ericsson2; Huawei2; Nokia2; Average2; Std.Dev2. FR1; 15; 24; 110; 113; 128; 117; 10. FR1; 15; 44; 66; 157; 64; 96; 54; 58; 57; 64; 60; 4. FR1; 15; 88; 33; 56; 32; 41; 14; 30; 29; 32; 31; 2. FR1; 15; 176; 15; 15; 16; 16; 1; 15; 14; 16; 15; 1. FR1; 30; 48; 31; 48; 32; 37; 10; 30; 29; 32; 31; 2. FR1; 30; 88; 15; 27; 16; 20; 7; 15; 15; 16; 16; 1. FR1; 30; 176; 8; 8; 8; 8; 0; 7; 8; 8; 8; 1. FR1; 60; 48; 18; 21; 16; 19; 3; 15; 14; 16; 15; 1. FR1; 60; 88; 9; 13; 8; 10; 3; 8; 7; 8; 8; 1. FR2; 60; 132; 8; 7; 8; 8; 1; 8; 7; 8; 8; 1. FR2; 60; 176; 5; 4; 8; 6; 3; 4; 4; 8; 6; 3. FR2; 120; 32; 25; 15; 16; 19; 6; 15; 14; 16; 15; 1. FR2; 120; 44; 10; 18; 8; 12; 6; 8; 7; 8; 8; 1. FR2; 120; 88; 5; 6; 8; 7; 2; 4; 4; 8; 6; 3. |

Sub-topic 3-1:  Issue 3-1-2: Principles for updating gNB Rx-Tx accuracy requirements

| Company | Comments |
| --- | --- |
| Ericsson | Option 1. As seen in issue 3-1-1 in the preliminary combined result collection table for gNB Rx-Tx, the standard deviation is mostly small except for some corner cases. When adding additional margin as per option 2, what is the threshold to decide when to add the margin and furthermore, how much margin is being added? Since adding margin depended on the actual std. dev. value is a reasonable approach and adding 0 or 1Tc where std. dev. is small, we suggest adding margin based on option 1. There, the extreme cases get handled appropriately and for cases with low std. dev., not much of an impact is created in terms of size of additional margin in relation to the average value. Applying option 1 with combined and averaged simulation results from all companies (as per issue 3-1-1) would lead to the following tables for a revision of the respective draft CR: Table 13.2.2.2-1: gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 1-C, 1-H and 1-O Accuracy; SRS Ês/Iot; SCS; SRS bandwidth range. Unit: Tc; Unit: dB; Unit: kHz; Unit: RB. 145; ≥ -13; 15; 44 ≤ BW ≤ 84. 54; 88 ≤ BW ≤ 168. 15; 176 ≤ BW. 122; ≥ +3; 24 ≤ BW ≤ 40. 59; 44 ≤ BW ≤ 84. 31; 88 ≤ BW ≤ 168. 16; 176 ≤ BW. 42; ≥ -13; 30; 48 ≤ BW ≤ 84. 26; 88 ≤ BW ≤ 168. 8; 176 ≤ BW. 32; ≥ +3; 48 ≤ BW ≤ 84. 15; 88 ≤ BW ≤ 168. 9; 176 ≤ BW. 21; ≥ -13; 60; 48 ≤ BW ≤ 84. 13; 88 ≤ BW. 16; ≥ +3; 48 ≤ BW ≤ 84. 9; 88 ≤ BW. Table 13.2.2.2-2: gNB Rx-Tx time difference absolute accuracy in FR2 for gNB type 2-O Accuracy; SRS Ês/Iot; SCS; SRS bandwidth range. Unit: Tc; Unit: dB; Unit: kHz; Unit: RB. 9; ≥ -13; 60; 132 ≤ BW ≤ 168. 6; 176 ≤ BW. 9; ≥ +3; 132 ≤ BW ≤ 168. 4; 176 ≤ BW. 22; ≥ -13; 120; 32 ≤ BW ≤ 40. 17; 44 ≤ BW ≤ 84. 7; 88 ≤ BW. 16; ≥ +3; 32 ≤ BW ≤ 40. 9; 44 ≤ BW ≤ 84. 4; 88 ≤ BW. |
| Huawei | We are also fine with option 1 which is more systematic than option 2. |
| Nokia | We propose to take into account our results previously submitted to RAN4 #99-e, as commented for sub-topic 1-1. For the margin, we support going along option 1 and then discuss the few other cases separately. |
|  | Considering updates to issue 3-1-1 we can update the tables for a possible CR revision as following: Table 13.2.2.2-1: gNB Rx-Tx time difference absolute accuracy in FR1 for gNB type 1-C, 1-H and 1-O Accuracy; SRS Ês/Iot; SCS; SRS bandwidth range. Unit: Tc; Unit: dB; Unit: kHz; Unit: RB. 123; ≥ -13; 15; 44 ≤ BW ≤ 84. 48; 88 ≤ BW ≤ 168. 17; 176 ≤ BW. 122; ≥ +3; 24 ≤ BW ≤ 40. 62; 44 ≤ BW ≤ 84. 32; 88 ≤ BW ≤ 168. 16; 176 ≤ BW. 42; ≥ -13; 30; 48 ≤ BW ≤ 84. 24; 88 ≤ BW ≤ 168. 8; 176 ≤ BW. 32; ≥ +3; 48 ≤ BW ≤ 84. 17; 88 ≤ BW ≤ 168. 9; 176 ≤ BW. 21; ≥ -13; 60; 48 ≤ BW ≤ 84. 12; 88 ≤ BW. 16; ≥ +3; 48 ≤ BW ≤ 84. 9; 88 ≤ BW. Table 13.2.2.2-2: gNB Rx-Tx time difference absolute accuracy in FR2 for gNB type 2-O Accuracy; SRS Ês/Iot; SCS; SRS bandwidth range. Unit: Tc; Unit: dB; Unit: kHz; Unit: RB. 9; ≥ -13; 60; 132 ≤ BW ≤ 168. 8; 176 ≤ BW. 9; ≥ +3; 132 ≤ BW ≤ 168. 8; 176 ≤ BW. 22; ≥ -13; 120; 32 ≤ BW ≤ 40. 15; 44 ≤ BW ≤ 84. 8; 88 ≤ BW. 16; ≥ +3; 32 ≤ BW ≤ 40. 9; 44 ≤ BW ≤ 84. 8; 88 ≤ BW. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2114050 (Ericsson) | Some updates are furthermore updated as per issues 3-1-1 and 3-1-2. |
|  | Nokia: Update due to issues 3-1-1 and 3-1-2 is needed. On the title page, the field “Clauses affected “ should just contain subclause 13.2.2.2 |
| R4-2114297 (Huawei) | Needs revision based on same comment as for E/// draft CR |
|  | Nokia: Update due to issues 3-1-1 and 3-1-2 is needed. On the title page, the field “Clauses affected “ should just contain subclause 13.2.2.2 |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic 3-1 | Issue 3-1-1: SRS BW ranges for updating gNB Rx-Tx accuracy requirements Tentative agreements: Candidate options: Merged with issue 3-1-2. Recommendations for 2nd round: None |
| Sub-topic 3-1 | Issue 3-1-2: Principles for updating gNB Rx-Tx accuracy requirements Tentative agreements: gNB Rx-Tx accuracy requirements in TS 38.133 v16.8.0 are updated for certain SRS BW ranges based on the following principles: - Take into account new data points from simulation results for lowest RB number of bandwidth group to define gNB Rx-Tx measurement accuracy requirements. - Add half of standard deviation rounded to the next integer as additional margin. Recommendations for 2nd round: Discuss the draft CR updating the gNB Rx-Tx accuracy based on the above tentative agreements. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2115318 (Rev of R4-2114050) (Ericsson) | Nokia: The revised draft CR is agreeable. |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on gNB positioning measurement requirements | Ericsson |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2114048 | Corrections to gNB SRS-RSRP measurement in 38.133 | Ericsson | Not Pursued | Merged with R4-2114294 |
| R4-2114294 | CR to update SRS-RSRP requirements | Huawei, HiSilicon | Revised |  |
| R4-2114050 | Corrections to gNB Rx-Tx measurement in 38.133 | Ericsson | Revised |  |
| R4-2114297 | CR to update gNB Rx-Tx time difference requirements | Huawei, HiSilicon | Not Pursued | Merged with R4-2114050. |
| R4-2114044 | Summary of link level simulation results of SRS RSRP and gNB TOA | Ericsson | Revised | To compile results from all companies |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2115316 | WF on gNB positioning measurement requirements | Ericsson | Agreeable |  |
| R4-2115317 (rev of R4-2114294) | CR to update SRS-RSRP requirements | Huawei, HiSilicon | Agreeable |  |
| R4-2115318 (Rev of R4-2114050) | Corrections to gNB Rx-Tx measurement in 38.133 | Ericsson | Agreeable |  |
| R4-2115319 (Rev of R4-2114044) | Summary of link level simulation results of SRS RSRP and gNB TOA | Ericsson | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Dominik Frank | dominik.frank@ericsson.com |
| Nokia | Juergen Hofmann | juergen.hofmann@nokia.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)