3GPP TSG-RAN WG4 Meeting # 100-e 												R4-2115406

Electronic Meeting, 16th – 27th August 2021

Agenda item:			9.16.7.4, 9.16.7.5

Source:	Moderator (Intel)

Title:	Email discussion summary for [100-e][231] NR_ext_to_71GHz_RRM_2

Document for:	Information

# Introduction

In RAN #89e meeting a new WI on Extending current NR operation to 71GHz was approved. During the RAN4 #99 the initial scope of RRM work for NR_ext_to_71GHz WI was defined and captured in the way forward R4-2108354. A number of topics which need the new requirements definition due to higher data/SSB SCS were defined. Among them there were Active BWP switching delay requirements and MG interruption requirements. These two topics corresponding to agenda items 9.16.7.4 and 9.16.7.5 are the subject of current email discussion.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Companies are expected to provide views and/or comments on the listed open issues.

- 2nd round: Continue discussion on the open issues.

For the Email discussion guidelines please refer to the Meeting Arrangements document provided by RAN4 chair before the meeting.

# Topic #1: Active BWP switching

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112136 | Apple | Proposal 1: Use the values in brackets below for 480/960kHz SCS as a starting point: NR Slot length; BWP switch delay TBWPswitchDelay (slots). (ms); Type 1Note 1; Type 2Note 1. 0; 1; 1; 3. 1; 0.5; 2; 5. 2; 0.25; 3; 9. 3; 0.125; 6; 18. 5; 0.03125; [20]; [72]. 6; 0.015625; [39]; [144]. Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch.. |
| R4-2112561 | vivo | Proposal 1: Depending on UE capability, UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 3. Table 3: BWP switch delay NR Slot length; BWP switch delay TBWPswitchDelay (slots). (ms); Type 1Note 1; Type 2Note 1. 0; 1; 1; 3. 1; 0.5; 2; 5. 2; 0.25; 3; 9. 3; 0.125; 6; 18. 5; 0.03125; 20; 65. 6; 0.015625; 39; 129. Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch.. Observation 1: RAN4 should study if shorter BWP switch time can be considered for 52.6G-71GHz. |
| R4-2113223 | Nokia, Nokia Shanghai Bell | Observation 1: BWP switching delay requirements are defined based on a minimum UE delay of 600 us for Type 1 UEs and 2000 us for Type 2 UEs. Observation 2: If we keep the same BWP delay in ms for 480 kHz and 960 kHz SCS, the BWP delay would be 64 and 128 slots for Type 2 UEs. Proposal 1: RAN4 to study if BWP delay reduction for Type 2 UEs is possible for the operation on 480 and 960 kHz SCS. Proposal 2: Adopt the following values for DCI and timer based BWP switch delay as baseline BWP switch delay TBWPswitchDelay (slots); BWP switch delay in ms. NR Slot length (ms); Type 1; Type 2; Type 1; Type 2. 5; 0.03125; 20; 64; 0.625; 2.0. 6; 0.015625; 39; 128; 0.609375; 2.0. |
| R4-2113335 | Ericsson | Proposal 1: Active BWP switch delay for 52.6-71GHz still follows 600us for type 1 UE and 200us for type 2 UE. Proposal 2: Table of Active BWP switch delay is updated with higher SCS’s following assumption in Proposal1. Table 8.6.2-1: BWP switch delay NR Slot length; BWP switch delay TBWPswitchDelay (slots). (ms); Type 1Note 1; Type 2Note 1. 0; 1; 1; 3. 1; 0.5; 2; 5. 2; 0.25; 3; 9. 3; 0.125; 6; 18. 4; 0.03125; [20]; [65]. 5; 0.015625; [39]; [129]. Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch.. |
| R4-2114145 | Huawei, Hisilicon | Proposal 1: To define the BWP switching requirements for 480 kHz and 960 kHz as follows Table 8.6.2-1: BWP switch delay NR Slot length; BWP switch delay TBWPswitchDelay (slots). (ms); Type 1Note 1; Type 2Note 1. 0; 1; 1; 3. 1; 0.5; 2; 5. 2; 0.25; 3; 9. 3; 0.125; 6; 18. 5; 0.03125; 24; 72. 6; 0.015625; 48; 144. Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch.. Proposal 2: Reuse the existing requirements for RRC-based BWP switch. |
| R4-2114191 | Intel Corporation | Proposal 1: RAN4 to use the same assumptions for FR2-2 BWP switching delay definition as in Rel-15, i.e. 600us and 2000us switching delay for Type 1 and Type 2 respectively plus 3 OFDM symbols carrying DCI. Proposal 2: For new SCSs RAN4 to define BWP switching delay (Table 8.6.2-1) as shown in Table 1. Table 1: BWP switch delay NR Slot length; BWP switch delay TBWPswitchDelay (slots). (ms); Type 1Note 1; Type 2Note 1. 0; 1; 1; 3. 1; 0.5; 2; 5. 2; 0.25; 3; 9. 3; 0.125; 6; 18. 5; 0.03125; 20; 65. 6; 0.015625; 39; 129. Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch.. Proposal 3: The existing requirements for BWP Switching on Multiple CCs can be applied for new SCSs |

## Open issues summary

### Sub-topic 1-1. Active BWP switching delay requirements

Sub-topic description: This sub-topic discusses Active BWP switching delay and corresponding requirements for new SCSs 480 kHz and 960 kHz

Open issues and candidate options before e-meeting:

Issue 1-1-1: BWP switching delay for FR2-2

- Proposals

  - Proposal 1 (Ericsson, Intel): Follow 600us and 2000us switching delay for Type 1 and Type 2 respectively

  - Proposal 2 (vivo): RAN4 to study if shorter BWP can be considered for 52.6G-71GHz.

  - Proposal 2a (Nokia): RAN4 to study if BWP delay reduction for Type 2 UEs is possible for the operation on 480 and 960 kHz SCS.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | To our understanding, it is rational to keep 600us and 2000us switching delay for Type 1 and Type 2 unless BWP switching delay is unacceptable. |
| MTK | Prefer to Option 1 to keep existing 600us and 2000us. |
| Nokia | We agree with the proposals as a baseline but would also like to keep the option open for studying the reduction of BWP switching delay. So we propose to change the text as: - As baseline, follow 600us and 2000us switching delay for Type 1 and Type 2 respectively - FFS: if BWP delay reduction for Type 2 UEs is possible for the operation on 480 and 960 kHz SCS. |
| LG | Support to proposal 1 to keep existing switching delay |
| vivo | Support Proposal 2. We agree with Proposal 1 as a baseline. However, based on Type 2 UE, the maximum BWP switch delay is 129 slots. It makes the feature not useable in practical network. We see the value to re-consider the BWP switching delay for FR2-2. |
| Qualcomm | We support proposal 1. Existing 600us and 2000us can be used. |
| Apple | Agree with Proposal 1. |
| Huawei | Support option 1. |
| CMCC | Proposal 2 and 3. Support to study if BWP switching delay can be reduced for FR20-2. |
| Intel | Support Proposal 1. Proposals 2/2a are also valid. We can agree with the text provided by Nokia in their comment. |
| OPPO | Existing 600us and 2000us (in option 1) can be used, but not precluding other type of BWP switching delay, e.g., type 3? Still open to study the feasibility of fast BWP switch for higher frequency (option 2). |

Issue 1-1-2: Number of slots for DCI and timer based BWP switch delay for Type 1 UEs for 480kHz and 960kHz respectively

- Proposals

  - Option 1 (Apple, vivo, Nokia, Ericsson, Intel): 20 and 39

  - Option 2 (Huawei): 24 and 48

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | We followed the same mathematical procedure as before. |
| Nokia | Agree with Option 1. |
| LG | Support option 1 |
| vivo | Support Option 1. The BWP switching delay for Type 1 UE equals to 600us (switching delay) plus 3 OFDM symbols carrying DCI. |
| Qualcomm | Option 1 seems correct based on 600us delay assumption. |
| Apple | Either option1 or option 2 is acceptable to us |
| Huawei | We would like to further clarify the motivation of option 2. It is scaled based on the number of slots of 120 KHz. The consideration is that whether we should also consider the cross carrier scheduling here. In previous RAN4 meeting, one additional slot is allowed for cross carrier scheduling. But for FR2-2, the BWP switching time is based on some absolute time (600/2000) but the addition delay for cross scheduling is not. For the extremely case, only one slot of 960 KHz is allowed which apparently is not enough. |
| CMCC | Option 1 |
| Intel | Support option 1. |
| OPPO | Option 1 is fine |

Issue 1-1-3: Number of slots for DCI and timer based BWP switch delay for Type 2 Ues for 480kHz and 960kHz respectively

- Proposals

  - Option 1 (Apple, Huawei): 72 and 144

  - Option 2 (vivo, Ericsson, Intel): 65 and 129

  - Option 3 (Nokia): 64 and 128

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Support Option 2. We believe the difference of 1 from Nokia is related to the rounding up/rounding operation that was used. If that’s the case, we believe rounding up is the better option. |
| Nokia | We agree with Option 2 as a baseline, but would rather make it clear that value can be reviewed according to the outcome of Issue 1-1-1. Our proposal is to say Option 4: Use as baseline 65 and 129, FFS: if BWP delay reduction for Type 2 Ues is possible for the operation on 480 and 960 kHz SCS. |
| LG | Support option 2 |
| vivo | Support Option 2. The BWP switching delay for Type 2 UE equals to 2000us (switching delay) plus 3 OFDM symbols carrying DCI. |
| Qualcomm | Option 2 looks correct based on 2000us assumption |
| Apple | Either option1 or option 2 is acceptable to us |
| Huawei | Same comments in 1-1-3 for cross carrier scheduling case. |
| CMCC | Option 2 |
| Intel | Support Option 2. And we agree with Nokia that the values may change if different switching delay will be introduced for Type 2 UEs. |
| OPPO | Option 2 is fine. |

Issue 1-1-4: RRC-based BWP switch

- Proposals

  - Proposal 1 (Huawei): Reuse the existing requirements for RRC-based BWP switch

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Agree with proposal 1. |
| Nokia | Agree with proposal 1 |
| vivo | Fine with Proposal 1. |
| Qualcomm | Agree with proposal 1 |
| Apple | Agree with Proposal 1 |
| CMCC | Proposal 1 |
| Intel | Agree with Proposal 1 |
| OPPO | Agree with Proposal 1 |

Issue 1-1-5: BWP Switching on Multiple CCs

- Proposals

  - Proposal 1 (Intel): The existing requirements for BWP Switching on Multiple CCs can be applied for new SCSs

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Agree with proposal 1. |
| MTK | Suggest to focus on one BWP first |
| Nokia | We are fine with proposal 1, but we don’t have a problem on focusing on one BWP first as suggested by MTK. |
| Qualcomm | We are fine with proposal 1 and agree with MTK’s comment too. |
| Apple | Agree with MTK that we can discuss the case of multiple CCs later |
| Huawei | Suggest to focus on one BWP first. |
| CMCC | OK with other companies’ comments. Focus on single CC at first. |
| Intel | Support Proposal 1. |
| OPPO | Fine with proposal 1 and agree with MTK’s comment. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1: Active BWP switching | Issue 1-1-1: BWP switching delay for FR2-2 Companies’ views: Companies have the same views to consider BWP switching delay equal to 600us and 2000us for Type 1 and Type 2 as a baseline. There are proposals to study if the delay reduction is possible for FR2-2 since long delay makes the feature not useable in practical network. Tentative agreements: - Agreements from GTW session (August 20th): - As baseline, follow 600us and 2000us switching delay for Type 1 and Type 2 respectively - FFS: if BWP delay reduction is possible for the operation on 480 and 960 kHz SCS. Candidate options: Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 1-1-2: Number of slots for DCI and timer based BWP switch delay for Type 1 UEs for 480kHz and 960kHz respectively Companies’ views: Most of the companies are aligned about the procedure of derivation of number of slots for BWP switch delay – consider switching delay plus 3 OFDM symbols carrying DCI. One company suggested also to consider cross-carrier scheduling and extend BWP switch delay by several slots considering scaling the existing requirements. Additionally, Option 3 with was proposed during the GTW session. Tentative agreements: N/A Candidate options: - Option 1 (Apple, vivo, Nokia, Ericsson, Intel, LGE, QC, CMCC, OPPO): 20 and 39 - Option 2 (Huawei, Apple): 24 and 48 - Option 3 (MTK): 22 and 41 Recommendations for 2nd round: Continue discussion in the 2nd round |
|  | Issue 1-1-3: Number of slots for DCI and timer based BWP switch delay for Type 2 Ues for 480kHz and 960kHz respectively Companies’ views: Same as for Issue 1-1-2 Tentative agreements: N/A Candidate options: - Option 1 (Apple, Huawei): 72 and 144 - Option 2 (vivo, Ericsson, Intel, Nokia, Apple): 65 and 129 Recommendations for 2nd round: Continue discussion in the 2nd round |
|  | Issue 1-1-4: RRC-based BWP switch Companies’ views: All the companies agreed with the proposal to reuse the existing requirements for RRC-based BWP switch Tentative agreements: Reuse the existing requirements for RRC-based BWP switch Candidate options: Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 1-1-5: BWP Switching on Multiple CCs Companies’ views: Most companies are fine with the proposal that the existing requirements for BWP Switching on Multiple CCs can be applied for new SCSs. At the same time companies prefer to focus on one BWP first. Tentative agreements: Candidate options: Recommendations for 2nd round: Postpone the discussion for the later stage |

## Discussion on 2nd round

### Sub-topic 1-1. Active BWP switching delay requirements

Issue 1-1-2: Number of slots for DCI and timer based BWP switch delay for Type 1 UEs for 480kHz and 960kHz respectively

- Proposals

  - Option 1 (Apple, vivo, Nokia, Ericsson, Intel, LGE, QC, CMCC, OPPO): 20 and 39

  - Option 2 (Huawei, Apple): 24 and 48

  - Option 3 (MTK): 22 and 41

- Recommended WF

  - Continue the discussion in the 2nd round.

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| Ericsson | Support Option 1. Option1 aims to add related columns in <Table 8.6.2-1: BWP switch delay> for higher SCS. Cross scheduling which needs 1 more slot can follow CR: R4-201732 and doesn’t impact <Table 8.6.2-1: BWP switch delay> . |
| vivo | Support Option 1. |
| LG | Support option 1 |
| MTK | After check, support for Option 1. The cross carrier scheduling prospective can be FFS. |
| Intel | Support Option 1 |
| Nokia | We still think it is Option 1. As part of this discussion, Huawei commented that Option 2 includes cross-carrier scheduling. We think that this issue related only to single CC and that Huawei’s comment would only apply for the Multiple CC case. It is clear that multiple CCs are not considered when defining requirements for single CC, as can be noted from 8.6.2 in 38.133 ### 8.6.2 DCI and timer based BWP switch delay on a single CC The requirements in this clause only apply to the case that the BWP switch is performed on a single CC with more than one BWP configurations configured.. In order to make the topic clearer, we propose to change the text of Option 1 as : - Option 1a: Number of slots for DCI and timer based BWP switch delay on a Single CC for Type 1 UEs is - 20 for 480kHz SCS - 39 for 960kHz SCS |
| Qualcomm | Support option 1 |
| Apple | Ok with option 1 for the single CC case |
| Huawei | Ok with option 1 for single CC |

Issue 1-1-3: Number of slots for DCI and timer based BWP switch delay for Type 2 UEs for 480kHz and 960kHz respectively

- Proposals

  - Option 1 (vivo, Ericsson, Intel, Nokia, Apple): 65 and 129

  - Option 2 (Apple, Huawei): 72 and 144

- Recommended WF

  - Continue the discussion in the 2nd round.

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| Ericsson | Support Option 1. Reason is same to Issue 1-1-2 |
| vivo | Support Option 1. |
| LG | Support option 1 |
| Intel | Support Option 1 |
| Nokia | Similarly as in the previous issue, we still think it is Option 1. As part of this discussion, Huawei commented that Option 2 includes cross-carrier scheduling. We think that this issue related only to single CC and that Huawei’s comment would only apply for the Multiple CC case. It is clear that multiple CCs are not considered when defining requirements for single CC, as can be noted from 8.6.2 in 38.133 ### 8.6.2 DCI and timer based BWP switch delay on a single CC The requirements in this clause only apply to the case that the BWP switch is performed on a single CC with more than one BWP configurations configured.. In order to make the topic clearer, we propose to change the text of Option 1 as : - Option 1a: Number of slots for DCI and timer based BWP switch delay on a Single CC for Type 2 Ues is - 65 for 480kHz SCS - 129 for 960kHz SCS |
| Qualcomm | Support Option 1 |
| Apple | Ok with option 1 |
| Huawei | OK with option 1 with single CC |

## Summary for 2nd round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #1-1: Active BWP switching | Issue 1-1-2: Number of slots for DCI and timer based BWP switch delay for Type 1 UEs for 480kHz and 960kHz respectively Companies’ views: All the companies reached the consensus. An issue raised during the 1st round discussion related to the cross carrier scheduling would only apply for the Multiple CC case, while the Issue 1-1-2 is related to single CC case. Agreement: 20 and 39 on a Single CC Moderator comments In the WF the agreement was combined with agreement from issue 1-1-3 in a table as it was initially present in the companies’ contributions. |
|  | Issue 1-1-3: Number of slots for DCI and timer based BWP switch delay for Type 2 Ues for 480kHz and 960kHz respectively Companies’ views: All the companies reached the consensus. An issue raised during the 1st round discussion related to the cross carrier scheduling would only apply for the Multiple CC case, while the Issue 1-1-3 is related to single CC case. Agreement: 65 and 129 on a Single CC Moderator comments In the WF the agreement was combined with agreement from issue 1-1-2 in a table as it was initially present in the companies’ contributions. |

# Topic #2: Measurement gaps and measurement gap interruption requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2112562 | vivo | Proposal 1: RAN4 should reuse 0.25ms as MG timing advance for 52.6-71GHz. Proposal 2: RAN4 should modify NOTE 2 for the case with per-UE gap or per-FR gap for FR1 from 52.6GHz to 71GHz. Proposal 3: Measurement gap interruption requirements shall be shown as Table 1-3 to support 480kHz and 960kHz. Table 1: Total number of interrupted slots on all serving cells during MGL for Synchronous EN-DC, NR standalone operation (with single carrier, NR CA and synchronous NR-DC configuration) and NE-DC, and on all serving cells in MCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1 NR; Total number of interrupted slots on serving cells. SCS; When MG timing advance of 0ms is applied; When MG timing advance of 0.5ms is applied. (kHz); MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms; MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms. 15; 20; 10; 6; 4; 3; 21Note3; 11Note3; 7Note3; 5Note3; 4Note3. 30; 40; 20; 12; 8; 6; 40; 20; 12; 8; 6. 60; 80; 40; 24; 16; 12; 80; 40; 24; 16; 12. 120; 160; 80; 48; 32; 24; 160; 80; 48; 32; 24. 480; 640; 320; 192; 128; 96; 640; 320; 192; 128; 96. 960; 1280; 640; 384; 256; 192; 1280; 640; 384; 256; 192. NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz ,480 kHz and 960 kHz is only applicable to the case with per-UE measurement gap. NOTE 3: Non-overlapped half-slots occur before and after the measurement gap. Whether a Rel-15 UE can receive and/or transmit in those half-slots is up to UE implementation.. Table 2: Total number of interrupted slots on serving cells during MGL for Asynchronous EN-DC, and on all serving cells in SCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1 NR; Total number of interrupted slots on serving cells. SCS; When MG timing advance of 0ms is applied; When MG timing advance of 0.5ms is applied. (kHz); MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms; MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms. 15; 21; 11; 7; 5; 4; 21; 11; 7; 5; 4. 30; 41; 21; 13; 9; 7; 41; 21; 13; 9; 7. 60; 81; 41; 25; 17; 13; 81; 41; 25; 17; 13. 120; 161; 81; 49; 33; 25; 161; 81; 49; 33; 25. 480; 641; 321; 193; 129; 97; 641; 321; 193; 129; 97. 960; 1281; 641; 385; 257; 193; 1281; 641; 385; 257; 193. NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz ,480 kHz and 960 kHz is only applicable to the case with per-UE measurement gap.. Table 3: Total number of interrupted slots on FR2 serving cells during MGL for EN-DC, NR standalone operation (with single carrier, NR CA and NR-DC configuration) and NE-DC with per-UE measurement gap or per-FR measurement gap for FR2 NR; Total number of interrupted slots on FR2 serving cells. SCS; When MG timing advance of 0ms is applied; When MG timing advance of 0.25ms is applied. (kHz); MGL= 20ms; MGL= 10ms; MGL= 5.5ms; MGL= 3.5ms; MGL= 1.5ms; MGL= 20ms; MGL= 10ms; MGL= 5.5ms; MGL= 3.5ms; MGL= 1.5ms. 60; 80; 40; 22; 14; 6; 80; 40; 22; 14; 6. 120; 160; 80; 44; 28; 12; 160; 80; 44; 28; 12. 480; 640; 320; 176; 112; 48; 640; 320; 176; 112; 48. 960; 1280; 640; 352; 224; 96; 1280; 640; 352; 224; 96. NOTE 1: The total number of interrupted slots is based on that SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR2 serving cell. NOTE 2: Slot occurs before or after the measurement gap may be interrupted additionally if SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR1 serving cell.. |
| R4-2113224 | Nokia, Nokia Shanghai Bell | Observation 1: When considering measurement gaps, it is not clear whether the per-FR gaps should consider FR2-1 and FR2-2 as independent bands. Proposal 1: RAN4 to study if per-FR measurement gaps for FR2-1 and FR2-2 should be defined independently. |
| R4-2113337 | Ericsson | Proposal 1: There are 26 different measurement gap patterns (ID # 0 to # 25). Some of these gap patterns (ID #12 to # 23) are applicable to only FR2. While the existing gaps are also feasible for higher SCS (e.g., 480 kHz and 960 kHz) applicable for 52.6 – 71 GHz. Proposal 2: For per-FR measurement gap capable UE in NR standalone operation (with single carrier, NR CA and NR-DC configuration), for per-FR gap-based measurement, when there is no serving cell in a particular FR, where measurement objects are configured, regardless if explicit per-FR measurement gap is configured in this FR, the effective MGRP in this FR is used to determine requirements: - 20 ms for FR2 NR measurements also is applicable for 52.6 – 71 GHz. Proposal 3: Table 9.1.2-4, Table 9.1.2-4a and Table 9.1.2-4b need to include higher SCS’s for 52.6 – 71 GHz. Table 9.1.2-4: Total number of interrupted slots on all serving cells during MGL for Synchronous EN-DC, NR standalone operation (with single carrier, NR CA and synchronous NR-DC configuration) and NE-DC, and on all serving cells in MCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1 NR; Total number of interrupted slots on serving cells. SCS; When MG timing advance of 0ms is applied; When MG timing advance of 0.5ms is applied. (kHz); MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms; MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms. 15; 20; 10; 6; 4; 3; 21Note3; 11Note3; 7Note3; 5Note3; 4Note3. 30; 40; 20; 12; 8; 6; 40; 20; 12; 8; 6. 60; 80; 40; 24; 16; 12; 80; 40; 24; 16; 12. 120; 160; 80; 48; 32; 24; 160; 80; 48; 32; 24. 480; 640; 320; 192; 128; 96; 640; 320; 192; 128; 96. 960; 1280; 640; 384; 256; 192; 1280; 640; 384; 256; 192. NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz is only applicable to the case with per-UE measurement gap. NOTE 3: Non-overlapped half-slots occur before and after the measurement gap. Whether a Rel-15 UE can receive and/or transmit in those half-slots is up to UE implementation.. Table 9.1.2-4a: Total number of interrupted slots on serving cells during MGL for Asynchronous EN-DC, and on all serving cells in SCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1 NR; Total number of interrupted slots on serving cells. SCS; When MG timing advance of 0ms is applied; When MG timing advance of 0.5ms is applied. (kHz); MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms; MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms. 15; 21; 11; 7; 5; 4; 21; 11; 7; 5; 4. 30; 41; 21; 13; 9; 7; 41; 21; 13; 9; 7. 60; 81; 41; 25; 17; 13; 81; 41; 25; 17; 13. 120; 161; 81; 49; 33; 25; 161; 81; 49; 33; 25. 480; 641; 321; 193; 129; 97; 641; 321; 193; 129; 97. 960; 1281; 641; 385; 257; 193; 1281; 641; 385; 257; 193. NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz is only applicable to the case with per-UE measurement gap.. Table 9.1.2-4b: Total number of interrupted slots on FR2 serving cells during MGL for EN-DC, NR standalone operation (with single carrier, NR CA and NR-DC configuration) and NE-DC with per-UE measurement gap or per-FR measurement gap for FR2 NR; Total number of interrupted slots on FR2 serving cells. SCS; When MG timing advance of 0ms is applied; When MG timing advance of 0.25ms is applied. (kHz); MGL= 20ms; MGL= 10ms; MGL= 5.5ms; MGL= 3.5ms; MGL= 1.5ms; MGL= 20ms; MGL= 10ms; MGL= 5.5ms; MGL= 3.5ms; MGL= 1.5ms. 60; 80; 40; 22; 14; 6; 80; 40; 22; 14; 6. 120; 160; 80; 44; 28; 12; 160; 80; 44; 28; 12. 480; 640; 320; 176; 112; 48; 640; 320; 176; 112; 48. 960; 1280; 640; 352; 224; 96; 1280; 640; 352; 224; 96. NOTE 1: The total number of interrupted slots is based on that SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR2 serving cell. NOTE 2: Slot occurs before or after the measurement gap may be interrupted additionally if SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR1 serving cell.. |
| R4-2114146 | Huawei, Hisilicon | Observation 1: The length of SSB bursts could be much shorted for SCS of 480 kHz and 960 kHz. Proposal 1: The switching time for FR2-2 is assumed be same as FR2-1 as 0.25 ms. Proposal 2: RAN4 to discuss whether to have new gap pattern with more RAN1 conclusion on SSB distribution. Proposal 3: The impact of MRTD should be considered when defining the requirements for gap interruptions. |
| R4-2112488 | MediaTek Inc. | Proposal 5: The existing FR2-1 measurement gap patterns can be reused for FR2-2. Proposal 6: RAN4 further to discuss whether to introduce independent gap for FR2-2. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: Measurement gaps applicability

Sub-topic description: This sub-topic discusses the measurement gaps applicability for FR2-2

Open issues and candidate options before e-meeting:

Issue 2-1-1: Existing measurement gap patterns applicability

- Proposals

  - Option 1 (Ericsson, MediaTek): The existing gaps are also feasible for higher SCS

  - Option 2 (Huawei): RAN4 to discuss whether to have new gap pattern with more RAN1 conclusion on SSB distribution.

- Recommended WF

  - Continue the discussion in the 1st round.

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Support Option 1. Discussion on symbols of SSB indexes in RAN1 doesn’t impact half-frame structure of SSB which is relevant to MGL. |
| MTK | Support option 1. |
| Nokia | Agree with Option 1. |
| LG | Support option 1 |
| vivo | Support option 1. In current requirements, there exists 3 different measurement gap lengths, namely 5.5us, 3.5us and 1.5us for FR2. Although the slot length is shorter for 480kHz and 960kHz, the total number of SSBs occupy 2ms and 1ms, respectively, which can be covered by existing gaps. |
| Qualcomm | Support option 1. |
| Apple | Given the increased SSB SCS, we wonder if new gap patterns with smaller MGL should be studied in order to reduce impact on throughput |
| Huawei | No strong views on this. We are fine to go with option 1. |
| CMCC | Option 1 is OK. Meanwhile, RAN4 can also study whether new gap patterns with smaller MGL can be introduced. |
| Intel | Support Option 1 |
| OPPO | Open to further discuss if any progress in RAN1 |

Issue 2-1-2: Per-FR measurement gaps for FR2-1 and FR2-2

- Proposals

  - Option 1 (Nokia, MediaTek): RAN4 to study if per-FR measurement gaps for FR2-1 and FR2-2 should be defined independently.

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Support Option 1. We can discuss it further if it’s really necessary. |
| MTK | Support option 1. |
| Nokia | We would like to verify the assumptions on the per-FR measurement gaps with other companies, in order to determine if a FR2-2 per_FR measurement gap should be defined or if the measurement gap should be for the whole FR2 (FR2-1 + FR2-2) |
| LG | Support option 1. |
| Vivo | It may be not necessary to separate FR2-1 from FR2-2 for per-FR measurement gaps. In our understanding FR2-1 and FR2-2 would be implemented with single RF IC. |
| Qualcomm | We don’t see a need to separately define per-FR MGs for FR2-1 and FR2-2. Open to discuss only if it’s really necessary. |
| Apple | Agree with option 1 |
| Huawei | Further investigation is needed. |
| CMCC | OK with option1. More understanding on the UE implementation for FR2-1 and FR2-2 is needed. |
| Intel | Support Option 1. |
| OPPO | Option 1 is fine |

Issue 2-1-3: Effective MGRP used to determine requirements

- Proposals

  - Option 1 (Ericsson): For per-FR measurement gap capable UE in NR standalone operation, for per-FR gap-based measurement, when there is no serving cell in a particular FR, where measurement objects are configured, regardless if explicit per-FR measurement gap is configured in this FR, reuse value for FR2 NR measurements – 20ms

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Our intention is to retain the same requirements for FR2. |
| Nokia | We are ok with Option 1 |
| vivo | This follows existing requirements and should be fine. |
| Qualcomm | We are fine with option 1 |
| Apple | Option 1 is ok |
| CMCC | Option 1 is OK |
| Intel | We are fine with Option 1 |
| OPPO | Option 1 is fine. The principle of effective MGRP can be reused. |

### Sub-topic 2-2: Measurement gap interruption requirements for FR2-2

Sub-topic description: This sub-topic discusses the aspects related to measurement gap interruption requirements definition for FR2-2

Open issues and candidate options before e-meeting:

Issue 2-2-1: MG timing advance for FR2-2

- Proposals

  - Option 1 (vivo, Huawei): 0.25ms

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Agree with Option 1, it reuses requirement for FR2. |
| Nokia | Agree with Option 1. 0.25 ms is used for FR2 due to RF retuning time, |
| vivo | Support Proposal 1. |
| Qualcomm | Fine with option 1 |
| Huawei | Support option 1. |
| CMCC | Option 1 is OK |
| Intel | Agree with Option 1 |
| OPPO | Fine with option 1 |

Issue 2-2-2: High SCS in the case with per-UE gap or per-FR gap for FR1

- Proposals

  - Option 1 (vivo): RAN4 should modify NOTE 2 in Table 9.1.2-4 and Table 9.1.2-4a to capture that 480 kHz and 960 kHz are only applicable to the case with per-UE measurement gap

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Agree with option 1. |
| Nokia | Agree with Option 1. This is important to make clear that that requirement if for per-UE measurement gap, and not per-FR in FR1. |
| Vivo | Support Proposal 1. For the case of per-UE gap and per-FR gap for FR1, 120kHz, 480kHz and 960kHz is only applicable to the case with per-UE measurement gap. |
| Qualcomm | Fine with option 1. |
| Apple | Option 1 is ok |
| Huawei | Fine with Option 1. |
| CMCC | OK with option 1 |
| Intel | Support Option 1 |
| OPPO | Option 1 is fine. The clarification in vivo’s comment is clearer. |

Issue 2-2-3: MRTD impact on gap interruptions

- Proposals

  - Option 1 (Huawei): The impact of MRTD should be considered when defining the requirements for gap interruptions

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | We can check MRTD’s impact after its conclusion for FR2-2 in RRM_1 thread. |
| Nokia | Parallel discussion happening on thread 230 should influence the decision on Option 1 |
| vivo | Is this only for FR2-2 scenario or legacy FR2-1 also to be considered? In our opinion, the legacy FR2-1 requirements for gap interruption may be affected by MRTD. |
| Apple | Agree with Option 1 that impact of MRTD needs to be considered |
| Huawei | Suggest to come back after conclusion [230] |
| CMCC | Discuss later when MRTD is agreed. |
| Intel | We agree that MRTD should be considered when defining the requirements for gap interruptions. However, to our understanding it is already covered in the spec in the requirements for CA with non-aligned frame boundaries |
| OPPO | Open to discuss the MRTD impact. |

Issue 2-2-4: Measurement gap interruption requirements

- Proposals

  - Option 1 (vivo, Ericsson): Table 9.1.2-4, Table 9.1.2-4a and Table 9.1.2-4b to include higher SCS’s for 52.6 – 71 GHz as shown below

Table 9.1.2-4: Total number of interrupted slots on all serving cells during MGL for Synchronous EN-DC, NR standalone operation (with single carrier, NR CA and synchronous NR-DC configuration) and NE-DC, and on all serving cells in MCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1

| NR | Total number of interrupted slots on serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.5ms is applied |  |  |  |  |
| (kHz) | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms |
| 15 | 20 | 10 | 6 | 4 | 3 | 21Note3 | 11Note3 | 7Note3 | 5Note3 | 4Note3 |
| 30 | 40 | 20 | 12 | 8 | 6 | 40 | 20 | 12 | 8 | 6 |
| 60 | 80 | 40 | 24 | 16 | 12 | 80 | 40 | 24 | 16 | 12 |
| 120 | 160 | 80 | 48 | 32 | 24 | 160 | 80 | 48 | 32 | 24 |
| 480 | 640 | 320 | 192 | 128 | 96 | 640 | 320 | 192 | 128 | 96 |
| 960 | 1280 | 640 | 384 | 256 | 192 | 1280 | 640 | 384 | 256 | 192 |
| NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz ,480 kHz and 960 kHz is only applicable to the case with per-UE measurement gap. NOTE 3: Non-overlapped half-slots occur before and after the measurement gap. Whether a Rel-15 UE can receive and/or transmit in those half-slots is up to UE implementation. |  |  |  |  |  |  |  |  |  |  |

Table 9.1.2-4a: Total number of interrupted slots on serving cells during MGL for Asynchronous EN-DC, and on all serving cells in SCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1

| NR | Total number of interrupted slots on serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.5ms is applied |  |  |  |  |
| (kHz) | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms | MGL=20ms | MGL=10ms | MGL=6ms | MGL=4ms | MGL=3ms |
| 15 | 21 | 11 | 7 | 5 | 4 | 21 | 11 | 7 | 5 | 4 |
| 30 | 41 | 21 | 13 | 9 | 7 | 41 | 21 | 13 | 9 | 7 |
| 60 | 81 | 41 | 25 | 17 | 13 | 81 | 41 | 25 | 17 | 13 |
| 120 | 161 | 81 | 49 | 33 | 25 | 161 | 81 | 49 | 33 | 25 |
| 480 | 641 | 321 | 193 | 129 | 97 | 641 | 321 | 193 | 129 | 97 |
| 960 | 1281 | 641 | 385 | 257 | 193 | 1281 | 641 | 385 | 257 | 193 |
| NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz ,480 kHz and 960 kHz is only applicable to the case with per-UE measurement gap. |  |  |  |  |  |  |  |  |  |  |

Table 9.1.2-4b: Total number of interrupted slots on FR2 serving cells during MGL for EN-DC, NR standalone operation (with single carrier, NR CA and NR-DC configuration) and NE-DC with per-UE measurement gap or per-FR measurement gap for FR2

| NR | Total number of interrupted slots on FR2 serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0ms is applied |  |  |  |  | When MG timing advance of 0.25ms is applied |  |  |  |  |
| (kHz) | MGL= 20ms | MGL= 10ms | MGL= 5.5ms | MGL= 3.5ms | MGL= 1.5ms | MGL= 20ms | MGL= 10ms | MGL= 5.5ms | MGL= 3.5ms | MGL= 1.5ms |
| 60 | 80 | 40 | 22 | 14 | 6 | 80 | 40 | 22 | 14 | 6 |
| 120 | 160 | 80 | 44 | 28 | 12 | 160 | 80 | 44 | 28 | 12 |
| 480 | 640 | 320 | 176 | 112 | 48 | 640 | 320 | 176 | 112 | 48 |
| 960 | 1280 | 640 | 352 | 224 | 96 | 1280 | 640 | 352 | 224 | 96 |
| NOTE 1: The total number of interrupted slots is based on that SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR2 serving cell. NOTE 2: Slot occurs before or after the measurement gap may be interrupted additionally if SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR1 serving cell. |  |  |  |  |  |  |  |  |  |  |

- Recommended WF

  - Discuss the proposal

Companies views’ collection for 1st round:

| Company | Comments |
| --- | --- |
| Ericsson | Support Option1, we extended the tables with 480KHz and 960Khz. |
| Nokia | We are fine with option 1. |
| Vivo | Support Option1. |
| Qualcomm | Fine with option 1 |
| CMCC | OK with option1 |
| Intel | We are fine with Option 1 |
| OPPO | Fine with option 1 |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1: Measurement gaps applicability | Issue 2-1-1: Existing measurement gap patterns applicability Companies’ views: Most of the companies agree that the existing gaps are also feasible for higher SCS. Some companies are open for further discussion of new gap patterns with smaller MGL Tentative agreements: - The existing gaps are also feasible for higher SCS Candidate options: - No need to introduce new gap patterns - RAN4 to study if new gap patterns with smaller MGL can be introduced. Recommendations for 2nd round: Continue discussion in the 2nd round |
|  | Issue 2-1-2: Per-FR measurement gaps for FR2-1 and FR2-2 Companies’ views: Most of the companies mentioned that such study is needed and with more understanding on the UE implementation for FR2-1 and FR2-2. Two companies mentioned that separation is not needed. Thread [230] has an agreement on the scenarios prioritization, FR2-1 + FR2-2 scenarios are out of scope. Need to check whether the discussion on FR2-1 and FR2-2 separation is still relevant. Tentative agreements: N/A Candidate options: - Option 1 (Ericsson, MediaTek, Nokia, LG, Apple, Huawei, CMCC, Intel, OPPO): RAN4 to study if per-FR measurement gaps for FR2-1 and FR2-2 should be defined independently. - Option 2 (vivo, QC): no need to separate FR2-1 from FR2-2 for per-FR measurement gaps Recommendations for 2nd round: Continue discussion in the 2nd round considering the thread [230] agreement not to focus on FR2-1 + FR2-2 scenarios |
|  | Issue 2-1-3: Effective MGRP used to determine requirements Companies’ views: Companies have a consensus to follow existing requirements and agreed on the following Tentative agreements: For per-FR measurement gap capable UE in NR standalone operation (with single carrier, NR CA and NR-DC configuration), for per-FR gap-based measurement, when there is no serving cell in a particular FR, where measurement objects are configured, regardless if explicit per-FR measurement gap is configured in this FR, the effective MGRP in this FR is used to determine requirements: - 20 ms for FR2 NR measurements also is applicable for 52.6 – 71 GHz Candidate options: Recommendations for 2nd round: No further discussion is needed in the second round |
| Sub-topic #2-2: Measurement gap interruption requirements | Issue 2-2-1: MG timing advance for FR2-2 Companies’ views: All companies agreed with the proposal to reuse 0.25ms as MG timing advance for 52.6-71GHz Tentative agreements: RAN4 should reuse 0.25ms as MG timing advance for 52.6-71GHz Candidate options: Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-2-2: High SCS in the case with per-UE gap or per-FR gap for FR1 Companies’ views: All companies agreed with the proposal to extend NOTE 2 in tables 9.1.2-4 and 9.1.2-4a to 480kHz and 960kHz SCS Tentative agreements: RAN4 should modify NOTE 2 in Table 9.1.2-4 and Table 9.1.2-4a to capture that 480 kHz and 960 kHz are only applicable to the case with per-UE measurement gap Candidate options: Recommendations for 2nd round: No further discussion is needed in the second round |
|  | Issue 2-2-3: MRTD impact on gap interruptions Companies’ views: All companies agreed that the impact of MRTD should be considered when defining the requirements for gap interruptions. One company mentioned that it might be already covered in the spec Tentative agreements: Candidate options: - Option 1: The impact of MRTD should be considered when defining the requirements for gap interruptions - Option 2: The impact of MRTD is already covered in current specification and there is no need to additionally consider it while defining the requirements Recommendations for 2nd round: Continue discussion in the 2nd round |
|  | Issue 2-2-4: Measurement gap interruption requirements Companies’ views: All companies agreed with the proposal Tentative agreements: Table 9.1.2-4, Table 9.1.2-4a and Table 9.1.2-4b to include higher SCS’s for 52.6 – 71 GHz as shown below Table 9.1.2-4: Total number of interrupted slots on all serving cells during MGL for Synchronous EN-DC, NR standalone operation (with single carrier, NR CA and synchronous NR-DC configuration) and NE-DC, and on all serving cells in MCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1 NR; Total number of interrupted slots on serving cells. SCS; When MG timing advance of 0ms is applied; When MG timing advance of 0.5ms is applied. (kHz); MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms; MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms. 15; 20; 10; 6; 4; 3; 21Note3; 11Note3; 7Note3; 5Note3; 4Note3. 30; 40; 20; 12; 8; 6; 40; 20; 12; 8; 6. 60; 80; 40; 24; 16; 12; 80; 40; 24; 16; 12. 120; 160; 80; 48; 32; 24; 160; 80; 48; 32; 24. 480; 640; 320; 192; 128; 96; 640; 320; 192; 128; 96. 960; 1280; 640; 384; 256; 192; 1280; 640; 384; 256; 192. NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz ,480 kHz and 960 kHz is only applicable to the case with per-UE measurement gap. NOTE 3: Non-overlapped half-slots occur before and after the measurement gap. Whether a Rel-15 UE can receive and/or transmit in those half-slots is up to UE implementation.. Table 9.1.2-4a: Total number of interrupted slots on serving cells during MGL for Asynchronous EN-DC, and on all serving cells in SCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1 NR; Total number of interrupted slots on serving cells. SCS; When MG timing advance of 0ms is applied; When MG timing advance of 0.5ms is applied. (kHz); MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms; MGL=20ms; MGL=10ms; MGL=6ms; MGL=4ms; MGL=3ms. 15; 21; 11; 7; 5; 4; 21; 11; 7; 5; 4. 30; 41; 21; 13; 9; 7; 41; 21; 13; 9; 7. 60; 81; 41; 25; 17; 13; 81; 41; 25; 17; 13. 120; 161; 81; 49; 33; 25; 161; 81; 49; 33; 25. 480; 641; 321; 193; 129; 97; 641; 321; 193; 129; 97. 960; 1281; 641; 385; 257; 193; 1281; 641; 385; 257; 193. NOTE 1: For Gap Pattern ID 0, 1, 2 and 3, total number of interrupted subframes on MCG is MGL subframes when MG timing advance of 0ms is applied, and (MGL+1) subframes when MG timing advance of 0.5ms is applied. NOTE 2: NR SCS of 120 kHz ,480 kHz and 960 kHz is only applicable to the case with per-UE measurement gap.. Table 9.1.2-4b: Total number of interrupted slots on FR2 serving cells during MGL for EN-DC, NR standalone operation (with single carrier, NR CA and NR-DC configuration) and NE-DC with per-UE measurement gap or per-FR measurement gap for FR2 NR; Total number of interrupted slots on FR2 serving cells. SCS; When MG timing advance of 0ms is applied; When MG timing advance of 0.25ms is applied. (kHz); MGL= 20ms; MGL= 10ms; MGL= 5.5ms; MGL= 3.5ms; MGL= 1.5ms; MGL= 20ms; MGL= 10ms; MGL= 5.5ms; MGL= 3.5ms; MGL= 1.5ms. 60; 80; 40; 22; 14; 6; 80; 40; 22; 14; 6. 120; 160; 80; 44; 28; 12; 160; 80; 44; 28; 12. 480; 640; 320; 176; 112; 48; 640; 320; 176; 112; 48. 960; 1280; 640; 352; 224; 96; 1280; 640; 352; 224; 96. NOTE 1: The total number of interrupted slots is based on that SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR2 serving cell. NOTE 2: Slot occurs before or after the measurement gap may be interrupted additionally if SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR1 serving cell.. Candidate options: Recommendations for 2nd round: No further discussion is needed in the second round |

## Discussion on 2nd round

### Sub-topic 2-1: Measurement gaps applicability

Issue 2-1-1: Existing measurement gap patterns applicability

- Proposals

  - Option 1: No need to introduce new gap patterns

  - Option 2: RAN4 to study if new gap patterns with smaller MGL can be introduced

- Recommended WF

  - Continue the discussion in the 2nd round.

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| Ericsson | Support Option 1. We can revise it after detailed and robust solution on new gap patterns are proven. |
| Vivo | Support Option1. We don’t see justification to introduce new gap patterns. |
| LG | We think that the current MG patterns can be used for FR2-2. So, we prefer option 1. |
| MTK | Support Option 1. |
| Intel | Support Option 1 |
| Nokia | We support option 1 |
| Qualcomm | Support option 1. Can revisit if needed. |
| Apple | Can we rephrase option 1 as: Current gap patterns for FR2-1 can be reused, new gap patterns with smaller MGL, if justified, can be considered? In this way, we don’t completely close the door to new gap patterns. |
| Huawei | Fine with version from Apple. |

Issue 2-1-2: Per-FR measurement gaps for FR2-1 and FR2-2

- Proposals

  - Option 1 (Ericsson, MediaTek, Nokia, LG, Apple, Huawei, CMCC, Intel, OPPO): RAN4 to study if per-FR measurement gaps for FR2-1 and FR2-2 should be defined independently.

  - Option 2 (vivo, QC): no need to separate FR2-1 from FR2-2 for per-FR measurement gaps

- Recommended WF

  - Continue the discussion in the 2nd round. Please consider an agreement from thread [230] not to focus on FR2-1 + FR2-2 scenarios

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| Ericsson | Ok to support Option 2. No tech. objection in round1. |
| Vivo | Support option 2. We are also fine to FFS. |
| LG | Even if the FR2-1+FR2-2 is out of scope in this release, this is related MG configuration. So we think that RAN4 needs further discussion whether per-FR MG for FR2-1 an FR2-2 is needed. |
| Intel | Support Option 2. Also ok to FFS |
| Nokia | Considering that we are not focusing on FR2-1 + FR2-2 scenarios, we propose another option that leaves this topic for FFS. So we propose Option 3: - Option 3: no need to discuss separate FR2-1 from FR2-2 for per-FR measurement gaps as long as FR2-1 + FR2-2 scenarios are not agreed. |
| Qualcomm | Support option 2. Fine with Option 3 from Nokia too. |
| Apple | We would like to keep option 1 alive for now, knowing we can fall back to option 2 later. |
| Huawei | Option 1. The per-FR gap configuration should not be decided byt he prioritized scenarios in this release. Thus, it is suggested to FFS |

### Sub-topic 2-2: Measurement gap interruption requirements for FR2-2

Issue 2-2-3: MRTD impact on gap interruptions

- Proposals

  - Option 1: The impact of MRTD should be considered when defining the requirements for gap interruptions

  - Option 2: The impact of MRTD is already covered in current specification and there is no need to additionally consider it while defining the requirements

- Recommended WF

  - Continue the discussion in the 2nd round.

Companies views’ collection for 2nd round:

| Company | Comments |
| --- | --- |
| Ericsson | In general, Support option1. How to consider MRTD needs more study, e.g. coping with aligned frame boundaries and non-aligned frame boundaries |
| vivo | We are fine to FFS. |
| Intel | Agree that the impact of MRTD should be considered in the requirements. Ok to further study the way to consider it. |
| Nokia | We are ok in further studying it. |
| Qualcomm | Needs further study |
| Apple | We prefer to further study the impact of MRTD. |
| Huawei | Option 1 FFS |

## Summary for 2nd round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #2-1: Measurement gaps applicability | Issue 2-1-1: Existing measurement gap patterns applicability Companies’ views: During the 1st round it was agreed that the existing gaps are also feasible for higher SCS. Here the discussion was focused on the question whether we need new gap patterns with smaller MGL Most of the companies agree that there is no need to introduce new gap patterns. However, it was proposed not to close completely the door to new gap patterns Agreement: - The existing gaps are also feasible for higher SCS - New gap patterns with smaller MGL, if justified, can be considered |
|  | Issue 2-1-2: Per-FR measurement gaps for FR2-1 and FR2-2 Companies’ views: Some companies prefer not to define independent gap patterns while FR2-1 + FR2-2 scenarios are not agreed. Some companies say that the decision should not be done based on currently prioritized scenarios. Companies are ok to keep open the question about whether independent per-FR measurement gaps for FR2-1 and FR2-2 should be defined Agreement: FFS if per-FR measurement gaps for FR2-1 and FR2-2 should be defined independently |
| Sub-topic #2-2: Measurement gap interruption requirements | Issue 2-2-3: MRTD impact on gap interruptions Companies’ views: Companies prefer to further study the impact of MRTD Agreement: FFS: The impact of MRTD when defining the requirements for gap interruptions |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on NR extension to 71 GHz – RRM - 2 | Intel | WF is supposed to capture the agreements and open issues |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2115352 | WF on NR extension to 71 GHz – RRM - 2 | Intel | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Intel | Ilya Bolotin | ilya.bolotin@intel.com |
| Ericsson | Ming Li | ming.l.li@ericsson.com |
| MediaTek Inc. | Hsuanli Lin | Hsuanli.Lin@mediatek.com |
| Nokia | Rafael Paiva | Rafael.paiva@nokia.com |
| LG | Jin-Yup Hwang | jinyup.hwang@lge.com |
| Apple | Steven Chen | steven.x.chen AT apple.com |
| Huawei | Zhongyi Shen | shenzhongyi3@huawei.com |
| CMCC | Xiaoran ZHANG | zhangxiaoran@chinamobile.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)