3GPP TSG-RAN WG4 Meeting # 100-e 												R4-2115435

Electronic Meeting, 16 – 27 August, 2021

Agenda item:			9.8.2

Source:	CMCC

Title:	WF on RRM for FR1 HST

Document for:	Approval

Introduction

This is the WF to capture all agreements and open issues in [100-e][217] NR_HST_FR1_enh_RRM for RAN4#100e meeting

Topic #1: intra-frequency measurements

Sub-topic 1-1 NSCC_SSB for CSSFoutside_gap,i

Issue 1-1: NSCC_SSB for CSSFoutside_gap,i

- Agreements

  - NSCC_SSB clarification/correction should cover both non-HST and HST, i.e. unified NSCC_SSB design for both HST and non-HST. FFS from which release to have NSCC_SSB clarification/correction.

- Open issues

  - Continue to discuss from which release to have NSCC_SSB clarification/correction:

    - Option 1: from Rel-15

    - Option 2: from Rel-17, i.e. R15/R16 UEs are not impacted. FFS whether to introduce UE capability or using AccessStratumRelease to differentiate the expected UE behaviour between R15/R16 UEs and R17 UEs.

    - Option 3: from Rel-16 , i.e. R15 UEs are not impacted. FFS whether to introduce UE capability or using AccessStratumRelease to differentiate the expected UE behaviour between R15 UEs and R16 UEs

Sub-topic 1-2 Kp

Issue 1-2: Kp for deactivated Scell measurement

- Agreements

    - The Kp modification for measurement on deactivated SCC agreed in Rel-15 maintenance shall also apply to R17 HST. And the details are:

      - Time period for PSS/SSS detection, deactivated Scell (FR1):

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCycleSCell, M2 Note 1xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: When [RRM enhancement for high speed] is not configured, M2 = 1.5; When [RRM enhancement for high speed is configured], M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

      - Time period for time index detection, deactivated Scell (FR1):

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(3 x Kp) x max(measCycleSCell, M2 Note 1xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(3 x Kp)x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: When [RRM enhancement for high speed] is not configured, M2 = 1.5; When [RRM enhancement for high speed is configured], M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 |  |

      - Measurement period, deactivated Scell (FR1):

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | ceil( 5 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 160ms | ceil(5 x Kp) x max(measCycleSCell, M2 Note 1 x DRX cycle) x CSSFintra |
| 160ms < DRX cycle≤ 320ms | ceil(4 x Kp) x max(measCycleSCell, M2 Note 1 x DRX cycle) |
| DRX cycle>320ms | ceil( Y Note 2 x Kp ) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 2: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms |  |

Topic #2: inter-frequency measurements

Sub-topic 2-1: inter-frequency measurement in idle state

Issue 2-1: whether to define the enhancement for inter-frequency measurement in idle mode for HST

- Agreements

  - Define the enhancement for inter-frequency measurement in IDLE mode

  - Based on UE declaration of whether idle mode inter-frequency measurement enhancement for HST is supported.

    - Note: the support of HST Idle mode inter-frequency measurement enhancements is optional without capability signalling.

Issue 2-2: how to perform the enhancement for inter-frequency measurement in idle mode for HST

- Option 1 (QC, CATT, CMCC, Ericsson): using the same requirement as for intra-frequency measurement for HST

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x M2 (8 x M2) | 0.32 x M3 (1 x M3) | 0.96 x M4 (3 x M4) |  |
| 0.64 | 5.12 (8) | 0.64 (1) | 1.92 (3) |  |
| 1.28 | 8.96 (7) | 1.28 (1) | 3.84 (3) |  |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |  |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 |  |  |  |  |

- Option 2 (OPPO, HW, MTK, vivo, Xiaomi): The R16 enhanced EUTRA-NR inter-RAT measurement requirements in idle mode could be reused for NR inter-frequency measurements

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 4.16 x M2 (13 x M2)Note 1 | 0.64 x M3 (2 x M3)Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | 7.68 (12) | 1.28 (2) | 1.92 (3) |
| 1.28 | 12.8(10) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=M2=M3=1. |  |  |  |

Issue 2-3: FFS on the principle on the requirements if there are both HST inter-frequency layers and non-HST inter-frequency layers to be measured

- Option 1 (CATT, CMCC, OPPO, Ericsson, HW, MTK, QC, vivo, Xiaomi): NHST_inter-f carrier * THST_interf + NnonHST_inter-f carrier * TnonHST_interf can be specified if there are HST inter-frequency carriers and non-HST inter-frequency carriers to be measured in idle mode

- Option 2: FFS

Sub-topic 2-2: inter-frequency measurement with MG, connected state

Issue 2-4: PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST

- Option 1 (QC, Nokia):

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 6 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(6*M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 6 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5; When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 |  |

- Option 2 (CATT, MTK, OPPO, vivo, HW):

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5. When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |  |

- Option 3 (Ericsson, Nokia):

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, N Note 4 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(N Note 4 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | N Note 4 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 4: When highSpeedMeasFlag-r16 is not configured, N = 8; otherwise N = 5. |  |

- Option 4 (Apple):

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, Ceil(8*M2/1.5) * Max(MGRP, SMTC period)) * CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*M2 Note 3) * Max(MGRP, SMTC period, DRX cycle)) * CSSFinter |
| DRX cycle > 320ms | Ceil(8*M2/1.5) * DRX cycle * CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5; When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 |  |

- Option 5 (CMCC):

| Condition NOTE1,2 | T PSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 6 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle≤ 320ms | Max(600ms, Ceil(6 M Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Y Note 4 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M = 1.5 if SMTC periodicity > 40 ms, otherwise M=1 NOTE 4: Y= 4 when SMTC <= 40ms, Y= 6 when SMTC > 40ms |  |

- Option 6 (QC, Nokia, CMCC, Ericsson, Xiaomi):

| Condition NOTE1,2 | TPSS/SSS_sync_inter for FR1 HST |
| --- | --- |
| No DRX | Max(600ms, 6 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(6 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 6 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Issue 2-5: measurement delay requirement for inter-frequency measurement with MG in HST in connected state for HST

- Agreements:

| DRX cycle | T SSB_measurement_period_intra for FR1 HST |
| --- | --- |
| No DRX | max(200ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7 |
| DRX cycle ≤ 160ms | max(200ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2 |
| 160ms < DRX cycle ≤ 320ms | ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2 |
| DRX cycle>320ms | N4 x DRX cycle x CSSFinter N4 = 4 x M2 |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

Topic #3: other

Sub-topic 3-1: L1-SINR

Issue 3-1: whether L1-SINR measurement is applicable to HST

- Option 1 (CMCC, vivo): Yes. The configuration of CMR/IMR is up to network configuration, there is no restriction on L1-SINR for HST scenario

- Option 2 (MTK, OPPO, Ericsson, HW, Apple, QC, Nokia, Intel): RAN4 needs to confirm whether L1-SINR measurement is applicable for HST or not

Issue 3-2: If L1-SINR measurement is applicable to HST, whether it is necessary to specify upper bound of side condition for L1-SINR measurement

- Companies are encouraged to evaluate whether an applicable upper bound for L1-SINR is needed and what is the concrete value of the upper bound, if needed.

- Following is the simulation assumption for information

| Simulation parameters | Comments/values |  |
| --- | --- | --- |
| Carrier frequency for Cell 1 | 3.6 GHz |  |
| channel | AWGN |  |
| BS transmit antennas for CSI-RS | 1 tx |  |
| UE receive antennas | 2 rx |  |
| Data channel subcarrier spacing | The same as CSI-RS subcarrier spacing |  |
| Measurement period (in number of measurement samples) | 1 sample |  |
| Subcarrier spacing | 15 kHz; 30 kHz |  |
| CSI-RS periodicity | 5 ms |  |
| Carrier frequency offset | Option 1: 972Hz for 15kHz; 1667Hz for 30kHz | Option 2: 1944Hz for 15kHz; 3334Hz for 30kHz |
| Propagation condition | High speed train condition specified in Table G.3 in TS 38.104 |  |
| Number of PRBs | 48 |  |
| Density | 3 |  |
| Side condition (SNR) on CMR | -3 dB |  |
| Frequency tracking | TRS |  |

Sub-topic 3-2: SCell link recovery

Issue 3-3: Scell link recovery

- Agreements:

- For Scell link recovery for HST:

- There is no limitation on the number of band(s)in the spec, it depends on network

- UE capability of maxNumberSCellBFR-r16 introduced in Rel-16 can be reused

- The requirements of Scell link recovery for non-HST can be applied for HST

- the requirements of Scell link recovery for non-HST include BFD, CBD, beam failure recovery in SCell

  - FFS: Performance degradation may occur when total evaluation period is longer than 5 secs in HST

Sub-topic 3-3: CSSF

Issue 3-4: CSSF

- Agreements

  - For CSSF, it depends on network. There is no need to have the limitation on the number of Scell (s) in the spec

    - FFS: Performance degradation may occur when total evaluation period is longer than 5 secs in HST

Sub-topic 3-4: signalling

Issue 3-5-1: for HST CA, whether highSpeedMeasFlag-r16 can be reused for the indication of application of enhanced CA requirements

Background for information: According to current RAN2 spec TS38.331, highSpeedMeasFlag-r16 is configured in IE ServingCellConfigCommon and IE ServingCellConfigCommonSIB. And the IE ServingCellConfigCommon is used to configure cell specific parameters when configuring a UE with a SCells.

- Option 1: Yes

- Option 2: No

Issue 3-5-2: for inter-frequency measurement, whether highSpeedMeasFlag-r16 can be reused for the indication of application of enhanced requirements

- Option 1: Yes

- Option 2: No

Issue 3-6-1: for idle state, whether NW shall indicate which inter-frequency layers need to be measured more often, for which enhanced inter-frequency measurement requirements shall apply

- Option 1: Yes

- Option 2: No

Issue 3-6-2: for connected state, whether NW shall indicate which inter-frequency layers need to be measured more often, for which enhanced inter-frequency measurement requirements shall apply

- Option 1: Yes

- Option 2: No

Sub-topic 3-5: release independent

Issue 3-7: release independent

- FFS whether Rel-17 NR HST RRM enhancement can be release independent from Rel-15

- Reference

R4-2103678 WF on RRM for NR HST, RAN4#98-e

R4-2105793 WF on RRM for NR HST, RAN4#98-e-bis

R4-2108341 WF on RRM for NR HST, RAN4#99-e