3GPP TSG-RAN4 WG4 Meeting # 100-e	R4-2115449

# Electronic meeting, August 16 - 27, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.171 | CR | 0015 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR to TS 38.171 on requirements maintenance (Rel-16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC, Spirent |  |  |  |  |  |  |  |  |  |
| Source to TSG | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15, NR_newRAT-Perf, TEI16 |  |  |  |  |  | Date: |  |  | 2021-08-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This big CR merges the mutiple endorsed draft CRs. The reason for change in each endorsed draft CR is copied below. R4-2112482 Draft CR for 38.171 Rel-16 CR on satellite allocation The way the GNSS scenarios are defined, the complexity of the test case increases with the number of constellations used. This complexity increase is not consistent with the real field behavior: UEs supporting multiple constellations will generally see many more satellites. R4-2115270 Draft CR for 38.171 Rel-16 Frequency bands for testing of A-GNSS sensitivity requirements In EN-DC operation mode, the performance of the A-GNSS sensitivity test may be influenced by the bearer bands used for the test scenario. However, the A-GNSS test requirements and scenarios do not specify any band-combinations in which the test cases should be validated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of change in each endorsed draft CR is copied below. R4-2112482 Draft CR for 38.171 Rel-16 CR on satellite allocation Increase the number of satellites to maintain always an overdetermined system with 1 equation more than unknowns for Table 6.2, Table 6.5, Table 6.8, Table 6.11, Table 6.14, and Table 6.18. R4-2115270 Draft CR for 38.171 Rel-16 Frequency bands for testing of A-GNSS sensitivity requirements 1. Only those EN-DC configurations that can generate second or third order intermodulation (IM) products falling into the GNSS reception bands need to be tested. 2. The EN-DC configurations are divided into groups with similar IMD level and risks. For each group, only one of the EN-DC configurations in the group need to be tested. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consequences if not approved for each endorsed draft CR are copied below. R4-2112482 Draft CR for 38.171 Rel16 CR on satellite allocation Not consistent UE behavior R4-2115270 Draft CR for 38.171 Rel-16 Frequency bands for testing of A-GNSS sensitivity requirements It remains unclear which frequency band combinations should be used for verifying A-GNSS sensitivity requirements in EN-DC operation mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | R4-2112482 Draft CR for 38.171 Rel16 CR on satellite allocation 6.1.1, 6.1.2, 6.2, 6.3, 6.4, 6.5, B.1.5.2 R4-2115270 Draft CR for 38.171 Rel-16 Frequency bands for testing of A-GNSS sensitivity requirements B.1.13 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 37.571-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR’s revision history: |  |
| --- | --- |

<Start of Change 1>

### 6.1.1	Coarse time assistance

In this test case 6 satellites are generated for the terminal for single constellation and dual constellation, and 7 satellites are generated for triple constellation. AWGN channel model is used.

Table 6.1: Test parameters

| System | Parameters | Unit | Value |
| --- | --- | --- | --- |
|  | Number of generated satellites per system | - | See Table 6.2 |
|  | Total number of generated satellites | - | 6 or 7(2) |
|  | HDOP range |  | 1.4 to 2.1 |
|  | Propagation conditions | - | AWGN |
|  | GNSS coarse time assistance error range | seconds | 2 |
| BDS | Reference high signal power level | dBm | -136 |
|  | Reference low signal power level | dBm | -145 |
| Galileo | Reference high signal power level | dBm | -142 |
|  | Reference low signal power level | dBm | -147 |
| GLONASS | Reference high signal power level | dBm | -142 |
|  | Reference low signal power level | dBm | -147 |
| GPS(1) | Reference high signal power level | dBm | -142 |
|  | Reference low signal power level | dBm | -147 |
| NOTE 1: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. NOTE 2: 7 satellites apply only for case of triple constellation. |  |  |  |

Table 6.2: Power level and satellite allocation

|  |  | Satellite allocation for each constellation |  |  |
| --- | --- | --- | --- | --- |
|  |  | GNSS-1(1) | GNSS-2 | GNSS-3 |
| Single constellation | High signal level | 1 | - | - |
|  | Low signal level | 5 | - | - |
| Dual constellation | High signal level | 1 | - | - |
|  | Low signal level | 2 | 3 | - |
| Triple constellation | High signal level | 1 | - | - |
|  | Low signal level | 2 | 2 | 2 |
| Note 1: GNSS-1, i.e. the system having the satellite with high signal level, shall be selected by the device manufacturer. |  |  |  |  |

<End of Change 1>

<Start of Change 2>

### 6.1.2	Fine time assistance

This requirement is only valid for fine time assistance capable UEs. In this requirement 6 satellites are generated for the terminal for single constellation and dual constellation, and 7 satellites are generated for triple constellation. AWGN channel model is used.

Table 6.4: Test parameters

| System | Parameters | Unit | Value |
| --- | --- | --- | --- |
|  | Number of generated satellites per system | - | See Table 6.5 |
|  | Total number of generated satellites | - | 6 or 7(2) |
|  | HDOP range |  | 1.4 to 2.1 |
|  | Propagation conditions | - | AWGN |
|  | GNSS coarse time assistance error range | seconds | 2 |
|  | GNSS fine time assistance error range | s | 10 |
| BDS | Reference signal power level | dBm | -147 |
| Galileo | Reference signal power level | dBm | -147 |
| GLONASS | Reference signal power level | dBm | -147 |
| GPS(1) | Reference signal power level | dBm | -147 |
| NOTE 1: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. NOTE 2: 7 satellites apply only for case of triple constellation. |  |  |  |

Table 6.5: Satellite allocation

|  | Satellite allocation for each constellation |  |  |
| --- | --- | --- | --- |
|  | GNSS-1 | GNSS-2 | GNSS-3 |
| Single constellation | 6 | - | - |
| Dual constellation | 3 | 3 | - |
| Triple constellation | 3 | 2 | 2 |

<End of Change 2>

<Start of Change 3>

## 6.2	Nominal accuracy

Nominal accuracy requirement verifies the accuracy of A-GNSS position estimate in ideal conditions. The primarily aim of the test is to ensure good accuracy for a position estimate when satellite signal conditions allow it. This test case verifies the performance of the first position estimate.

In this requirement 6 satellites are generated for the terminal for single constellation and dual constellation, and 7 satellites are generated for triple constellation. If SBAS is to be tested one additional satellite shall be generated. AWGN channel model is used. The number of simulated satellites for each constellation is as defined in Table 6.8.

Table 6.7: Test parameters

| System | Parameters | Unit | Value |
| --- | --- | --- | --- |
|  | Number of generated satellites per system | - | See Table 6.8 |
|  | Total number of generated satellites | - | 6 or 7(2) or 8(3) |
|  | HDOP Range | - | 1.4 to 2.1 |
|  | Propagation conditions | - | AWGN |
|  | GNSS coarse time assistance error range | seconds | 2 |
| BDS | Reference signal power level for all satellites | dBm | -133 |
| Galileo | Reference signal power level for all satellites | dBm | -127 |
| GLONASS | Reference signal power level for all satellites | dBm | -131 |
| GPS(1) | Reference signal power level for all satellites | dBm | -128.5 |
| QZSS | Reference signal power level for all satellites | dBm | -128.5 |
| SBAS | Reference signal power level for all satellites | dBm | -131 |
| NOTE 1: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. NOTE 2: 7 satellites apply for case of single or dual constellation and SBAS is supported or case of triple constellation and SBAS is not supported. NOTE 3: 8 satellites apply only for case of triple constellation and SBAS is supported. |  |  |  |

If QZSS is supported, one of the GPS satellites will be replaced by a QZSS satellite with respective signal support. If SBAS is supported, the SBAS satellite with the highest elevation will be added to the scenario.

Table 6.8: Satellite allocation

|  | Satellite allocation for each constellation |  |  |  |
| --- | --- | --- | --- | --- |
|  | GNSS 1(1) | GNSS 2(1) | GNSS 3(1) | SBAS |
| Single constellation | 6 | -- | -- | 1 |
| Dual constellation | 3 | 3 | -- | 1 |
| Triple constellation | 3 | 2 | 2 | 1 |
| NOTE 1: GNSS refers to global systems i.e. BDS, Galileo, GLONASS, GPS. |  |  |  |  |

<End of Change 3>

<Start of Change 4>

## 6.3	Dynamic range

The aim of a dynamic range requirement is to ensure that a GNSS receiver performs well when visible satellites have rather different signal levels. Strong satellites are likely to degrade the acquisition of weaker satellites due to their cross-correlation products. Hence, it is important in this test case to keep use AWGN in order to avoid loosening the requirements due to additional margin because of fading channels. This test case verifies the performance of the first position estimate.

In this requirement 6 satellites are generated for the terminal for single constellation and dual constellation, and 7 satellites are generated for triple constellation. Two different reference power levels, denoted as "high" and "low" are used for each GNSS. The allocation of "high" and "low" power level satellites depends on the number of supported GNSSs and it is defined in Table 6.11. AWGN channel model is used.

Table 6.10: Test parameters

| System | Parameters | Unit | Value |
| --- | --- | --- | --- |
|  | Number of generated satellites per system | - | See Table 6.11 |
|  | Total number of generated satellites | - | 6 or 7(2) |
|  | HDOP Range | - | 1.4 to 2.1 |
|  | Propagation conditions | - | AWGN |
|  | GNSS coarse time assistance error range | seconds | 2 |
| BDS | Reference high signal power level | dBm | -133.5 |
|  | Reference low signal power level | dBm | -145 |
| Galileo | Reference high signal power level | dBm | -127.5 |
|  | Reference low signal power level | dBm | -147 |
| GLONASS | Reference high signal power level | dBm | -131.5 |
|  | Reference low signal power level | dBm | -147 |
| GPS(1) | Reference high signal power level | dBm | -129 |
|  | Reference low signal power level | dBm | -147 |
| NOTE 1: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. NOTE 2: 7 satellites apply only for case of triple constellation. |  |  |  |

Table 6.11: Power level and satellite allocation

|  |  | Satellite allocation for each constellation |  |  |
| --- | --- | --- | --- | --- |
|  |  | GNSS 1(1) | GNSS 2(1) | GNSS 3(1) |
| Single constellation | High signal level | 2 | -- | -- |
|  | Low signal level | 4 | -- | -- |
| Dual constellation | High signal level | 1 | 1 | -- |
|  | Low signal level | 2 | 2 | -- |
| Triple constellation | High signal level | 1 | 1 | 1 |
|  | Low signal level | 2 | 1 | 1 |
| NOTE 1: GNSS refers to global systems i.e. BDS, Galileo, GLONASS, GPS. |  |  |  |  |

<End of Change 4>

<Start of Change 5>

## 6.4	Multi-path scenario

The purpose of the test case is to verify the receiver's tolerance to multipath while keeping the test setup simple. This test case verifies the performance of the first position estimate.

In this requirement 6 satellites are generated for the terminal for single constellation and dual constellation, and 7 satellites are generated for triple constellation. Some of the satellites have a one tap channel representing the LOS signal. The other satellites have a two-tap channel, where the first tap represents the LOS signal and the second represents a reflected and attenuated signal as specified in Annex C.2. The number of satellites generated for each GNSS as well as the channel model used depends on the number of systems supported by the UE and is defined in Table 6.14. The channel model as specified in Annex C.2 further depends on the generated signal.

Table 6.13: Test parameter

| System | Parameters | Unit | Value |
| --- | --- | --- | --- |
|  | Number of generated satellites per system | - | See Table 6.14 |
|  | Total number of generated satellites | - | 6 or 7(2) |
|  | HDOP range |  | 1.4 to 2.1 |
|  | Propagation conditions | - | AWGN |
|  | GNSS coarse time assistance error range | seconds | 2 |
| BDS | Reference signal power level | dBm | -133 |
| Galileo | Reference signal power level | dBm | -127 |
| GLONASS | Reference signal power level | dBm | -131 |
| GPS(1) | Reference signal power level | dBm | -128.5 |
| NOTE 1: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. NOTE 2: 7 satellites apply only for case of triple constellation. |  |  |  |

Table 6.14: Channel model allocation

|  |  | Channel model allocation for each constellation |  |  |
| --- | --- | --- | --- | --- |
|  |  | GNSS-1 | GNSS-2 | GNSS-3 |
| Single constellation | One-tap channel | 2 | -- | -- |
|  | Two-tap channel | 4 | -- | -- |
| Dual constellation | One-tap channel | 1 | 1 | -- |
|  | Two-tap channel | 2 | 2 | -- |
| Triple constellation | One-tap channel | 1 | 1 | 1 |
|  | Two-tap channel | 2 | 1 | 1 |

<End of Change 5>

<Start of Change 6>

## 6.5	Moving scenario and periodic update

The purpose of the test case is to verify the receiver's capability to produce GNSS measurements or location fixes on a regular basis, and to follow when it is located in a vehicle that slows down, turns or accelerates. A good tracking performance is essential for certain location services. A moving scenario with periodic update is well suited for verifying the tracking capabilities of an A-GNSS receiver in changing UE speed and direction. In the requirement the UE moves on a rectangular trajectory, which imitates urban streets. AWGN channel model is used. This test is not performed as a TTFF test.

In this requirement 6 satellites are generated for the terminal for single constellation and dual constellation, and 7 satellites are generated for triple constellation. The UE is requested to use periodical reporting with a reporting interval of 2 seconds.

The UE moves on a rectangular trajectory of 940 m by 1 440 m with rounded corner defined in Figure 6.1. The initial reference is first defined followed by acceleration to final speed of 100 km/h in 250 m. The UE then maintains the speed for 400 m. This is followed by deceleration to final speed of 25 km/h in 250 m. The UE then turn 90 degrees with turning radius of 20 m at 25 km/h. This is followed by acceleration to final speed of 100 km/h in 250 m. The sequence is repeated to complete the rectangle.

Table 6.16: Trajectory Parameters

| Parameter | Distance (m) | Speed (km/h) |
| --- | --- | --- |
| l11, l15, l21, l25 | 20 | 25 |
| l12, l14, l22, l24 | 250 | 25 to 100 and 100 to 25 |
| l13 | 400 | 100 |
| l23 | 900 | 100 |

Figure 6.1: Rectangular trajectory of the moving scenario and periodic update test case

Table 6.17: Test Parameters

| System | Parameters | Unit | Value |
| --- | --- | --- | --- |
|  | Number of generated satellites per system | - | See Table 6.18 |
|  | Total number of generated satellites | - | 6 or 7(2) |
|  | HDOP Range per system | - | 1.4 to 2.1 |
|  | Propagation conditions | - | AWGN |
|  | GNSS coarse time assistance error range | seconds | 2 |
| BDS | Reference signal power level for all satellites | dBm | -133 |
| Galileo | Reference signal power level for all satellites | dBm | -127 |
| GLONASS | Reference signal power level for all satellites | dBm | -131 |
| GPS(1) | Reference signal power level for all satellites | dBm | -128.5 |
| NOTE 1: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. NOTE 2: 7 satellites apply only for case of triple constellation. |  |  |  |

Table 6.18: Satellite allocation

|  | Satellite allocation for each constellation |  |  |
| --- | --- | --- | --- |
|  | GNSS 1(1) | GNSS 2(1) | GNSS 3(1) |
| Single constellation | 6 | -- | -- |
| Dual constellation | 3 | 3 | -- |
| Triple constellation | 3 | 2 | 2 |
| NOTE 1: GNSS refers to global systems i.e. BDS, Galileo, GLONASS, GPS. |  |  |  |

<End of Change 6>

<Start of Change 7>

### B.1.5.2	UE supports other A-GNSSs

In the case of test cases in clause 6 (UE supports other GNSSs), the satellite constellation shall consist of 35 satellites for BDS (5 GEO, 27 MEO, 3 IGSO); 27 satellites for Galileo; 24 satellites for GLONASS; 27 satellites for GPS/Modernized GPS; 3 satellites for QZSS; 2 satellites for SBAS. Almanac assistance data shall be available for all these satellites. At least 7 of the satellites per BDS, Galileo, GLONASS, GPS/Modernized GPS constellation shall be visible to the UE (that is, above 15 degrees elevation with respect to the UE). At least 1 of the satellites for QZSS shall be within 15 degrees of zenith; and at least 1 of the satellites for SBAS shall be visible to the UE. For BDS with reference location in Asia, at least 1 of the visible satellites shall be a GEO (above 15 degrees elevation with respect to the UE). All other satellite specific assistance data shall be available for all visible satellites. In each test, signals are generated for only 6 satellites for single constellation and dual constellation and 7 satellites for triple constellation (or one additional satellite if SBAS is included). The HDOP for the test shall be calculated using these satellites. The simulated satellites for BDS, Galileo, GLONASS GPS/Modernized GPS shall be selected from the visible satellites for each constellation consistent with achieving the required HDOP for the test. For BDS with reference location in Asia, 1 of the simulated satellites shall be a GEO.

<End of Change 7>

<Start of Change 8>

## B.1.13	EN-DC band combinations for testing A-GNSS sensitivity

### B.1.13.1	EN-DC band combination groups

For the A-GNSS sensitivity requirements in EN-DC operation mode with uplink assigned to E-UTRA and NR frequency bands, the requirements in clause 5.1 can be verified by one EN-DC band combination in each of the applicable Frequency Group Combination specified in Table B.1.13.1-1. The A-GNSS sensitivity requirements for the remaining applicable EN-DC band combinations in each Frequency Group Combination are considered to have been verified by using the one EN-DC band combination in each Frequency Group Combination. The Frequency Groups are defined in Table B.1.13.1-2.

The applicable EN-DC band combinations for verifying A-GNSS sensitivity requirements in EN-DC operation mode are specified in clause B.1.13.2.

Table B.1.13.1-1: EN-DC band combination groups for verifying A-GNSS sensitivity requirements in EN-DC operation mode

| Frequency Group Combination | EN-DC Band Combinations |
| --- | --- |
| Group VHF-VHF | NA |
| Group VHF-LB | NA |
| Group VHF-MLB | NA |
| Group VHF-MB | NA |
| Group VHF-HB | NA |
| Group VHF-UHB1 | NA |
| Group VHF-UHB2 | NA |
| Group LB-VHF | NA |
| Group LB-LB | DC_5A_n12A DC_5A_n71A DC_8A_n20A DC_8A_n28A DC_12A_n5A DC_20A_n8A DC_20A_n28A DC_28A_n5A DC_28A_n8A DC_71A_n5A DC_20A_n83A |
| Group LB-MLB | DC_20A_n50A DC_20A_n51A DC_28A_n51A DC_28A_n50A |
| Group LB-MB | DC_5A_n2A DC_5A_n66A DC_8A_n1A DC_8A_n3A DC_8A_n34A DC_8A_n39A DC_12A_n2A DC_12A_n25A DC_12A_n66A DC_18A_n3A DC_20A_n1A DC_20A_n3A DC_26A_n25A DC_28A_n3A DC_71A_n66A DC_8A_n80A DC_20A_n80A |
| Group LB-HB | DC_5A_n7A DC_5A_n38A DC_5A_n40A DC_8A_n40A DC_8A_n41A DC_12A_n7A DC_12A_n38A DC_12A_n41A DC_20A_n7A DC_20A_n38A DC_20A_n41A DC_26A_n41A DC_28A_n7A DC_28A_n40A DC_28A_n41A DC_71A_n38A |
| Group LB-UHB1 | DC_5A_n48A DC_5A_n78A DC_8A_n77A DC_8A_n78A DC_12A_n78A DC_18A_n77A DC_18A_n78A DC_19A_n77A DC_19A_n78A DC_20A_n77A DC_20A_n78A DC_26A_n77A DC_26A_n78A DC_28A_n77A DC_28A_n78A DC_71A_n48A DC_71A_n78A |
| Group LB-UHB2 | DC_5A_n79A DC_8A_n79A DC_18A_n79A DC_19A_n79A DC_26A_n79A DC_28A_n79A |
| Group MLB-VHF | NA |
| Group MLB-LB | DC_11A_n28A |
| Group MLB-MLB | NA |
| Group MLB-MB | DC_11A_n3A |
| Group MLB-HB | NA |
| Group MLB-UHB1 | DC_11A_n77A DC_11A_n78A DC_21A_n77A DC_21A_n78A |
| Group MLB-UHB2 | DC_11A_n79A DC_21A_n79A |
| Group MB-VHF | NA |
| Group MB-LB | DC_1A_n5A DC_1A_n8A DC_1A_n20A DC_1A_n28A DC_1A_n71A DC_2A_n5A DC_2A_n12A DC_2A_n71A DC_3A_n5A DC_3A_n8A DC_3A_n20A DC_3A_n28A DC_3A_n71A DC_66A_n5A DC_66A_n12A DC_66A_n71A DC_3A_n82A |
| Group MB-MLB | DC_1A_n50A DC_1A_n51A DC_3A_n50A DC_3A_n51A |
| Group MB-MB | DC_1A_n3A DC_2A_n66A DC_3A_n1A DC_3A_n34A DC_66A_n2A DC_66A_n25A DC_1A_n80A DC_2A_n2A2 DC_66A_n66A2 DC_3A_n3A2 DC_3A_n84A |
| Group MB-HB | DC_1A_n7A DC_1A_n38A DC_1A_n40A DC_1A_n41A DC_2A_n7A DC_2A_n38A DC_2A_n41A DC_3A_n7A DC_3A_n38A DC_3A_n40A DC_3A_n41A DC_4A_n38A DC_4A_n41A DC_25A_n41A DC_39A_n40A DC_39A_n41A DC_66A_n7A DC_66A_n38A DC_66A_n41A |
| Group MB-UHB1 | DC_1A_n77A DC_1A_n78A DC_2A_n48A DC_2A_n78A DC_3A_n77A DC_3A_n78A DC_4A_n78A DC_39A_n78A DC_66A_n48A DC_66A_n78A |
| Group MB-UHB2 | DC_1A_n79A DC_3A_n79A DC_39A_n79A |
| Group HB-VHF | NA |
| Group HB-LB | DC_7A_n5A DC_7A_n8A DC_7A_n20A DC_7A_n28A DC_7A_n71A DC_30A_n5A DC_41A_n28A |
| Group HB-MLB | DC_7A_n51A |
| Group HB-MB | DC_7A_n1A DC_7A_n3A DC_7A_n66A DC_30A_n2A DC_30A_n66A DC_40A_n1A DC_41A_n3A DC_7A_n80A DC_38A_n3A |
| Group HB-HB | DC_7A_n40A DC_40A_n41A DC_7A_n7A2 DC_41A_n41A |
| Group HB-UHB1 A(NOTE 1): | DC_40A_n77A DC_40A_n78A |
| B(NOTE 2): | DC_7A_n78A DC_38A_n78A DC_41A_n78A |
| C(NOTE 3): | DC_7A_n77A DC_41A_n77A |
| Group HB-UHB2 | DC_40A_n79A DC_41A_n79A |
| Group UHB1-VHF | NA |
| Group UHB1-LB | NA |
| Group UHB1-MLB | NA |
| Group UHB1-MB | NA |
| Group UHB1-HB | NA |
| Group UHB1-UHB1 | NA |
| Group UHB1-UHB2 | NA |
| Group UHB2-VHF | NA |
| Group UHB2-LB | NA |
| Group UHB2-MLB | NA |
| Group UHB2-MB | NA |
| Group UHB2-HB | NA |
| Group UHB2-UHB1 | NA |
| Group UHB2-UHB2 | NA |
| NOTE 1: This sub-group generates second order intermodulation products. NOTE 2: This sub-group generates third order intermodulation products. NOTE 3: This sub-group generates second and third order intermodulation products. |  |

Table B.1.13.1-2: Definition of Frequency Groups

| Frequency Group | Frequency Range (MHz) |
| --- | --- |
| VHF | 400.0 – 458.0 |
| LB | 662.0 – 916.0 |
| MLB | 1426.0 – 1518.0 |
| MB | 1626.0 – 2025.0 |
| HB | 2300.0 – 2690.0 |
| UHB1 | 3300.0 – 4201.0 |
| UHB2 | 4400.0 – 5000.0 |

### B.1.13.2	Applicable EN-DC band combinations for verifying A-GNSS sensitivity requirements

The A-GNSS sensitivity requirements in clause 5.1 when in EN-DC operation mode shall be verified for EN-DC band combinations that can generate second or third order intermodulation products falling into the following GNSS receiver bands for the particular GNSS (where supported by the UE):

-	GPS L1 C/A:				1574.3970 – 1576.4430 MHz

-	Galileo E1 / GPS L1C:	1573.3740 – 1577.4660 MHz

-	GLONASS G1:			1597.5515 – 1605.8860 MHz

-	BDS B1I:					1559.0520 – 1563.1440 MHz

For each frequency group combination in Table B.1.13.2-1 only one EN-DC band combination needs to be tested for the supported GNSS.

Table B.1.13.2-1: EN-DC band combinations for verifying A-GNSS sensitivity requirements

| Frequency Group Combination | EN-DC band combinations |  |  |
| --- | --- | --- | --- |
|  | GPS L1 / Galileo E1 | GLONASS G1 | BDS B1 |
| Group LB-LB | DC_20A_n28A DC_28A_n5A DC_20A_n83A | DC_8A_n28A DC_20A_n28A DC_28A_n8A DC_20A_n83A | DC_5A_n12A DC_12A_n5A DC_20A_n28A DC_28A_n5A DC_20A_n83A |
| Group LB-MLB | NA | NA | NA |
| Group LB-MB | NA | NA | NA |
| Group LB-HB | DC_5A_n40A DC_28A_n40A | DC_8A_n41A DC_28A_n40A | DC_5A_n40A DC_28A_n40A |
| Group LB-UHB1 | DC_8A_n77A DC_8A_n78A DC_20A_n77A DC_20A_n78A | DC_5A_n78A DC_8A_n77A DC_8A_n78A DC_20A_n77A DC_20A_n78A DC_26A_n77A DC_26A_n78A | DC_8A_n77A DC_8A_n78A |
| Group LB-UHB2 | NA | NA | NA |
| Group MLB-LB | NA | NA | NA |
| Group MLB-MLB | NA | NA | NA |
| Group MLB-MB | NA | NA | NA |
| Group MLB-HB | NA | NA | NA |
| Group MLB-UHB1 | NA | NA | NA |
| Group MLB-UHB2 | DC_11A_n79A DC_21A_n79A | DC_11A_n79A DC_21A_n79A | DC_11A_n79A DC_21A_n79A |
| Group MB-LB | NA | NA | NA |
| Group MB-MLB | NA | NA | NA |
| Group MB-MB | DC_1A_n3A DC_2A_n66A DC_3A_n1A DC_66A_n2A DC_66A_n25A DC_1A_n80A DC_3A_n84A | DC_1A_n3A DC_2A_n66A DC_3A_n1A DC_66A_n2A DC_66A_n25A DC_1A_n80A DC_3A_n84A | DC_1A_n3A DC_2A_n66A DC_3A_n1A DC_66A_n2A DC_66A_n25A DC_1A_n80A DC_3A_n84A |
| Group MB-HB | DC_1A_n40A | DC_1A_n7A | DC_1A_n7A |
| Group MB-UHB1 | DC_1A_n77A DC_1A_n78A DC_2A_n78A DC_3A_n77A DC_3A_n78A DC_4A_n78A DC_39A_n78A DC_66A_n78A | DC_1A_n77A DC_1A_n78A DC_2A_n78A DC_3A_n77A DC_3A_n79A DC_4A_n78A DC_39A_n78A DC_66A_n78A | DC_1A_n77A DC_1A_n78A DC_2A_n78A DC_3A_n77A DC_3A_n79A DC_4A_n78A DC_39A_n78A DC_66A_n78A |
| Group MB-UHB2 | DC_3A_n79A | NA | DC_3A_n79A |
| Group HB-LB | NA | DC_7A_n8A | NA |
| Group HB-MLB | NA | NA | NA |
| Group HB-MB | DC_40A_n1A | DC_40A_n1A | DC_40A_n1A |
| Group HB-HB | NA | NA | NA |
| Group HB-UHB1 A: | DC_40A_n77A | DC_40A_n77A | DC_40A_n77A |
| B: | DC_7A_n78A DC_38A_n78A DC_41A_n78A | DC_7A_n78A DC_38A_n78A DC_41A_n78A | DC_7A_n78A DC_38A_n78A DC_41A_n78A |
| C: | DC_7A_n77A DC_41A_n77A | DC_7A_n77A DC_41A_n77A | DC_7A_n77A DC_41A_n77A |
| Group HB-UHB2 | NA | NA | NA |

### B.1.13.3	Test frequencies for EN-DC band combinations

For verifying the sensitivity requirements in clause 5.1 in EN-DC operation mode, the E-UTRA and NR frequency and channel configuration shall be selected to ensure the intermodulation products fall into the GNSS receiver bands as defined in clause B.1.13.2 for the particular GNSS.

<End of Change 8>