3GPP TSG-RAN WG4 Meeting # 100-e	R4-2115728

# Electronic Meeting, August 16th – 27th, 2021

Title: 	TP: Simulation assumptions for intra-cell MMSE-IRC receiver

Source: 	Huawei, HiSilicon

Agenda item:	9.12.2.2

Document for:	Discussion

- Background

In RAN4 #99-e meeting, WF R4-2108666 is approved and corresponding initial simulation assumptions are determined. In this TP, we provide the simulation assumptions for intra-cell MMSE-IRC receiver.

Inter-user interference suppression for MU-MIMO

  - Link performance characterization

- 4.3.1 Parameters for link level evaluation

General simulation assumptions are listed in Table 4.3.3-1 and detail simulation assumptions are listed in Table 4.3.3-2

Table 4.3.3-1: General simulation assumptions

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 for tested channel bandwidth and subcarrier spacing |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  | Symbols | 0, 1 |
|  | Number of PRBs in CORESET |  |  | Table 5.2-2 of TS 38.101-4 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL8 |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI state |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | Single Panel Type I, Random per slot with equal probability of each applicable i1, i2 combination, and with REG bundling granularity for number of Tx larger than 1 |
| Cross carrier scheduling |  |  |  | Not configured |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | 'No CDM' for 1 transmit antenna 'FD-CDM2' for 2 and 4 transmit antenna |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| PT-RS configuration |  |  |  | PT-RS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 of TS 38.101-4 OP.1 TDD as defined in Annex A.5.2.1 of TS 38.101-4 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 of TS 38.101-4 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 for tested channel bandwidth and subcarrier spacing. |  |  |  |  |

Table 4.3.3-2: Detail simulation assumptions

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 for FDD and 40/30 for TDD |  |
| Duplex mode |  |  | FDD,TDD |  |
| MCS |  |  | 13 for rank 1, 13 and 19 for rank 2. | 16 QAM random symbols |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 1 |
|  |  |  | 2 | 2 |
|  | DMRS port allocation |  | Port 1000 for rank 1 Port 1000 and 1001 for rank 2 | For rank 1: - Option 1: Port 1001 - Option 2: Port 1002 For rank 2: - Port 1002 and Port 1003 |
|  | Scrambling ID |  | - Option 1: Same scrambling ID for both UEs - Option 2: Different scrambling ID for both UEs - Option 3: Variable scrambling configurations |  |
| MIMO configuration |  |  | 2T2R ULA Low and 2T4R ULA Low for case with rank1+1 and 4T4R XPL Low (Note 1) for case with rank 2+1 and rank 2+2 |  |
| Number of CDM groups without data for paired UEs not scheduled in same CDM groups |  |  | - Option 1: 1 - Option 2: 2 |  |
| HARQ process number |  |  | 4 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - Option 1: Select the precoding matrix to ensure orthogonality with target UE - Option 2: Select the precoding matrix randomly ensuring the selected precoding matrix shall not be identical to the precoding matrix of target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | TDLA30-10, TDLC300-100 |  |
| Receiver type |  |  | MMSE-IRC and MMSE-MRC | N/A |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |
| Note 1: The correlation matrix of XPL Low is defined in B.2.3.2.2 of TS 38.101-4 with 1=0, 2=0, =0 and  |  |  |  |  |

- Reference

- R4-2108666, Way Forward on MMSE-IRC for intra-cell inter-user interference, Huawei, HiSilicon.