3GPP TSG-RAN WG4 Meeting #100-e		R4-2115730

E-meeting, 16 August – 27 August, 2021

Agenda item:	9.12.2.1.1

Source:	Intel Corporation

Title:	WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC

Document for:	Approval

# Introduction

This document provides the way forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC.

# Common test parameters

## Background (agreements from #98-bis-e and #99-e meeting)

- Physical cell ID of 0 for the serving cell, and cell ID i for interferer i (where i ≥ 1, maximum number of i is FFS)

- SCS: FDD 15kHz, TDD 30kHz (Same SCS is used for the target and the interference cells)

- TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

- Number of carriers: In Rel-17, MMSE-IRC receiver performance requirements with interference cell condition is defined only for single carrier scenario

- PDCCH and PDSCH allocation in time domain

  - Use symbols #0 and #1 of each slot for PDCCH

  - PDSCH mapping type A, Start symbol 2, Duration 12

- PDSCH allocation in frequency domain: Full PRB

- DMRS configuration

  - For both serving and interfering cells, DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data (number of CDM groups without data is equal to 1), i.e., overlapped DMRS between target and interferer

- TRS/CSI-RS configuration

  - TRS/CSI-RS colliding with TRS/CSI-RS interference

- Antenna configuration

  - Tx as baseline for serving cell and interfering cells

  - UE with 2 and 4 RX

  - ULA low correlation as baseline

## Network type

- Previous meeting status

  - Synchronized for FDD and TDD

  - FFS asynchronized for FDD

- Way forward

  - Keep previous meeting agreements

  - Interested companies are encouraged to provide view on test setup for asynchronized FDD scenarios (especially, time and frequency offsets)

## Channel bandwidth

- Previous meeting status

  - Use 10MHz for FDD 15kHz and 40MHz for TDD 30kHz for initial simulation purpose

  - FFS whether to consider 40 MHz for FDD 15kHz and 100 MHz for TDD 30kHz for requirements definition

- Way forward

  - RAN4 introduce test case under 10MHz for FDD 15kHz and 40MHz for TDD 30kHz only with assumption that interference-plus-noise covariance estimation granularity does not exceed the PRB bundling size regardless the channel bandwidth from RAN4 requirements introduction perspective.

## SSB configuration

- Previous meeting status

  - Option 1: All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources

  - Option 2: Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources

- Way forward

  - Keep both options for further discussion

  - Interested companies can check the PBCH performance for both options

    - Use simulation assumptions from Section 5.4 of TS 38.101-4 for serving cell

    - Use the following SSB configuration for Option 2

      - SSB of serving cell is transmitted in PRB 0~19 in first occasion

      - For case with 1 interference cell (if agreed), SSB of interference cell is transmitted in PRB 0~19 in second occasion.

      - For case with 2 interference cell, SSB of interference cell 1 is transmitted in PRB 20~39 in first occasion, SSB of interference cell 2 is transmitted in PRB 0~19 in second occasion.

    - Other simulation assumptions are FFS

## Propagation condition

- Previous meeting status

  - Consider TDLA30-10 and TDLC300-100 channel models for evaluation purpose and select only one for requirements definition

- Way forward

  - Keep previous meeting agreements

  - Interested companies are encourage to provide views on down-selection criteria

## Test setup methodology for signal powers

- Previous meeting status

  - Performance measurement point

    - Option 1: SINR at 70% TP

    - Option 2: SNR at 70% TP

  - Methodology for interference profile configuration

    - Option 1: Use the DIP methodology

    - Option 2: Use the INR methodology

- Way forward

  - Use SNR and INR methodology for signal powers setup

# Target PDSCH parameters for scenario 1

## Background (agreements from #98-bis-e and #99-e meeting)

- Transmission rank: Use rank 1 as baseline

- PRB bundle size: Set PRB bundle size as 2 for target PDSCH

- HARQ process number: 4 for FDD 15kHz SCS and 8 for TDD 30kHz SCS as baseline

- Precoding model: Random precoder with Type I SP codebook

## MCS

- Previous meeting status

  - Use MCS 4 (QPSK, CR=0.3) and MCS 13 (16QAM, CR=0.5) for initial simulation purpose

  - Further discuss MCS for requirements definition

  - Consider MCS corresponding to QPSK and 16QAM modulation formats

- Way forward

  - Down selection between MCS 4 and MCS 13 based on results for agreed INR values based on the following criteria

    - Option 1: Testable performance benefit (i.e. > 1 dB) of MMSE-IRC vs MMSE-MRC

    - Option 2: SINR is not lower than -6 dB

    - Option 3: Consider the difference between SNR and INR to avoid possible handover (SNR-INR > -3dB)

    - Option 4: SNR > INR

    - Other options are not precluded

    - Using of multiple options is not precluded

  - Interested companies are encouraged to provide views on criteria for MCS down selection

# Interference model for scenario 1

## Background (agreements from #98-bis-e and #99-e meeting)

- Transmission rank of interfering PDSCH

  - Use the following assumptions initial simulation purpose: random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s)

- Precoding of interfering PDSCH

  - Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2.

- Modulation order of interfering PDSCH

  - Use 16QAM randomly modulated symbols for initial simulation assumption and other options not precluded.

## Deployment

- Previous meeting status

  - Consider Homogeneous deployment assumptions

  - FFS whether for consider HetNet deployment assumptions

- Way forward

  - Keep previous meeting agreement

## INR values for Homogeneous deployment assumptions

- Previous meeting status

  - Consider DIP1/2 = -1.73/-8.66 dB (INR1/2 = 5.43/-1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell) as baseline for initial link level analysis for Synchronous network

  - Results for other interference profiles are welcome

  - Other options are not precluded for requirements definition

  - FFS assumptions for asynchronous network

- Way forward

  - Further discuss the following options for PDSCH requirements definition for synchronous network

    - Option 1: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell

    - Option 2: INRs 7.77 and 2.29 dB in case of 2 interference cells and FFS in case of 1 interference cell

    - Option 3: INRs 13.91 and 3.34 dB in case of 2 interference cells and FFS in case of 1 interference cell

    - Other options are not precluded

  - FFS assumptions for asynchronous network

## INR values for HetNet deployment assumptions (in case HetNet is agreed)

- Previous meeting status

  - Option 1: INRs 11.39 and 5.45 dB (DIPs -1.23 and -7.16 dB)

  - Other options are not precluded

- Way forward

  - Keep previous meeting agreement

## Number of explicitly modeled interference cells

- Previous meeting status

  - Companies are encouraged to check performance with 1 and 2 interference cells for initial simulations

  - Further discuss the assumptions for requirements definition

- Way forward

  - Keep previous meeting agreement

# Receiver assumptions

## TRS-IC/IM processing

- Way forward

  - Further discuss whether to add in the simulation assumptions the clarification that no TRS interference cancellation/mitigation is considered for inter-cell MMSE-IRC requirements definition

  - No such clarification will be added in the TS 38.101-4

  - Interested companies are encouraged to check with performance for scenarios with and without TRS-IC/IM

# Release independence

- Way forward

  - RAN4 discuss whether the UE demodulation with inter-cell interference is released independent from Rel-15 or not, after RAN4 agree with the detailed simulation assumption

# References

- R4-2108664 “Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel Corporation, RAN4 #99-e, May 2021.