3GPP TSG-RAN WG4 Meeting # 100-e	R4-2115733

# Electronic Meeting, August 16th – 27th, 2021

Title: 	Way Forward on MMSE-IRC for intra-cell inter-user interference

Source: 	Huawei, HiSilicon

Agenda item:	9.12.2.2

Document for:	Discussion

Inter-user interference modelling

Number of paired UEs

1 target UE + 1 interference UE

Rank for target and interference PDSCH

- For rank of target UE is 1

  - Use rank 1(Target UE) + 1(Co-scheduled UE)

- For rank of target UE is 2

  - Option 1: Rank 2(Target UE)+ Rank 1(Co-schedule UE)

  - Option 2: Rank 2(Target UE)+ Rank 2(Co-schedule UE)

  - Option 3: Rank 2(Target UE)+Variable rank(Co-scheduled UE)

Antenna configuration

- 2T2R,2T4R for rank 1+1

- 4T4R for rank 2+1(if introduced) and rank 2+2(if introduced)

Codebook Type

- Type I SP codebook

PRB bundling size

- 2 PRBs for 2 and 4Tx cases

Precoding granularity

- 2 PRBs for 2 and 4Tx cases

PMI matrix selection for target UE

- Random for 2 and 4Tx cases

PMI matrix selection for Co-scheduled UE for 2TX and 4TX

- Option 1: Select the PMI matrix from the codebook of Co-scheduled UE to ensure it and PMI matrix of target UE are orthogonal.

- Option 2: Select the PMI matrix randomly from the codebook of Co-scheduled UE to ensure it is not equal to PMI matrix of target UE.

  - Option 2A : Use following method to randomly select PMI matrix for interference UE that is not identical to that of Target UE for rank 2+1

1) Randomly select the PMI matrix in codebook with rank2 and rank1 respectively.

2) Normalize the PMI matrix for each layer for both target UE and co-scheduled UE to make the norm of each PMI matrix of each layer equal to 1/3.

3) If the PMI matrix of rank 1 equals to the PMI matrix of any one layer of rank 2, reselect PMI matrix for Rank 1 with PMI index plus 1 and go back to step 2)

- Option 3: Cover both Option 1 and Option 2 for phase 1 evaluation.

DMRS ports for case with rank 1+1

- Option 1: DMRS port 0 for target UE, DMRS port 1 for the interference UE, i.e., same CDM group

- Option 2: DMRS port 0 for target UE, DMRS port 2 for the interference UE, i.e., different CDM groups

- Option 3: Variable DMRS port mapping during the test.

  - FFS the percent of each mapping and other details

    - Number of CDM groups without data configuration for case with rank 1+1 if same CDM group is agreed for target UE and co-scheduled UE

Number of CDM groups without data configuration for case with rank 1+1 if same CDM group is agreed for target UE and co-scheduled UE

- Option 1: 1 for target UE and co-scheduled UE.

- Option 2: 2 for target and co-scheduled UE

  - Note: It depends on issue with DMRS ports mapping.

DMRS scrambling ID for target UE and co-scheduled UE

- Option 1: Same scrambling ID when paired UEs are in the same CDM group. Different scrambling ID when paired UEs are in different CDM groups.

- Option 2: Same scrambling ID for all cases

- Option 3: Configure variable scrambling ID during the test. FFS the details

Reference receiver

Candidate Receiver

- Use MMSE-IRC processing with serving signal demodulation as baseline

Network assistance

- Not introduce network assistance for MMSE-IRC processing with serving signal demodulation

QCL assumptions for target UE and co-scheduled UE locating in different CDM groups

- Specifying of QCL assumptions for different CDM groups is not required for MMSE-IRC processing with serving signal demodulation

PDSCH parameters

Channel bandwidth

- For FDD 15kHz SCS: Cover 10MHz CBW

- For TDD 30kHz SCS: Cover 40MHz CBW

MIMO correlation for each UE

- Option 1: Use ULA Low for 2TX and XPL Low for 4TX

  - FFS the correlation matrix for XPL Low

- Option 2: ULA Low

Propagation conditions

- Option 1: Only TDLA30-10

- Option 2: Only TDLC300-100

- Option 3: Further down select based on analysis

MCS

- For case with rank 1+1

  - Option 1: MCS 13

  - Option 2: Not consider 16QAM/MCS 13

- For case with rank 2+1(if introduced)

  - Option 1: 13

  - Option 2: 19

- For case with 2+2(if introduced)

  - Option 1: 13

  - Option 2: 19

  - Option 3: Not consider 64QAM

Criteria for down selection of test parameters

- The agreed baseline performance evaluation metric, i.e., evaluate the gain of MMSE-IRC over MMSE in terms of 70% max throughput performance of the target UE, under the same simulation setup, will be used for down-selection of critical test parameter for both phase I and phase II.

Signal power assumptions

- Option 1: Average target UE signal power is equal to 1 and average interference UE signal power is equal to 1

- Option 2: Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal

SNR assumptions

- Option 1: SNR = STargetUE/N

- Option 2: SNR = (STargetUE+ SInterfUE)/N