3GPP TSG-RAN WG4 Meeting #100-e		R4-2115736

E-meeting, 16 August – 27 August, 2021

Agenda item:	9.12.2.3

Source:	Intel Corporation

Title:	TP to TR 38.833: Link level simulation results for LTE CRS interference handling for NR UE

Document for:	Approval

# Introduction

This TP provides the draft of “Link level simulation results” section for LTE CRS interference handling for NR UE.

# Reference

- RP-211135, Revised WID on Further enhancement on NR demodulation performance, China Telecom, RAN #92e, June 2021.

START OF TEXT PROPOSAL

### 5.3.2	Link level simulation results

In this section link level simulation results from different companies are collected for analysis on UE CRS interference handling for NR UE in scenario with overlapping spectrum for LTE and NR. The link level analysis of NR UE PDSCH performance is performed under assumptions from Section 5.3.1 and presented in Sections 5.3.2.1 and 5.3.2.2. Also, this section contains the analysis from different companies with impact of considered CRS interference handling schemes on LTE UE performance which is presented in Section 5.3.2.3.

The details of NR UE PDSCH performance analysis are presented in the following file:

#### 5.3.2.1	NR UE PDSCH performance for Scenario 1

This section contains the summary of simulation results of link level analysis of NR UE performance for Scenario 1.

Table 5.3.2.1-1 provides the summary of average simulation results from different companies with information about SNR points corresponding to 70% of maximum achievable throughput of Reference scheme. Table 5.3.2.1-2 provides the information about SNR performance difference of different schemes in comparison to Reference scheme for average simulation results.

Table 5.3.2.1-1: Average SNR simulation results for Scenario 1

| MIMO | MCS index | Ref scheme | Scheme #1 | Scheme #2 | Scheme #3 | Scheme #4 | Scheme #5 | Scheme #6 | Scheme #7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4Tx 2Rx Low | QPSK MCS4 | 3.3 | 1.6 | 3.3 | 1.7 | -0.1 | 0.2 | 0.6 | -0.4 |
|  | 16QAM MCS13 | 11.2 | 9.9 | 11.7 | 11.2 | 7.8 | 7.7 | 8.9 | 8.3 |
| 4Tx 4Rx Low | QPSK MCS4 | -0.2 | -2.0 | -0.5 | -2.1 | -3.4 | -3.4 | -2.5 | -3.9 |
|  | 16QAM MCS13 | 7.4 | 5.9 | 7.6 | 7.0 | 3.8 | 3.7 | 5.4 | 4.4 |

Table 5.3.2.1-2: SNR performance difference for Scenario 1

| MIMO | MCS index | Scheme #1 | Scheme #2 | Scheme #3 | Scheme #4 | Scheme #5 | Scheme #6 | Scheme #7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4Tx 2Rx Low | QPSK MCS4 | 1.7 | 0.0 | 1.6 | 3.3 | 3.1 | 2.7 | 3.7 |
|  | 16QAM MCS13 | 1.3 | -0.5 | 0.0 | 3.4 | 3.4 | 2.3 | 2.9 |
| 4Tx 4Rx Low | QPSK MCS4 | 1.8 | 0.2 | 1.9 | 3.2 | 3.2 | 2.2 | 3.7 |
|  | 16QAM MCS13 | 1.5 | -0.2 | 0.5 | 3.6 | 3.7 | 2.1 | 3.0 |

#### 5.3.2.2	NR UE PDSCH performance for Scenario 2

This section contains the summary of simulation results of link level analysis of NR UE performance for Scenario 2.

Table 5.3.2.2-1 provides the summary of simulation results from different companies and average results with information about SNR points corresponding to 70% of maximum achievable throughput of Reference scheme. Table 5.3.2.2-2 provides the information about SNR performance difference of different schemes in comparison to Baseline scheme for average simulation results.

Table 5.3.2.2-1: Average SNR simulation results for Scenario 2

| MIMO | MCS index | Ref scheme | Scheme #4 | Scheme #5 | Scheme #6 | Scheme #7 |
| --- | --- | --- | --- | --- | --- | --- |
| 4Tx 2Rx Low | QPSK MCS4 | 2.0 | -0.6 | -0.5 | -0.1 | -0.9 |
|  | 16QAM MCS13 | 9.9 | 7.4 | 7.6 | 8.1 | 7.8 |
| 4Tx 4Rx Low | QPSK MCS4 | -1.7 | -4.0 | -4.1 | -3.3 | -3.6 |
|  | 16QAM MCS13 | 6.1 | 3.6 | 3.6 | 4.7 | 4.8 |

Table 5.3.2.2-2: SNR performance difference for Scenario 2

| MIMO | MCS index | Scheme #4 | Scheme #5 | Scheme #6 | Scheme #7 |
| --- | --- | --- | --- | --- | --- |
| 4Tx 2Rx Low | QPSK MCS4 | 2.6 | 2.5 | 2.2 | 2.9 |
|  | 16QAM MCS13 | 2.5 | 2.3 | 1.8 | 2.1 |
| 4Tx 4Rx Low | QPSK MCS4 | 2.3 | 2.4 | 1.6 | 1.9 |
|  | 16QAM MCS13 | 2.5 | 2.6 | 1.4 | 1.3 |

For Scenario 2, SNR results and SNR performance difference in comparison to Reference Scheme is not provided for Schemes #1,2 and 3, because per slot TBS values and, as a result, maximum achievable throughput values are different for these schemes.

Table 5.3.2.2-3 provides the information about TBS values for different schemes and different MCSs.

Table 5.3.2.2-3: TBS values for Scenario 2

| MCS index | Reference Scheme, Scheme #4, 5, 6, 7 | Scheme #1, 2, 3 |  |
| --- | --- | --- | --- |
|  |  | PDSCH 9 symb | PDSCH 11 symb |
| QPSK MCS4 | 4096 | 2472 | 3240 |
| 16QAM MCS13 | 13064 | 7680 | 10248 |

#### 5.3.2.3	LTE UE performance

This section provides the observations and analysis from different companies with performance impact of CRS-RM schemes on LTE UE performance:

- Due to RM applied in interference cells, the CRS REs and data REs under LTE cells will observe different interference level with SINR offset.

  - From companies’ analysis:

    - Based on the INR levels used for RAN4 link-level simulation, for UE at 5% geometry, the delta of SINR observed at CRS RE and data RE is 5.86 dB and 11.75 dB for one dominant interference cell CRS-RM and two interference cell CRS-RM respectively.

  - From one company result:

    - Based on the system level simulation for ISD of 1000m from one company in R4-2115629, the average delta of SINR observed at CRS RE and data RE is ~2.5 dB to ~4.5 dB for one dominant interference cell CRS-RM and two interference cell CRS-RM respectively.

- The interference mismatch among CRS REs and data REs may bring impact on LTE cells considering the following aspects:

  - LTE CQI/RI/PMI is computed based on CRS for TM 1-8 and certain configuration of TM9 (when the parameter pmi-RI-Report is not configured by higher layers for TM9).

  - CRS is used for LTE PDSCH demodulation processing for TMs 1-6.

  - LTE RSSI is measured only from OFDM symbols containing CRS port 0 of measurement subframes unless indicated otherwise by higher layers, and it can be measured from all OFDM symbols of the DL part of measurement/indicated subframes if indicated by higher layers. LTE RSRQ is calculated based on RSRP and RSSI.

The following NW implementation solutions were provided by one company to address above LTE cell impact and the feasibility of such solutions from network implementation perspective needs further discussion:

- Transmit signal energy in rate-matched REs on top of rate matching. This signal energy could be NZP CSI-RS, random data, copy of PDSCH data or some other signal.

END OF TEXT PROPOSAL