3GPP TSG-RAN WG4 Meeting #100-e	R4-2114933

# Electronic Meeting, August 16-27, 2021

Title:	LS on UE capability for UE power class 2 NR inter-band CA and SUL configurations

Response to:

Release:	Release 17

Work Item:	NR_SAR_PC2_interB_SUL_2BUL

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact person:	Bo Liu

liubo1@chinatelecom.cn

1	Overall description

The power class 2 NR inter-band CA include 4 cases as shown in the WID RP-201584.

|  | UE power class | NR Carrier x power class | NR Carrier y power class |
| --- | --- | --- | --- |
| Case a | 26dBm | 23dBm | 23dBm |
| Case b | 26dBm | 23dBm | 26dBm |
| Case c | 26dBm | 26dBm | 23dBm |
| Case d | 26dBm | 26dBm | 26dBm |

The power class 2 NR SUL configurations include 1 case as shown in the WID RP-201584.

| UE power class on SUL band | UE power class on NR TDD band |
| --- | --- |
| 23dBm | 26dBm |

RAN4 has discussed the SAR solutions for UE power class 2 NR inter-band CA and SUL configurations, and achieved the agreements on duty cycle based SAR solution.

It is agreed to report one total dutycycle capability of  maxUplinkDutyCycle-interBandCA-PC2 independent of power class cases for power class 2 NR inter-band CA, and report one total dutycycle capability of  maxUplinkDutyCycle- SULcombination-PC2 for power class 2 NR SUL configurations. The dutycycle capability indicates the maximum uplink duty threshold that UE can guarantee to keep working on power class 2.

maxUplinkDutyCycle-interBandCA-PC2

The maxUplinkDutyCycle-interBandCA-PC2 capability indicates the maximum average percentage of symbols during a certain evaluation period that can be scheduled for uplink transmission so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. The average percentage of uplink symbols is specified in 6.2A.1.3 in 38101-1 and the capability applies to the CA combinations listed in table 6.2A.1.3-1 in 38101-1. The capability is optional and only for FR1.

The maxUplinkDutyCycle-interBandCA-PC2 capability is reported by UE as per band combination capability. The values and range is listed as below, if the field is absent, UE shall work on power class 2 regardless of UL duty cycle and may use P-MPRc as defined in 6.2.4 in 38101-1 if necessary (Note that specific targeted UL duty cycle percentage is not assumed  if the field is absent).

maxUplinkDutyCycle-interBandCA-PC2  {n50, n60, n70, n80, n90, n100 }

where, value n50 corresponds to 50%, value n60 corresponds to 60%  and so on.

maxUplinkDutyCycle-SULcombination-PC2

The maxUplinkDutyCycle-SULcombination-PC2 capability indicates the maximum average percentage of symbols during a certain evaluation period that can be scheduled for uplink transmission so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. The average percentage of uplink symbols is specified in 6.2C.1 in 38101-1 and the capability applies to all the SUL configurations with 1 SUL band + 1 TDD band. The capability is optional and only for FR1.

The maxUplinkDutyCycle-SULcombination-PC2 capability is reported by UE as per band combination capability. The values and range is listed as below, if the field is absent, UE shall work on power class 2 regardless of UL duty cycle and may use P-MPRc as defined in 6.2.4 in 38101-1 if necessary (Note that specific targeted UL duty cycle percentage is not assumed  if the field is absent).

maxUplinkDutyCycle-SULcombination-PC2 {n50, n60, n70, n80, n90, n100 }

where, value n50 corresponds to 50%, value n60 corresponds to 60%  and so on.

2	Actions

To RAN WG2

ACTION: 	RAN4 kindly asks RAN2 to take the above agreements into consideration and design the capability signalling.

3	Dates of next TSG-RAN WG4 meetings

3GPP TSG-RAN WG4 Meeting #101-e	Nov. 01-12, 2021, Electronic Meeting

3GPP TSG-RAN WG4 Meeting #102	Feb. 21-25, 2022, Electronic Meeting